TECHNICAL NOTE  ·  THE COMPLETE SYSTEM  ·  OCTOBER 2026

The Fushë-Kuqe system in full: variable-density flow, multi-species hydrogeochemistry, boundary and initial conditions, and the objective that places the wells

D. Zeqiraj

Faculty of Geology and Mining, Polytechnic University of Tirana, Albania. Collects in one place the equations of the author's Fushë-Kuqe models, experiments and reviews, and adds the innovation PUSI-GO (pusi: the well).

Abstract. This note writes down, in one place and in one notation, the complete mathematical system used for the Fushë-Kuqe coastal aquifer in Albania: three-dimensional variable-density flow with storage, leakage and wells; the transport of seven major ions with cation exchange, mass transfer to clays, sulfate reduction, calcite and electro-diffusion; every boundary condition (sea, land, recharge, rivers, base, wells) and every initial condition, with the conditions that make them compatible; and the objective functions that place monitoring, recharge and supply wells. For every equation it states the values used at Fushë-Kuqe and whether, and where, it is coded in the author's repository, so that the four reductions actually computed (an areal steady model, a Strack sharp-interface chloride model, a vertical variable-density section and a seven-ion flow line) can be read as cuts through one system. The innovation, PUSI-GO, puts the hydrochemistry into the objective that places wells. The repository's placement methods judge a supply well by the head proxy h < 1 m; PUSI-GO uses the drinking-water standard in the well screen. Three propositions make this exact and cheap: the standard is a threshold on a quantity that is linear in every well rate (P1); for top-of-aquifer screens the threshold is the same at every well and is set by the width of the mixing zone (P2); and chloride, not sodium, is the standard that binds, during intrusion and behind a recharge well (P3). On the 20,000 calibrated Fushë-Kuqe draws the head proxy sees a 0.044 probability that a supply well fails while the chloride standard gives 0.594; the head proxy keeps recharge sites that leave 2.3 percentage points more risk; one shallow chloride sample is worth 0.312 nats against 0.089 for the best three heads, and doubles the pumping that can be allowed at a 10% risk.

Keywords: seawater intrusion; variable-density flow; reactive transport; cation exchange; boundary conditions; initial conditions; optimal experimental design; managed aquifer recharge; well placement; Fushë-Kuqe

# 1. What this note contains

Over the past months the author's series has built, piece by piece, the equations of the Fushë-Kuqe coastal aquifer: the variable-density flow and goal-oriented design of the base paper [1], the chloride map of the companion study [2], the multi-species chemistry and memory of reviews 42 and 43 and their synthesis [3–5], the boundary and initial conditions of reviews 44 and 45 [6, 7], and the placement of recharge wells in reviews 53 and 54 [8, 9]. This note collects them as one system. Section 2 fixes the domain, the data and the unknowns; Section 3 the flow; Section 4 the transport and the hydrogeochemistry; Sections 5 and 6 the boundary and initial conditions; Section 7 the objective functions that place wells; Section 8 the innovation PUSI-GO; and Section 9 lists the whole system with the status of every term.

Two kinds of statements are mixed in a model description and are kept apart here. Specified terms belong to the complete system that the COST proposal aims at; coded terms are computed in the repository, and for these the file is named. Table 1 gives the map. The computed models are four reductions of the complete system, each exact for its own question, and the main practical result of this note is that the objective that places wells can be made consistent with the chemistry without leaving the cheapest of them.

Table 1. Map of the system. Equation numbers refer to this note. "Specified" means that the term is part of the complete system and has been stated and analysed in the series, but no field-scale code solves it yet.

| Part of the system | Equations | Coded in the repository | Status |
| --- | --- | --- | --- |
| Domain, data, prior of the conductivity | (1)–(4) | experiments/prag_go_fushe_kuqe/fk.py; chloride_fk/python/fkcl_model.py | coded |
| 3-D variable-density flow with storage and memory | (5)–(9) | Henry benchmark only (src/vd_residual.m) | specified |
| R1: areal steady flow + Ghyben–Herzberg | (10)–(12) | fk.py, fk_geo.py, src/fushe_kuqe_field_design.m | coded |
| R2: Strack potential + dispersive interface + chloride | (13)–(16) | chloride_fk/python/fkcl_model.py | coded |
| R3: vertical variable-density section A–A′ | (17) | chloride_fk/python/fkcl_vd.py | coded |
| R4: seven-ion flow line with exchange and memory | (18); (19)–(28) | experiments/gjurma_go, ritmi_go | coded (1-D) |
| Exchange, calcite, sulfate reduction, electro-diffusion | (22)–(27) | gjurma_go, ritmi_go, tretja_go, lidhja_go, ngarkesa_go | coded in 1-D or batch |
| Boundary conditions | (29)–(38) | fk_geo.py, fkcl_model.py, kufiri_go, fillimi_go | partly coded |
| Initial conditions, compatibility, handover | (39)–(42) | ritmi_go (spin-up), fillimi_go, kufiri_go | partly coded |
| Well-placement objectives | (43)–(50) | src/eig_operator.m, fk_prag_geo.py, fk_mar_geo.py, vendi_go | coded |
| PUSI-GO (new) | (51)–(59) | experiments/pusi_go | coded and tested |

INNOVATION PUSI-GO (pusi: the well) puts the hydrochemistry into the objective that places wells. The repository's placement methods (PRAG-GO, PRAG-MAR, VENDI-GO) call a supply well salinised when its head falls below 1 m, the Ghyben–Herzberg test of a 40-m-deep aquifer base. PUSI-GO calls it salinised when the chloride averaged over its screen exceeds the drinking-water standard of 250 mg/L, and checks the sodium standard of 200 mg/L. Proposition P1 shows that this costs nothing: in the Strack model the potential is linear in every pumping and recharge rate and the screen chloride falls monotonically with it, so the standard is a threshold event on a linear quantity, exactly like the head proxy but with a random threshold that carries the chemistry. Proposition P2 shows that for screens at the top of the aquifer this threshold is the same at every well and is set by the width of the mixing zone, a quantity that heads cannot see. Proposition P3 shows that chloride is the standard that binds, both while sea water intrudes and behind a recharge well, and gives a bound on sodium in freshened water. On the calibrated Fushë-Kuqe ensemble the head proxy sees almost no risk where the standard is exceeded in more than half of the aquifers, keeps recharge sites that leave 2.3 percentage points more risk, would allow pumping that fails the standard in 46% of aquifers, and prefers head observations that carry less than a third of the information of one chloride sample.

# 2. Domain, data and unknowns

The Fushë-Kuqe aquifer lies on the Adriatic coast north of Durrës. It consists of two gravel and sand horizons separated by silt and clay and confined by a silt-clay cap; the Gorre–Fushë-Kuqe well field abstracts about 750 L/s, the artesian head has fallen from 8.0 to 1.5 m, and the salinity reaches about 1,500 and 3,500 mg/L in the two horizons [10]. The data are in the workbook data/Fushe_Kuqe_All_Data.xlsx: the aquifer polygon (3,411 vertices, 349.09 km²), 180 wells with Gauss–Krüger coordinates (Krasovsky 1940, zone 4), 43 of them with hydraulic conductivity (50–200 m/d), four major-ion analyses and two chloride series of 1984–2001 [10]. All models use the local frame

| x=X−4 381 751.0 m1000, y=Y−4 576 243.97 m1000 km | (1) |
| --- | --- |

where X,Y are the Gauss–Krüger coordinates. The polygon spans 20.68 km by 52.17 km; the computational grid has Δx=0.25 km, 84 × 210 nodes and 5,589 active cells (349.31 km²).

The three-dimensional domain is the aquifer between the depth dx,y of its top and the depth d+B of its base, both measured downward from sea level, and the model sector Ωs of the chloride study is the part north of y=24 km that is connected to the sea (3,545 cells, 221.56 km²; Figure 1):

| Ω=x,y,z: x,y∈Ω2, −d−B≤z≤−d, d=maxdcx,y−g0, 1 m, B=be+bc−be clip1−rx,yLB,0,1 | (2) |
| --- | --- |

where z is the elevation (m a.s.l.), Ω2 the aquifer polygon, dc the top depth interpolated (inverse distance, power 2) from the 43 depth values of the workbook, g0 a ground-elevation offset, r the distance to the GLOBE shoreline (km), and the thickness B thins from bc at the coast to be at the distance LB (chloride_fk/python/fkcl_model.py). The areal design model R1 uses a constant thickness b=30 m and a base 40 m below sea level instead.

The hydraulic conductivity is the interpolated field of the 43 well values, scaled zone by zone:

| Kx=aK KIDWx eθzx, KIDWx=j​Kjx−xj−2j​x−xj−2 | (3) |
| --- | --- |

where Kj is the conductivity of well j, θz the log-multiplier of zone zx (12 zones, the Voronoi cells of a pilot lattice), and aK a scale factor of the chloride model (aK=1 in R1).

The prior of the zonal log-multipliers is facies-aware [1]:

| θ∼N0,ΓF, ΓFij=sisj e−dij/Lc1Fi=Fj+ρF 1Fi≠Fj+10−9δij | (4) |
| --- | --- |

where dij is the distance between pilot points, Lc=8 km, si=0.30, 0.45 or 0.60 for coarse, medium or fine facies Fi (by the zone-mean conductivity: coarse above 150 m/d), and ρF the cross-facies correlation, 0.6 in the design prior and 0.805 as estimated from the field (fk.py; demo/gate3_field.m).

The remaining uncertain inputs of the chloride model and their priors are listed in Table 2 together with the fixed constants. The vector of unknowns is p=θ,R,aK,bc,be,LB,g0,w,Cls, twenty numbers; the state is the head (or potential), the seawater fraction and the seven ion concentrations with the exchanger composition.

Table 2. Constants and priors. ξ denotes a standard normal variable. The density contrast 0.025 is used by every code; the Adriatic seawater composition of the chloride study implies a density of 1.0285, i.e. 0.0285, a small inconsistency noted in Section 9.

| Symbol | Meaning | Value or prior | Source in the repository |
| --- | --- | --- | --- |
| δ | density contrast ρs−ρf/ρf | 0.025 | fk.py, fkcl_model.py, vd_params.m |
| R | recharge | 5×10−4 m/d (182.5 mm/yr); prior 5×10−4e0.7ξ | fk.py; fkcl_model.py |
| aK | scale of K (chloride model) | U0.3,0.8 | fkcl_model.py |
| bc, be, LB | thickness at the coast, inland, thinning distance | U180,200 m, U5,10 m, U12,20 km | fkcl_model.py |
| g0 | ground-elevation offset of the top depth | U0,5 m | fkcl_model.py |
| w | width of the mixing zone | 20 e0.6ξ m | fkcl_model.py |
| Clf, Cls | chloride of fresh (well 193) and sea water | 12.92 mg/L; U20 500,22 000 mg/L | fkcl_model.py |
| b, base | thickness and base depth of R1 | 30 m; 40 m below sea level | fk.py |
| n, Kz/Kx | porosity and anisotropy (section R3) | 0.25; 0.1 | fkcl_vd.py |
| αL, αT, Dm | dispersivities and molecular diffusion (R3) | 10 m, 1 m, 8.64×10−5 m²/d | fkcl_vd.py |
| E | exchange capacity per litre of pore water | median 60 meq/L (log-sd 0.4) | gjurma_exp.py, ritmi_core.py |
| KNa∖K, KNa∖Ca, KNa∖Mg | Gaines–Thomas selectivities | 0.2, 0.4, 0.5 (log-sd 0.25) | gjurma_model.py |
| a1,b1; a2,b2 | rates and capacities of silt and clay zones | 1.0 /yr, 0.3; 0.05 /yr, 0.6 | ritmi_core.py |
| ksr | sulfate-reduction rate in the clay | median 0.05 /yr (log-sd 0.6) | gjurma_exp.py |
| Cllim, Nalim | drinking-water standards | 250 mg/L; 200 mg/L | fkcl_model.py; this note [11] |

Figure 1. The model sector of the chloride study. Shading: posterior probability that chloride averaged over the upper 40 m of the aquifer exceeds 250 mg/L without pumping (2,000 resampled calibrated members). Orange dots: the 92 cells linked to the sea by the Robin condition (32); every other margin, including the southern cut at y=24 km, is no-flow (34). Triangles: the supply wells A1 and P10; circles: the 16 candidate monitoring wells (green: the best chloride sample of Section 8); diamonds and squares: recharge sites chosen with the head proxy and with the chemistry threshold.

# 3. Flow

## 3.1 The complete three-dimensional system

The flow of water of variable density is written for the equivalent freshwater head hf=p/ρfg+z [12, 13], with storage, wells, a leakage and storage memory of the aquitards and a learned closure [4, 7]:

| Ss ∂thf+∇⋅q=w​Qwt δx−xw−0tκht−s ∂shf ds+Ψhf;γ | (5) |
| --- | --- |

where Ss is the specific storage (1/m), q the Darcy flux (m/d), Qw the rate of well w at xw (positive for injection), κh a completely monotone memory kernel that represents delayed storage in silt and clay lenses [14, 15], and Ψ a closure expanded in basis functions of the head with coefficients γ learned from data (zero in all computed models).

| q=−K∇hf+ρ−ρfρf ez, K=diagK,K,Kz | (6) |
| --- | --- |

where K is the conductivity tensor for fresh water and ez the upward unit vector. The buoyancy term drives the salt wedge.

| ρ=ρf1+δ c, c=fsea=cCl−cCl,fcCl,s−cCl,f | (7) |
| --- | --- |

where c is the relative seawater fraction, computed from chloride, the reaction invariant of Section 4.6, so that density follows the conservative tracer and not the reactive ions. Viscosity is taken constant (Boussinesq approximation), consistent with δ=0.025.

The memory term splits into a leakage part and a storage memory (Lemma 1 of [5]): with κ=0∞κh and the tail Khr=r∞κh,

| 0∞κhr hft−r dr=κ hft−0∞Khr ∂thft−r dr, κhs=k​bkake−aks | (8) |
| --- | --- |

where ak are exchange rates (1/yr) and bk capacities of the immobile zones. A sum of exponentials is completely monotone and has a Markovian embedding with one state per zone, which keeps the system assimilable [5].

Two clocks run in this system: heads diffuse with Dh=T/S≈2×107 m²/d (transmissivity 4,000–8,000 m²/d, storativity about 3×10−4) and feel a pumping change 5 km away within about a day, while salt moves 5 km in about a century [5]. On the clock of salinity the head is therefore quasi-steady, which is why the steady reductions below are exact for questions about salinity at a given pumping.

## 3.2 Wells

Wells enter (5) as point sources. In a grid model the well is a source in its cell; the head in the well itself, needed when a pumping constraint is written for the well, follows from the cell head by the Peaceman relation [16]:

| hwell=hcell+Qw2πTlnrerw, re≈0.2 Δx | (9) |
| --- | --- |

where Qw<0 for abstraction, rw is the well radius and re the equivalent radius of a square cell. All repository codes use the cell value (specified only; with Δx=250 m and rw=0.15 m the correction for 10,000 m³/d in a transmissivity of 5,000 m²/d is 1.8 m).

## 3.3 Reduction R1: areal steady flow with the Ghyben–Herzberg test

The design model of the base paper [1] keeps one confined layer of constant thickness and steady flow:

| ∇⋅T∇h+R+w​Qw δx−xw=0, T=KIDW b eθzx | (10) |
| --- | --- |

where h is the freshwater head (m a.s.l.), discretised as l​Tklhl−hk=−RΔx2−Qk with arithmetic face means (fk.py; src/fushe_kuqe_field_design.m).

| ζ=hδ, cell intruded ⇔ h<hc=δ⋅base=1.0 m | (11) |
| --- | --- |

where ζ is the Ghyben–Herzberg depth of the interface below sea level. This is the head proxy of salinisation used by every placement method of Section 7.

| Ah=10−6k​Δx2 σhc−hks, σu=11+e−u | (12) |
| --- | --- |

where A is the smoothed intruded area (km²) with s=0.15 m, the goal of the published design.

## 3.4 Reduction R2: Strack potential, dispersive interface and chloride

The chloride model [2] keeps a sharp interface with a mixing zone of width w and a thickness that varies in space, and solves for Strack's single potential [17]:

| ∇⋅K∇ϕ+R+w​Qw δx−xw=0 | (13) |
| --- | --- |

where ϕ (m²) is the discharge potential, Q=−K∇ϕ the discharge per unit width; the equation is linear in ϕ, R and every Qw (fkcl_model.py).

| ϕ<12δB2: ζ=d+2ϕδ, h=δd+2δϕ; ϕ≥12δB2: h=ϕB+δd+B2, ζ=hδ | (14) |
| --- | --- |

where the interface lies inside the aquifer in the first zone (the wedge) and below its base in the second; both branches meet continuously at ϕ=δB2/2, with ζ=d+B.

| cz=Φz−ζw, cz1,z2=wz2−z1Gz2−ζw−Gz1−ζw, Gu=u Φu+Φ′u | (15) |
| --- | --- |

where z is here the depth below sea level (positive down), Φ the standard normal distribution function, so that the mixing zone is a Gaussian smoothing of the sharp interface, and c is the closed-form average over a well screen from z1 to z2.

| Cl=Clf+cCls−Clf | (16) |
| --- | --- |

where Cl is the chloride that a well samples; the code reports three 40-m screens: at the top of the aquifer (first horizon), at mid-thickness and at the base.

The model was calibrated by likelihood weighting of 20,000 prior draws on the documented facts (15% of the upper layer affected, littoral head about 3 m, seawater fraction 4.7% at the mid-depth screens of wells 341 and 503; effective sample size 3,249) [2]. It is the reduction on which PUSI-GO is built.

## 3.5 Reduction R3: the vertical variable-density section

Along the section A–A′ (y=36.5 km, through the well field) the complete equations (6)–(7) are solved in two dimensions, steady, with transport of the seawater fraction (chloride_fk/python/fkcl_vd.py):

| ∇⋅K∇hf+δ c ez=0, ∇⋅qc−∇⋅D∇c=0, D=nDmI+αLq+αTq | (17) |
| --- | --- |

where Kz=0.1K, n=0.25, Dm=8.64×10−5 m²/d, αL=10 m and αT=1 m (cross terms of the dispersion tensor omitted); the equations are solved by implicit pseudo-time marching. The Henry benchmark of the base paper uses the same equations (src/vd_residual.m).

## 3.6 Reduction R4: the seven-ion flow line

For the chemistry, experiments GJURMA-GO and RITMI-GO use a coast-perpendicular flow line of 15 km in 75 cells with the abstraction field at 6 km and the four analysed wells at assumed distances (503 at 1 km, 341 at 2 km, 340 at 4.5 km, 193 at 12 km), driven by the pumping history [3, 4]:

| vt=−15 m/yr,t<1975+200 m/yr,1975≤t<1990−40 m/yr,1990≤t<2000+50 m/yr,t≥2000 | (18) |
| --- | --- |

where v is the pore velocity of the coastal segment, landward positive (inland v=−50 m/yr toward the well field); the transport equations of Section 4 are solved on this line with quarterly steps to 2000 and monthly steps to 2045 (gjurma_model.py, ritmi_model.py).

# 4. Transport and hydrogeochemistry

## 4.1 Species and end-members

Seven major ions are transported: Cl⁻, Na⁺, K⁺, Ca²⁺, Mg²⁺, SO₄²⁻ and HCO₃⁻ (charges zi=−1,1,1,2,2,−2,−1). Four cations can sit on the exchanger as NaX, KX, CaX₂ and MgX₂, with equivalent fractions βi. Calcite is the reactive mineral; sulfate is reduced in organic-rich clay. Two end-members define mixing (Table 3): the recharge water of well 193 in the north-east [10] and sea water, standard sea water of salinity 35 [18] scaled to the Adriatic salinity of 38.3 and density 1.0285 as in the chloride study [2].

Table 3. End-member compositions (gjurma_model.py; fkcl_hydrochem.py) and the most saline analyses of Cenameri & Beqiraj [10] (dashes: see the workbook). In mmol/L the fresh water is Cl 0.364, Na 0.772, K 0.046, Ca 0.817, Mg 1.011, SO₄ 0.610, HCO₃ 2.985.

| Water (mg/L) | Cl | Na | K | Ca | Mg | SO₄ | HCO₃ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fresh: well 193 (sheet 5) | 12.92 | 17.75 | 1.80 | 32.74 | 24.56 | 58.56 | 182.16 |
| Standard sea water, S = 35 | 19,353 | 10,781 | 399 | 412 | 1,284 | 2,712 | 145 |
| Adriatic sea water (× 38.3/35 × 1.0285) | 21,781 | 12,134 | 449 | 464 | 1,445 | 3,052 | 163 |
| Well 503 (SW, most saline) | 1,048 | 476 | – | – | – | – | – |
| Well 340 (central band) | 417 | – | – | – | – | – | – |

## 4.2 Multi-species transport with exchange and memory

With concentrations c in meq/L of pore water, the transport of the seven ions with an exchanger in local equilibrium, K immobile zones and reactions reads [3–5]

| ∂tc+E sc+k=1Kbkakc−ck+∇⋅vc−∇⋅D∇c=Nrc+Φc,x,t;ϑ | (19) |
| --- | --- |

where v=q/n is the pore velocity, E the exchange capacity (meq per litre of pore water), sc the equivalent fractions on the exchanger for the cations and zero for the anions, ck the concentrations in immobile zone k, N the stoichiometric matrix of the reactions of Section 4.6, r the known reaction rates and Φ a learned closure confined to the span of N, so that chloride stays closure-free.

| ∂tck=akc−ck+Γkck, k=1,…,K | (20) |
| --- | --- |

where Γk holds reactions inside zone k (sulfate reduction in the clay). Eliminating ck gives the memory kernel of (8) in the transport equation (Proposition 1 of [5]).

| D=Dm+αTvI+αL−αTvv⊤v | (21) |
| --- | --- |

where D is the Bear–Scheidegger dispersion tensor [19], common to all ions when electro-diffusion is neglected. On the flow line D=αLv+DM with αL of median 150 m and DM=0.03 m²/yr.

## 4.3 Cation exchange

Exchange follows the Gaines–Thomas convention with sodium as reference [20, 21]:

| KNa∖I=βNa aI1/zIβI1/zI aNa, I=K,Ca,Mg, βNa+βK+βCa+βMg=1 | (22) |
| --- | --- |

where a are activities (taken as molar concentrations in the flow-line code) and the selectivities are 0.2, 0.4 and 0.5. Given the solution, the fractions follow in closed form:

| βK=βNacKKKcNa, βCa=βNa2cCaKCa2cNa2, βMg=βNa2cMgKMg2cNa2, βNa=2bβ+bβ2+4aβ | (23) |
| --- | --- |

where aβ=cCa/KCa2+cMg/KMg2/cNa2 and bβ=1+cK/KKcNa (gjurma_model.py). Each transport step solves ziyi+Eβiy=Ti for the four cations with the totals Ti fixed, which conserves charge and every cation.

## 4.4 Minerals, sulfate reduction and activities

| Ω=aCa2+ aCO32−Ksp, rcal=κ gΩ, gΩ=1−Ωn1,Ω≥Ωc1−Ωcn11−Ω1−Ωcn2,Ω<Ωc | (24) |
| --- | --- |

where Ω is the calcite saturation ratio (logKsp=−8.43 at 15 °C from phreeqc.dat [22], as computed in experiments/shkalla_go) and g a rate law with a kink between a near-equilibrium and a far-from-equilibrium branch (experiments/tretja_go; priors log10κ∈−6,−2 mmol/L/d, n1∈0.5,2, n2∈2,5, Ωc∈0.6,0.9); the transition-state law 1−Ωn is the special case Ωc=0 [23]. Mixing of fresh and sea water undersaturates the mixture (minimum Ω=0.58 at 25% sea water in TRETJA-GO), the mixing-corrosion effect [24].

| dcSO4,kdt=−ksr cSO4,k, dcHCO3,kdt=+2ksr cSO4,k | (25) |
| --- | --- |

where ksr is the rate of first-order sulfate reduction inside the immobile clay zone k of the coastal strip, concentrations in mmol/L (ritmi_model.py). Steady data see it only through reff=bkakksr/ak+ksr (Proposition 1 of review 43 [4]).

| log10γi=−0.509 zi2I1+I−0.3 I, I=12i​zi2mi | (26) |
| --- | --- |

where γi is the activity coefficient of the Davies model [25] (experiments/ngarkesa_go) and mi the molality; the flow-line codes set γi=1. The ionic strength is 0.0086 for the fresh and 0.68 for the sea end-member (TRETJA-GO).

## 4.5 Electro-diffusion

When ions diffuse at different rates, a diffusion potential couples them. With zero electric current [26]

| Ji=vci−D+Di∇ci−ziDici∇ψ, ∇ψ=−j​zjDj∇cjj​zj2Djcj, i​zici=0 | (27) |
| --- | --- |

where Di=τDifree with free-water coefficients (10⁻⁹ m²/s) Cl 2.032, Na 1.334, K 1.957, Ca 0.792, Mg 0.706, SO₄ 1.065, HCO₃ 1.185 [27] (experiments/lidhja_go). At Fushë-Kuqe the Péclet number is about 2.3×105 and the electro-diffusive leak into the chloride forecast is suppressed by a factor of about 4×10−6 (Theorem 2 of the author's note [28]).

## 4.6 Invariants and the field indicators

Proposition H1 (chloride is the invariant). If W⊤N=0, the reaction invariants u=W⊤c obey (19) without reaction terms and with the same memory. For the seven ions and the five reactions of (28) the invariants are chloride and the charge balance; hence the seawater fraction (7) and the density are reaction-free, and a chloride goal does not depend on the closure Φ (Proposition 3 and Theorem 1 of [5]).

The field chemistry is read through mixing with these end-members [21, 29]:

| f=Cl−ClfCls−Clf, ΔX=Xobs−Xf+fXs−Xf=NξX, BEX=Na+K+Mg−1.0716 Cl | (28) |
| --- | --- |

where f is the seawater fraction, ΔX the departure of ion X from mixing (meq/L), which equals the integrated reaction extents ξ through the stoichiometric matrix N with columns Na–Ca, Na–Mg and K–Ca exchange, sulfate reduction and calcite (rank 5), and BEX the base-exchange index (meq/L; negative for salinisation, positive for freshening).

At Fushë-Kuqe the indicators split the wells into two regimes (chloride_fk/results/hydrochem.json; experiments/gjurma_go): wells 503 and 341 near the coast have f = 4.8% and 4.7%, Na/Cl = 0.52 and 0.71 (sea water 0.86) and BEX = −2.80 and −2.64 meq/L, the signature of intrusion with sodium taken up by the exchanger (Na–Ca extent 4.7 meq/L at well 503); well 340 in the central band has f = 1.9%, Na/Cl = 1.07 and BEX = +2.43, the signature of freshening. By Corollary 1 of [5] a non-zero exchange extent means the aquifer is not at steady state.

# 5. Boundary conditions

The boundary of the domain is split into the sea face, the top (recharge and rivers), the inland and lateral margins, and the base; wells are internal [7]:

| ∂Ω=Γsea∪Γtop∪Γriv∪Γin∪Γbase | (29) |
| --- | --- |

where n is the outward unit normal. Table 4 lists, for each part, the flow, transport and chemical conditions, their Fushë-Kuqe values and their status.

## 5.1 Sea

On the sea face the pressure is hydrostatic in sea water, which in equivalent freshwater head reads [7, 13]

| hf=ρsρf ηt−δ z on Γsea, ηt=η0+η t+j​Ajcosωjt−φj+ηsurget+ηiat | (30) |
| --- | --- |

where η is the sea level: mean, rise (η∼N4,22 mm/yr in KUFIRI-GO, not calibrated to Fushë-Kuqe), tidal constituents (M2, K1, MSf, Sa), surges and interannual anomalies (sd 0.06 m, correlation 1 yr). Where the water table meets the beach a seepage face hf=z with outflow only applies.

Tides are damped inland with the length 2T/ωjS [30]; with storage memory the transfer function is

| hx,ωh0,ω=exp−xiωS1+κSiωT, κSλ=k​bkakak+λ | (31) |
| --- | --- |

where h is the complex amplitude of the head at frequency ω; each constituent identifies one complex value of the memory symbol (Proposition 2 of review 43 [4]). A tide-averaged model must add the tidal overheight to η0.

In the coded areal models the sea face is collapsed onto the western edge of the grid. The geolocated version links the 92 western-edge cells whose gap to the GLOBE shoreline is at most 5 km (northing 29.25–52.0 km) to sea level through a conductance (fk_geo.py; fkcl_model.py):

| qsea,k=Kkbk Δxgk+Δx/2hk−0 in R1, qsea,k=Kk Δxgk+Δx/2ϕk−0 in R2 | (32) |
| --- | --- |

where gk is the gap from the cell edge to the shoreline (0–4.8 km, median 1.15 km). This Robin condition keeps the discrete operator symmetric, which Section 8 uses. The published design model instead fixes h=0 on the whole 52-km western margin (Dirichlet); the two boundaries give intruded areas of 92.7 and 10.1 km².

For transport the sea face switches with the direction of flow, for every ion [7]:

| c=csea where q⋅n<0, D∇c⋅n=0 where q⋅n≥0 | (33) |
| --- | --- |

where csea is the Adriatic composition of Table 3. The flow line uses the inflow mixture f0csea+1−f0cf with f0 of median 0.07 and a zero-gradient outflow; the Henry benchmark imposes c=1 for advection on inflow and for diffusion on the whole face, a small departure from (33).

## 5.2 Land margins

| q⋅n=0, D∇c⋅n=0 on Γin | (34) |
| --- | --- |

where the condition holds on every margin of the polygon that is not in contact with the sea, and on the southern cut y=24 km of the chloride sector, chosen because the southern plain drains to the Ishëm and Erzen rivers (fk_geo.py). A regional inflow q⋅n=−qin with the composition of well 193 is the specified alternative.

## 5.3 Top: recharge, rivers and drains

The coded models use a uniform recharge R=5×10−4 m/d. The complete system computes it from a root-zone water balance with soil memory [7, 31]:

| nZr dsdt=Pt−Es−Ls, Es=Epmin1,s−sws⋆−sw, R=Ls=KseβLs−sfc−1eβL1−sfc−1 | (35) |
| --- | --- |

where s is the relative soil moisture, Zr the root depth, P the rainfall, Ep the Priestley–Taylor potential evaporation and L the drainage to the water table; the soil filters rainfall with the memory τs=nZr/E′+L′.

| R cClgw=P cClP+DCl, c=c193 in the recharge water | (36) |
| --- | --- |

where DCl is the dry deposition of chloride; the balance is a check on R, and the recharge carries the composition of well 193.

| qr=Crhrt−maxh,zb, c=crt where qr>0; qd=Cdh−zd+ | (37) |
| --- | --- |

where Cr, hr and zb are the conductance, stage and bed elevation of a river, and Cd, zd the conductance and elevation of a drain. The Mat, Droja, Ishëm and Erzen rivers and the Patok lagoon are not modelled in any field code; KUFIRI-GO tests a river at 3 km with Cr=50 m/yr. A lagoon or a river mouth at low flow can be a salt source.

## 5.4 Base and wells

| −q⋅n=κhf−hbase+0tKht−s ∂shf ds on Γbase; c=cinj at injection wells | (38) |
| --- | --- |

where hbase is the head below the aquitard; the leakage follows the memory split of (8) and is zero in the coded models (no-flow base); cinj is the composition of the water injected at recharge wells, while extraction wells need no transport condition.

Table 4. Boundary conditions of the complete system, their Fushë-Kuqe values and their status in the code.

| Boundary | Flow | Transport and chemistry | Fushë-Kuqe value | Coded |
| --- | --- | --- | --- | --- |
| Sea face | hydrostatic sea head (30) or Robin link (32) | switching (33), Adriatic composition | 92 contact cells, gaps 0–4.8 km; δ=0.025 | Robin: fk_geo.py, fkcl_model.py |
| Seepage face | hf=z, outflow only | outflow | not mapped | no |
| Inland, lateral, southern cut | no flow (34) | no flux | all non-sea margins; y=24 km | yes |
| Top: recharge | flux R or soil balance (35) | well-193 water (36) | R=5×10−4 m/d | uniform R only |
| Top: rivers, drains, lagoon | Robin (37) | river water where infiltrating | Mat, Droja, Ishëm, Erzen; Patok lagoon | no (KUFIRI test only) |
| Base | leakage with memory (38) | clay water | silt-clay aquitard | no-flow |
| Wells | point sinks/sources (5) | injected composition (38) | A1, P10; MAR 2,500–10,000 m³/d | cell sources |

# 6. Initial conditions

A coastal aquifer under pumping is never at steady state, and its clays remember older seas. Four initial conditions are possible [7]: (i) a steady state for a reference forcing, wrong for anything stored in clays; (ii) a palaeo-reconstruction from the last lowstand; (iii) a start before development, for example 1950, followed by the pumping history; (iv) an initial state estimated from data. The computed models use (i) for the areal reductions, which are steady by construction, and (iii) for the flow line, started from a steady state after a 3,000-year spin-up with constant forcing:

| hx,t0=hssx; pre-1975 forcing, cx,t0=c193, βx,t0=βeqc193, ckx,t0=cx,t0 | (39) |
| --- | --- |

where the exchanger starts in equilibrium with fresh water (calcium-rich); a relic marine exchanger in the central band is available as an option (F_RELIC, off in all reported runs).

The fourth option treats the initial state as an unknown and estimates it with a smoother:

| pu0∣y0:T∝pu0k​pyk∣utk;u0, u0=h,c,ck,β∣t0 | (40) |
| --- | --- |

where u0 is the initial state; RITMI-GO estimates it with an ensemble smoother with multiple data assimilation [32].

Boundary and initial data must agree where they meet, or the solution has a corner singularity and time stepping loses order [6]:

| gx,0=u0x, ∂tgx,0=f−Au0x,0 on ∂Ω | (41) |
| --- | --- |

where g is the boundary datum, u0 the initial datum, f the source and A the spatial operator (compatibility of order 0 and 1). A sea level that jumps at t=0 violates the first and gives ∂xu0,t=−1/πDt.

How long does the initial condition matter? FILLIMI-GO [6] writes the linearised system as ∂tz=−Mz+Ft and compares the variance of a goal g=w⊤z that comes from the initial state with the variance forced by the boundaries:

| VICt=w⊤e−MtP0e−M⊤tw=∫φλ e−2λt dνgλ, t⋆≥12λlnVIC0VBC∞ | (42) |
| --- | --- |

where P0 is the covariance of the initial state, νg the spectral measure of the goal and t⋆ the handover time after which boundaries dominate (Theorem F of review 44: VIC is completely monotone; with a spectral density cλβ−1 near zero, VIC∼cΓβ2t−β, a power law instead of an exponential).

Table 5. Handover from initial to boundary control in the FILLIMI-GO test bed (5 km, T=1,000 m²/d, Sy=0.1, 12 aquitard compartments; experiments/fillimi_go). Heads forget the initial state in under a year; a clay piezometer remembers it for a century, so the initial state must be estimated for storage goals and the boundaries monitored for transmissive goals.

| Goal | Handover, no memory | Handover, with clay memory | Initial share at 20 yr |
| --- | --- | --- | --- |
| Head 250 m from the sea | 0.049 yr | 0.094 yr | 0.002 |
| Head 1 km | 0.290 yr | 0.443 yr | 0.004 |
| Head 4.75 km | 0.593 yr | 0.910 yr | 0.007 |
| Clay piezometer, 2.5 km | – | 105.27 yr (5.79 yr with the legacy prior) | 0.773 |

For a twenty-year toe forecast KUFIRI-GO [7] splits the variance by boundary forcing, Var gH=j​w⊤PjHw with Pj=APj+PjA⊤+GjGj⊤: the atmosphere (rain anomalies and the drying trend) carries 94.5%, the sea 4.4%, the initial state 0.8% and pumping noise 0.3%. The controllable boundary is pumping (and managed recharge), which is why the objective of the next sections places wells.

# 7. The objective functions that place wells

## 7.1 Decisions, data and the Bayesian setting

Three kinds of wells are placed: monitoring wells (where, what to measure and at what depth), recharge wells (where and at what rate) and supply wells (how much to pump and how to screen). The decision vector is

| ξ=M, S, q, Q, L, M⊂Cobs, S⊂Crch | (43) |
| --- | --- |

where M is a set of observations (well, type: head or chloride, screen depth) from the candidate set Cobs, S a set of recharge sites from Crch with rates q, Q the pumping of the supply wells and L their screens.

| y=HMup;ξ+e, e∼N0,Γe, pp∣y,ξ∝py∣p,ξpp | (44) |
| --- | --- |

where u is the model state, HM picks the chosen observations, and the errors are 0.2 m for heads and 0.1 in log10 chloride in the Fushë-Kuqe designs.

## 7.2 Goal-oriented expected information gain (base paper)

The base paper [1] chooses monitoring wells by the expected information gain about a goal g (the intruded area (12)), linearised with the data Jacobian GM and the goal Jacobian H obtained by adjoints [33, 34]:

| EIGM=12lndetHΓH⊤detHΓH⊤−IF, IF=HΓGM⊤GMΓGM⊤+Γe−1GMΓH⊤ | (45) |
| --- | --- |

where Γ=ΓF of (4) (src/eig_operator.m). Recharge sites are ranked by the adjoint −∂A/∂Qk=λk with A0⊤λ=∂A/∂h in one solve. Top-k cells at least 6 km apart in northing give the published monitoring design P21, A12, A13, A21, A10 and recharge design A18, P21, P16 (full-margin boundary).

## 7.3 PRAG-GO: information about the risk at the supply wells

PRAG-GO [35] replaces the area by the binary event that a supply well is salinised and chooses the monitoring set that is most informative about it:

| EIGM=Hbq−Ey HbPrq=1∣y, q=1minwhw<hc | (46) |
| --- | --- |

where Hb is the binary entropy and q the prior probability; the closed form approximates Prq=0∣y=Φ2μA+z1−hc/sA,μB+z2−hc/sB;r with z∼N0,KM, KM=CΣy−1C⊤ and the posterior moments of the two heads (experiments/prag_go_fushe_kuqe/fk_prag_geo.py). With the geolocated boundary the best three wells are A3, A10 and A23 (0.428 and 0.402 nats for one and two supply wells), which the closed form finds, while the area criterion does not.

## 7.4 PRAG-MAR and VENDI-GO: where to recharge

Because the head is linear in every recharge rate, the risk that a supply well is salinised and its gradient follow from one adjoint per supply well [35]:

| PAB=1−Φ2a,b;r, a=μA−hcsA, b=μB−hcsB | (47) |
| --- | --- |

where μw,sw are the mean and standard deviation of the pumped head at supply well w and r their correlation; the gradient with respect to the rate qk of a recharge site is

| −∂PAB∂qk=Φ′asA Φb−ra1−r2gA,k+Φ′bsB Φa−rb1−r2gB,k | (48) |
| --- | --- |

where gw,k=E∂hw/∂qk with gw=A−⊤ew (src/prag_mar.m; fk_mar_geo.py). With three sites of 5,000 m³/d the probability that A1 or P10 is salinised falls from 0.560 to 0.058 (PRAG-MAR sites), 0.035 (exact greedy search), 0.305 (area adjoint) and 0.526 (published sites).

VENDI-GO [8] adds the value of testing a site before building it, with a knowledge gradient and a cost per test:

| KGk=V0−EykminSPS∣yk, J=P+κ ntest, test while maxkKGk>κ | (49) |
| --- | --- |

where κ=0.01 is the cost of a test and there are 226 candidate sites; on 200 truths the risk is 0.159 with 4.96 tests, against 0.228 for PRAG-MAR and 0.145 for an oracle (experiments/vendi_go).

All placements obey the same constraints:

| yk−yl≥6 km k≠l∈S or M, xk−xw≥1 km k∈S, w supply, j∈M∪S​cj≤B | (50) |
| --- | --- |

where the three constraints are the spacing in northing, the exclusion around supply wells and a budget B with costs cj (src/greedy_design.m).

Every one of these objectives decides salinisation by the head proxy (11). None of them uses chloride, sodium or the chemistry of Section 4. Section 8 closes that gap.

# 8. PUSI-GO: the hydrochemistry in the objective

## 8.1 Three propositions

The question is what a supply well must protect: not a head, but the water it delivers. In the reduction R2 the chloride of a screen is a function of the potential at the well, and the potential is linear in every well rate. This structure carries over the cheapness of PRAG-MAR to the drinking-water standard.

Proposition P1 (the standard is a threshold on a linear quantity). In the Strack model (13) with the Robin sea condition (32), let supply wells pump Qj and recharge wells inject qk. Then

| ϕwp;Q,q=ϕw0p−j​Qj Gwjp+k​qk Gwkp, G=Ap−1=G⊤ | (51) |
| --- | --- |

where ϕ0 is the potential without wells and A the symmetric matrix of (13) with (32), and, with Clw the chloride averaged over the screen of well w,

| Clw>Cllim=ϕw<ϕw⋆p, hw<hc=ϕw<ϕhp, ϕh=hc−δd22δ δd<hc | (52) |
| --- | --- |

where ϕw⋆ is the potential at which the screen chloride equals the limit. The chloride standard and the head proxy are the same event on the same linear functional; they differ only in the threshold, and only ϕw⋆ depends on the chemistry (Cls, Clf, w) and on the screen.

Proof. Equation (13) and condition (32) are linear in ϕ with the well rates as point sources, and the face conductances of the finite-volume operator are symmetric, so G is symmetric and the superposition holds exactly. In the wedge zone of (14) ζ=d+2ϕ/δ increases with ϕ; beyond it ζ=h/δ with h increasing in ϕ; the branches meet at ζ=d+B. For every depth z, cz=Φz−ζ/w decreases with ζ, so the screen average (15) and the chloride (16) are non-increasing in ϕ, and the event {Clw>Cllim} is a lower set in ϕw. The head is increasing in ϕ in both zones, which gives the second identity; solving δd+2δϕ=hc gives ϕh. □

Proposition P2 (one threshold for every well, set by the mixing zone). Let a well be screened over the top L metres of the aquifer, B≥L, and let the threshold lie in the wedge zone. Then

| ϕ⋆=δ2 w2 σ⋆2, 1λGλ−σ⋆−G−σ⋆=εlim, λ=Lw, εlim=Cllim−ClfCls−Clf | (53) |
| --- | --- |

where λ is the screen length in units of the mixing width; ϕ⋆ therefore depends only on the mixing width w, on Cls and on the screen length L: not on the conductivity, the recharge, the top depth, the thickness or the location of the well. All supply wells with the same screen share one random threshold.

Proof. With z1=d, z2=d+L and, in the wedge zone, ζ=d+s with s=2ϕ/δ, the screen average (15) is w/LGL−s/w−G−s/w: the top depth d cancels and B enters only through minL,B=L. Setting it equal to εlim and writing s=wσ⋆ gives (53); the left side decreases in σ⋆, so the root is unique. □

Proposition P3 (chloride binds, and sodium is bounded behind a recharge well). (i) While sea water intrudes and the exchanger takes up sodium (ΔNa≤0 in (28), the signature of wells 503 and 341), sodium at the moment chloride reaches 250 mg/L is at most Naf+εlimNas−Naf≈150 mg/L, below 200 mg/L; by mixing alone sodium would reach its limit at a seawater fraction of 1.50%, chloride at 1.09%. (ii) Behind the chloride front of a recharge well, with conservative anions and no other source of sodium,

| Na≤MNai∈cations​ziciin | (54) |
| --- | --- |

where ciin is the recharge-water composition (mmol/L) and MNa the molar mass: 4.47 meq/L, i.e. 103 mg/L, for well-193 water. The sodium standard therefore holds behind any recharge well whose water carries less than 8.70 meq/L of cations. (iii) Hardness is not protected: the exchanger releases sodium for calcium and magnesium, and the freshened water is soft and sodic for a number of pore volumes that grows with E and with the seawater fraction left by the intrusion.

Proof. (i) Na=Naf+fNas−Naf+ΔNa by (28), and f=εlim when chloride is at its limit. (ii) In every cell ziyi+Eβi=Ti with ∑βi=1, so i​ziyi=i​Ti−E: exchange changes which cations are dissolved, never their total charge. Advection replaces the dissolved cation charge of a cell by that of the cell upstream and leaves the exchanger in place, so the dissolved cation charge is transported conservatively and equals that of the recharge water behind the front; sodium is part of it. □

## 8.2 The PUSI-GO objective

With (52) the complete placement problem keeps the structure of Section 7, with the chemistry in the event:

| maxξ Eyw​Qw⋆y−λRk​qk−λMM | (55) |
| --- | --- |

where Q⋆y is the largest pumping whose posterior risk of exceeding the chloride standard in any supply screen is at most α (10% here) and λR,λM are prices of recharged water and of observations; the maximum is taken subject to the placement constraints (50) and to

| Pr∃w: ϕwp;Q⋆,q<ϕw⋆p;Lw∣y,ξ≤α, i∈cations​ziciinj≤NalimMNa | (56) |
| --- | --- |

The second constraint is Proposition P3 (ii). The problem splits into three sub-problems, which are solved here. The recharge sites minimise the probability that any supply well exceeds the standard:

| S⋆=arg minS Prminwϕw−ϕw⋆+k∈S​qkGwk<0 | (57) |
| --- | --- |

The closed form (47)–(48) applies to it with the margin ϕw−ϕw⋆ in place of hw−hc. The observations maximise the mutual information with the chemistry event:

| M⋆=arg maxM I1∃w:ϕw<ϕw⋆; yM | (58) |
| --- | --- |

The information is estimated by likelihood reweighting of the ensemble, as in (46). The pumping is the safe yield given the data:

| Q⋆y=maxs Q0: Pr∃w: ϕwsQ0<ϕw⋆∣y≤α | (59) |
| --- | --- |

where Q0 is a reference pumping pattern and s≥0 its scale.

## 8.3 The test

The test uses the reduction R2 on the 20,000 calibrated draws of the chloride study, weighted by their likelihood; even draws design (effective sample size 1616) and odd draws evaluate (1634). The supply wells are A1 and P10, one in each half of the sector as in PRAG-MAR, screened over the upper 40 m (the first horizon; the mid-depth screens are already above 250 mg/L in more than 90% of draws, as documented). They pump 366 and 11694 m³/d, the rates at which each well's own probability of exceeding the standard is one half (the PRAG-MAR rule, now on chemistry). One factorisation per draw gives the potential and, by the symmetry of P1, the response of both supply wells to recharge or pumping anywhere (experiments/pusi_go).

Table 6. Part A: the two events at the supply wells (evaluation half). Either well fails: chemistry 0.594, head proxy 0.044, tuned head 0.541.

| Quantity | A1 | P10 |
| --- | --- | --- |
| Head without pumping, median (m a.s.l.) | 3.5 | 4.2 |
| Head with pumping, median (m a.s.l.) | 3.2 | 3.2 |
| P(shallow Cl > 250 mg/L) without pumping | 0.225 | 0.003 |
| P(Cl > 250) with pumping: chemistry threshold | 0.489 | 0.488 |
| P(h < 1 m) with pumping: head proxy | 0.000 | 0.044 |
| P(h < tuned head) with pumping: head threshold tuned to the same level | 0.483 (h < 3.17 m) | 0.473 (h < 3.14 m) |
| Tuned head and chemistry give the same verdict in | 49% | 71% |
| Threshold ϕ⋆, 5 / 50 / 95% (m²) | 30 / 66 / 259 | identical (P2) |
| Head-proxy threshold ϕh, 5 / 50 / 95% (m²) | 0.07 / 0.25 / 0.57 | 0.14 / 0.39 / 0.76 |
| Rank correlation of ϕ⋆ with w; correlation of ϕ⋆ with ϕ | 0.99997; 0.011 | 0.99997; 0.013 |

Figure 2. PUSI-GO on the calibrated Fushë-Kuqe ensemble. (a) Each point is an aquifer: potential at A1 under pumping against its chemistry threshold; points below the line exceed 250 mg/L; colours compare with the head threshold tuned to the same risk. (b) Probability that A1 or P10 fails with three recharge wells, by the chloride standard and by the head proxy. (c) Expected information about the chemistry event; dashed: its entropy. (d) Mean pumping of A1 + P10 allowed at a posterior risk of 10% with 20-m screens; in brackets the realised failure frequency. (e)–(f) A mixing-cell column freshened by well-193 water after intrusion to 5% sea water (E=60 meq/L): chloride and sodium (e), hardness and the duration of the soft tail for three exchange capacities (f).

## 8.4 What the test shows

- The head proxy sees almost no risk where the standard fails in most aquifers. Under pumping the median head at both supply wells is above 3 m, so the head proxy gives 0.044 for either well; the chloride standard gives 0.594. The proxy was built for a 40-m-deep base; the calibrated wedge is up to 200 m thick, and its mixing zone reaches the screen long before the head falls to 1 m (the proxy threshold is below 1 m², the chemistry threshold around 66 m²).

- Even a head threshold tuned to the right risk picks the wrong aquifers. A head threshold of 3.17 m gives the same risk as the standard, yet the two agree in only 49% of aquifers at A1 and 71% at P10 (Figure 2a). By Proposition P2 the chemistry threshold is a function of the mixing width (rank correlation 0.99997) and is uncorrelated with the potential at the well (0.011): heads do not carry the information that decides the standard.

- Where to recharge: the tuned threshold suffices, the proxy does not. The chemistry objective and the tuned head threshold choose the same three sites, (10.00, 24.00), (4.00, 36.00), (9.00, 30.00) km, the cells that the exact search had found with the head proxy in the design model (Section 7.4). The head proxy itself cannot rank designs here, because its risk is already near zero, and keeps the PRAG-MAR sites (3.25, 34.25), (8.75, 28.25), (9.75, 40.25) km, which leave 2.3 ± 0.2 percentage points more risk (Table 7). The proxy reports the problem solved (0.000); the standard still fails in 25% of aquifers with the best three sites, and reaching 10% needs 8,516 m³/d per site (the tuned threshold says 6,611, the proxy none).

Table 7. Part B: probability that A1 or P10 exceeds its event, evaluation half. Search: greedy selection refined by a multi-start one-site swap (27 starts) on the design half under the constraints (50). The closed forms of all three thresholds return the PRAG-MAR sites.

| Recharge design (3 × 5,000 m³/d) | Sites (km) | P, chloride standard | P, head proxy | P, tuned head |
| --- | --- | --- | --- | --- |
| none | – | 0.594 | 0.044 | 0.541 |
| head proxy (PRAG-MAR sites; closed form and search) | (3.25, 34.25), (8.75, 28.25), (9.75, 40.25) | 0.270 | 0.000 | 0.214 |
| tuned head threshold (search) | (4.00, 36.00), (9.00, 30.00), (10.00, 24.00) | 0.247 | 0.001 | 0.188 |
| PUSI-GO: chemistry threshold (search) | (10.00, 24.00), (4.00, 36.00), (9.00, 30.00) | 0.247 | 0.001 | 0.188 |

- What to measure: chloride, not heads. The chemistry event has an entropy of 0.669 nats. The best three head observations give 0.089 nats (PRAG-GO's three wells 0.038); one shallow chloride sample at A30 gives 0.312, and the best two observations of any type (shallow chloride at A30, mid-depth chloride at P12) give 0.394 (Table 8). Chloride informs the mixing width, which is the threshold.

- The value in water: chloride doubles the safe yield. With 20-m screens and a posterior risk of at most 10%, the supply wells can pump 4,221 m³/d on average without data, 5,349 with PRAG-GO's heads, 6,336 with the best three heads, 8,417 with one chloride sample and 9,324 with the best two observations. The head proxy, which knows no chemistry, would pump 13,869 m³/d and exceed the standard in 46% of aquifers. The realised failure frequencies of the data-driven rules are 11–16%, slightly above the 10% target because the posterior is computed from a finite weighted ensemble.

Table 8. Part C: information about the chemistry event (40-m screens, nested estimator with 2,000 outer and 10,000 inner draws; inner ESS: median effective inner sample size) and the safe yield of the adaptive rule Q⋆y of (59) with 20-m screens and α=10%.

| Observations | Information (nats) | Inner ESS | Safe yield (m³/d) | Realised failure |
| --- | --- | --- | --- | --- |
| none | 0 | – | 4,221 | 10.8% |
| PRAG-GO heads A3, A10, A23 | 0.038 | – | 5,349 | 12.7% |
| best 3 heads (P12, A13, A3) | 0.089 | 23 | 6,336 | 15.4% |
| 1 shallow chloride (A30) | 0.312 | 306 | 8,417 | 12.4% |
| best 3 shallow chloride (A30, P4, A9) | 0.322 | 226 | 8,642 | 12.6% |
| best 2 of any type (Cl A30 shallow, Cl P12 mid) | 0.394 | 85 | 9,324 | 13.2% |
| head proxy rule (no data) | – | – | 13,869 | 46.4% |

- How to screen. Shortening the screen is itself a placement decision: with the calibrated pumping the risk is 28%, 36% and 59% for 10-, 20- and 40-m screens, and the largest pumping with a risk of 10% is 7,875, 3,691 and 0 m³/d (A1's 40-m screen already fails in 23% of aquifers without pumping). The head proxy would allow 14,250 m³/d whatever the screen.

- Which standard binds. In the exchange column (Table 9) sodium is only 37–78 mg/L when chloride reaches 250 mg/L, and after freshening it never exceeds 98 mg/L, inside the bound of 103 mg/L of Proposition P3: chloride is the standard to design for. But the freshened water is soft (Ca + Mg down to 0.11–0.69 meq/L against 3.65 for the recharge water) and sodic (sodium adsorption ratio up to 18) for 0.8–8.5 pore volumes, which matters for irrigation and for the corrosivity of the supply, and should be part of the aftercare of a recharge site placed in a formerly intruded zone.

Table 9. Part D: Gaines–Thomas mixing-cell column (100 cells, selectivities 0.2/0.4/0.5) with well-193 and Adriatic end-members (experiments/pusi_go/pusi_exchange.py). PV: pore volumes of the flow path after the chloride front.

| Exchange capacity E (meq/L) | Sea fraction left | Na when Cl reaches 250 (mg/L) | Na max after freshening (mg/L) | Min Ca+Mg (meq/L) | Soft tail (PV) | Hardness back to 90% (PV) |
| --- | --- | --- | --- | --- | --- | --- |
| 30 | 2% | 37 | 85 | 0.69 | 0.8 | 1.8 |
| 30 | 5% | 56 | 95 | 0.25 | 1.3 | 2.8 |
| 30 | 10% | 78 | 98 | 0.11 | 2.1 | 3.3 |
| 60 | 2% | 37 | 85 | 0.69 | 1.6 | 2.6 |
| 60 | 5% | 56 | 95 | 0.25 | 2.7 | 4.7 |
| 60 | 10% | 77 | 98 | 0.11 | 4.3 | 5.6 |
| 120 | 2% | 37 | 85 | 0.69 | 3.2 | 4.3 |
| 120 | 5% | 56 | 95 | 0.25 | 5.3 | 8.3 |
| 120 | 10% | 77 | 98 | 0.11 | 8.5 | 10.2 |

## 8.5 What it means, and the limits

For Fushë-Kuqe the order of work is: protect the first horizon, because the second is already salinised at the coast; site recharge by the probability that the supply screens exceed the chloride standard, which in this model picks the cells (10.00, 24.00), (4.00, 36.00), (9.00, 30.00) km and needs about 8,516 m³/d per site to bring the risk to 10%; measure chloride in the shallow screen of a well between the supply wells and the coast (A30), which is worth more than any set of three heads and roughly doubles the pumping that can be allowed; screen the supply wells short; and use recharge water with less than 8.7 meq/L of cations, expecting soft, sodic water for a few pore volumes where the recharge freshens an intruded zone. The limits: the chloride model is steady and areal with a sharp interface smoothed by a Gaussian mixing zone, so transients, the vertical structure of two horizons and the exchange in the aquifer are not in the event; the posterior comes from likelihood weighting on a handful of documented facts; the supply wells and their rates follow the PRAG-MAR rule, not records; the information estimates rest on finite inner samples (effective size down to 85 for two observations); and the exchange column omits calcite and sulfate reduction.

# 9. The system at a glance

The complete system is: flow (5)–(8) with wells (9); transport (19)–(21) with exchange (22)–(23), calcite (24), sulfate reduction (25), activities (26) and electro-diffusion (27); the invariants and indicators (28); the boundary conditions (29)–(38) and the initial conditions (39)–(42); and the placement objectives (43)–(50) with the PUSI-GO objective (51)–(59). Four reductions are computed: R1 (10)–(12), R2 (13)–(16), R3 (17) and R4 (18) with (19)–(28). Seven points of consistency remain open in the repository:

- two sea boundaries: the published MATLAB design model fixes h=0 on the whole western margin, the geolocated models use the Robin link (32) on 23 km of coast; two of the published recharge sites lie outside the geolocated sector;

- two geometries: a 30-m aquifer with a 40-m base (R1, and hence the head proxy) against a wedge 180–200 m thick at the coast (R2); PUSI-GO resolves the conflict for placement by using the chemistry threshold;

- the density contrast 0.025 against 0.0285 implied by the Adriatic density of 1.0285;

- uniform recharge, no rivers, lagoon or leaky base in any field code (Table 4);

- the cross-facies correlation 0.6 of the design prior against the field estimate 0.805;

- the Henry benchmark imposes sea water by diffusion also on outflow, unlike (33);

- monitoring and recharge site names are the nearest K-wells to the chosen cells, some of them far away (P16 is 10 km from its cell).

The next step is to solve the three-dimensional system (5)–(7) with the seven ions (19)–(23) and the boundary conditions of Section 5 for the Fushë-Kuqe sector, calibrated to the chloride series of 1984–2001, and to evaluate the PUSI-GO objective on it; Propositions P1 and P3 then hold approximately rather than exactly, and the threshold of P2 becomes a property of the transition zone that chloride profiles can measure directly.

Prepared on 26 October 2026 from the code and results of the author's repository (folders experiments/prag_go_fushe_kuqe, chloride_fk, experiments/gjurma_go, ritmi_go, fillimi_go, kufiri_go, vendi_go and the new experiments/pusi_go); every number about PUSI-GO is read from experiments/pusi_go/res and checked when the document is built. References marked "not verified" were not checked against the publisher.

# References

[1]	Zeqiraj, D. (2026). Goal-oriented Bayesian experimental design for variable-density seawater intrusion: adjoint expected information gain for joint monitoring and managed-aquifer-recharge placement. Journal of Hydrology, 680, 136559. https://doi.org/10.1016/j.jhydrol.2026.136559

[2]	Zeqiraj, D. (in preparation). From sparse hydrochemistry to decision-relevant information: probabilistic chloride mapping and goal-oriented monitoring design for seawater intrusion in the Fushë-Kuqe coastal aquifer, Albania. Code: chloride_fk in the author's repository.

[3]	Zeqiraj, D. (2026). Missing dynamics in coastal-aquifer flow and multi-species transport: 30 papers for a COST candidate system of equations. Literature review 42 of the author's series (23 October 2026), with the numerical experiment GJURMA-GO.

[4]	Zeqiraj, D. (2026). The transient state: flow and multi-species transport with memory, and what time-dependent data identify. Literature review 43 of the author's series (23 October 2026), with the numerical experiment RITMI-GO.

[5]	Zeqiraj, D. (2026). The trace and the rhythm: an augmented transient system of variable-density flow and multi-species transport for coastal aquifers, and what chemistry and time-dependent data can identify. Synthesis article of reviews 42 and 43 (24 October 2026).

[6]	Zeqiraj, D. (2026). Boundary and initial conditions: the mathematics and computation of what a model is given. Literature review 44 of the author's series (24 October 2026), with the numerical experiment FILLIMI-GO.

[7]	Zeqiraj, D. (2026). All the boundary and initial conditions of a coastal aquifer: sea, atmosphere, soil moisture, rivers, pumping regimes and the initial state, as stochastic and causal objects in the augmented transient system. Literature review 45 of the author's series (24 October 2026), with the numerical experiment KUFIRI-GO.

[8]	Zeqiraj, D. (2026). Bandits and the value of information for siting managed aquifer recharge: which pilot tests to make, when to stop and where to build. Literature review 53 of the author's series, with the numerical experiment VENDI-GO.

[9]	Zeqiraj, D. (2026). Uncertainty, risk, robustness and uniqueness of a managed-aquifer-recharge scheme: new methods from Caltech and Oxford. Literature review 54 of the author's series, with the numerical experiment SIGURIA-GO.

[10]	Cenameri, S. & Beqiraj, A. (2016). Assessment of seawater intrusion in Fushë-Kuqe aquifer, Albania. Bulletin of the Geological Society of Greece, 50(2), 665–670. https://doi.org/10.12681/bgsg.11772

[11]	Council of the European Union (1998). Council Directive 98/83/EC of 3 November 1998 on the quality of water intended for human consumption. Official Journal of the European Communities, L 330; indicator parameters chloride 250 mg/L and sodium 200 mg/L (page numbers not verified).

[12]	Voss, C. I. & Souza, W. R. (1987). Variable density flow and solute transport simulation of regional aquifers containing a narrow freshwater–saltwater transition zone. Water Resources Research (volume, pages and DOI not verified).

[13]	Werner, A. D., Bakker, M., Post, V. E. A., Vandenbohede, A., Lu, C., Ataie-Ashtiani, B., Simmons, C. T. & Barry, D. A. (2013). Seawater intrusion processes, investigation and management: Recent advances and future challenges. Advances in Water Resources, 51, 3–26. https://doi.org/10.1016/j.advwatres.2012.03.004

[14]	Hantush, M. S. (1960). Modification of the theory of leaky aquifers. Journal of Geophysical Research (volume, pages and DOI not verified).

[15]	Haggerty, R. & Gorelick, S. M. (1995). Multiple-rate mass transfer for modeling diffusion and surface reactions in media with pore-scale heterogeneity. Water Resources Research (volume, pages and DOI not verified).

[16]	Peaceman, D. W. (1978). Interpretation of well-block pressures in numerical reservoir simulation. Society of Petroleum Engineers Journal (volume, pages and DOI not verified).

[17]	Strack, O. D. L. (1976). A single-potential solution for regional interface problems in coastal aquifers. Water Resources Research, 12(6), 1165–1174. https://doi.org/10.1029/WR012i006p01165

[18]	Millero, F. J., Feistel, R., Wright, D. G. & McDougall, T. J. (2008). The composition of Standard Seawater and the definition of the Reference-Composition Salinity Scale. Deep-Sea Research Part I (volume, pages and DOI not verified).

[19]	Bear, J. (1979). Hydraulics of Groundwater. McGraw-Hill (details not verified).

[20]	Gaines, G. L. & Thomas, H. C. (1953). Adsorption studies on clay minerals. II. A formulation of the thermodynamics of exchange adsorption. The Journal of Chemical Physics (volume, pages and DOI not verified).

[21]	Appelo, C. A. J. & Postma, D. (2005). Geochemistry, Groundwater and Pollution, second edition. Balkema (pages not verified).

[22]	Parkhurst, D. L. & Appelo, C. A. J. (2013). Description of input and examples for PHREEQC version 3. U.S. Geological Survey Techniques and Methods, book 6, chapter A43 (details not verified).

[23]	Plummer, L. N., Wigley, T. M. L. & Parkhurst, D. L. (1978). The kinetics of calcite dissolution in CO2–water systems at 5° to 60 °C and 0.0 to 1.0 atm CO2. American Journal of Science (volume, pages and DOI not verified).

[24]	Wigley, T. M. L. & Plummer, L. N. (1976). Mixing of carbonate waters. Geochimica et Cosmochimica Acta (volume, pages and DOI not verified).

[25]	Davies, C. W. (1962). Ion Association. Butterworths (details not verified).

[26]	Zeqiraj, D. (2026). Transport of charged species in water and porous media: from structural electroneutrality to hydrogeochemistry. Literature review 30 of the author's series.

[27]	Li, Y.-H. & Gregory, S. (1974). Diffusion of ions in sea water and in deep-sea sediments. Geochimica et Cosmochimica Acta (volume, pages and DOI not verified).

[28]	Zeqiraj, D. (2026). The electrodiffusive leak of the cut: how much error of a reactive closure can reach the chloride forecast through ion coupling. Research note of the author's series (Theorem 2, experiment LIDHJA-GO).

[29]	Stuyfzand, P. J. (2008). Base exchange indices as indicators of salinization or freshening of (coastal) aquifers. Proceedings of the 20th Salt Water Intrusion Meeting, Naples, Florida (details not verified).

[30]	Jacob, C. E. (1950). Flow of ground water. In H. Rouse (ed.), Engineering Hydraulics. Wiley (pages not verified).

[31]	Priestley, C. H. B. & Taylor, R. J. (1972). On the assessment of surface heat flux and evaporation using large-scale parameters. Monthly Weather Review (volume, pages and DOI not verified).

[32]	Emerick, A. A. & Reynolds, A. C. (2013). Ensemble smoother with multiple data assimilation. Computers & Geosciences (volume, pages and DOI not verified).

[33]	Alexanderian, A. (2021). Optimal experimental design for infinite-dimensional Bayesian inverse problems governed by PDEs: a review. Inverse Problems, 37(4), 043001. https://doi.org/10.1088/1361-6420/abe10c

[34]	Attia, A., Alexanderian, A. & Saibaba, A. K. (2018). Goal-oriented optimal design of experiments for large-scale Bayesian linear inverse problems. Inverse Problems, 34(9), 095009. https://doi.org/10.1088/1361-6420/aad210

[35]	Zeqiraj, D. (2026). PRAG-GO and PRAG-MAR at Fushë-Kuqe: monitoring and managed aquifer recharge oriented to the intrusion risk of supply wells, recomputed with the geolocated sea boundary. Research note of the author's series.