DIAGRAM 1  ·  TRANSFER BETWEEN AQUIFERS  ·  BUILD ONCE

A model that can travel: one operator for the whole family of coastal aquifers

| S · the studied aquifer (Fushë-Kuqe) Variable-density flow with aquitard memory and seven-ion transport; seven boundary classes; assimilation of boundary and initial conditions, with regimes for extreme events. | generalise → | The family of coastal aquifers Same equations and boundary classes, any thickness, conductivity, inflow, anisotropy, dispersivity and heterogeneity; S is one of its members. |
| --- | --- | --- |

▼

| 1 | Map the boundaries of T onto the seven classes of Fushë-Kuqe | ∂ΩT=Γsea∪Γtop∪Γriv∪Γdrn∪Γin∪Γbase∪Γw hf=ρsρf ηt−ρs−ρfρf z on Γsea qr=Crhr−maxh,zb on Γriv, qd=−Cdh−zd+ on Γdrn | Every boundary of T has a class: go on. A boundary with no class (a karst conduit, a tidal river, a sealing fault): the family does not cover T. Add that class to the simulations first. |
| --- | --- | --- | --- |
| ▼ |  |  |  |
| 2 | Make the problem dimensionless units: thickness d, inflow from land qb, conductivity KG | x⋆=xd, t⋆=qb tϕ d, h⋆=KG hqb d s ∂h⋆∂t⋆+∇⋆⋅q⋆=0, q⋆=−K⋆∇⋆h⋆+1a c ez ∂c∂t⋆+q⋆⋅∇⋆c=∇⋆⋅D⋆∇⋆c K⋆=diag1,rKeY⋆x⋆ D⋆=b+αTdq⋆I+αL−αTd q⋆q⋆⊤q⋆ a=qb ρ0KG Δρ, b=Dm ϕqb d, rK=KzKx, ξ=Ld, s=Ss qb dϕ KG | All boundary data of T in the same units: η⋆=KGη/qbd, R⋆=R/qb, g⋆=g d/KG, Q⋆=Q/qbd2, τ⋆=qbτ/ϕd. The storage number s is added for tides, surges and pumping; s=0 gives the family of proposal R1, in which heads adjust at once. |
| ▼ |  |  |  |
| 3 | One point per aquifer the descriptor z | z=θ, Y⋆, Y⋆=lnKxKG=k​ζk ψkx⋆ θ=a, b, rK, ξ, αLd, αTd, σY2, λd, s G†: z↦cx⋆,t⋆ | Two aquifers that look different in metres and days can have the same θ; with the same geometry and boundary classes the homogeneous transfer is then exact. The norm ⋅Z joins the normalised θ and the Karhunen–Loève coefficients ζk; λ is the correlation length of Y⋆. |
| ▼ |  |  |  |
| 4 | Learn the operator once on simulations of the family: FNO, DeepONet or kernels | Gw≈G† on zi, G†zii=1N εtr=maxiG†zi−Gwzi N≈C deff 1+4αlog1δ, deff=minp: k>p​μk≤ε2k​μk L†≈maxi,j near zG†zi−G†zjzi−zjZ | μk: PCA spectrum of the simulated concentration fields. The size N is a starting point (the bound is proved for Darcy flow); the empirical learning curve decides. The local Lipschitz constants L†, Lw come from finite differences on the simulations; Fushë-Kuqe itself is one of the training points. |

▼

OUTPUT OF DIAGRAM 1  The operator Gw, its training points zi, its training error εtr and the local constants L†, Lw. They are computed once and reused for every new aquifer (Diagram 2).

Steps 2–4 follow proposal R1; step 1, the storage number s and the local constants are added here. Check the exact definitions of a and b against Abarca et al. (2007), Advances in Water Resources 30, 913–926 (not verified) before publication.

DIAGRAM 2  ·  TRANSFER BETWEEN AQUIFERS  ·  FOR EVERY NEW AQUIFER

Certify, assimilate, watch: using the model at a new aquifer T

| From Diagram 1 The operator Gw, the training points zi, εtr, L† and Lw. | apply → | T · a new coastal aquifer Its geometry and boundaries (step 1 passed) and its own records: heads, tide gauge, chloride, meters, rainfall. |
| --- | --- | --- |

▼

| 5 | Place T and certify the transfer distance, uncertainty, decision | dT=min1≤i≤NzT−ziZ, uT=zT−zTZ G†zT−GwzT≤BT BT=εtr+L†+LwdT+L†uT | Decision: is BT≤ε⋆? Yes ▼ transfer; go to 6. No ↺ add simulations near zT (back to 4), or measure the group j with the largest Lj†uT,j. L† is large near a change of regime (small a: the toe moves fast inland); there the transfer is risky. uT: 90% radius of the descriptor estimated from the data of T. |
| --- | --- | --- | --- |
| ▼ |  |  |  |
| 6 | Assimilate at T the certificate becomes model error | XT=v, b, γ, zT, Cf←Cf+BT2 I Xa=Xf+CXyCyy+Γ−1y+e−Y μn+1j∝μj Nνn+1j;0,Sn+1j | The descriptor is a state of the assimilation: every datum of T shrinks uT and with it BT. Regimes j: normal, surge, drought or flood. Boundary priors, regimes and the initial state are re-learned at T (Diagram 3). |
| ▼ |  |  |  |
| 7 | Watch the transfer alarm and coverage, while it runs | NISn=νn⊤Sn−1νn>χm,1−α2 under every regime 1Mj=1M1xj−xj≤1.645 σj≈0.90 | False alarms occur at exactly the rate α in any aquifer (T9). Coverage is counted on twins of T, by regime. An alarm under every regime, or a drop in coverage: the transfer is broken. ↺ Back to 5 with the updated zT, or add a regime or a closure. |

INNOVATION · KALIMI-GO  (kalimi: the passage). The transfer certificate is more than a yes or a no: it becomes the model error of the filter of the new aquifer, Cf+BT2I. The descriptor zT is a state of the assimilation, so each datum of T shrinks the certificate, and the alarm of Theorem 9 watches the transfer while it runs. For a Gaussian forecast whose bias stays within BT, the widened 90% interval covers the truth with probability at least 89.99% for every ratio of bias to noise (checked numerically); for pointwise intervals the certificate is taken in the maximum norm.

THREE NUMBERS TO COMPUTE FIRST  (1) The skin distance xs=2Tτ/S of a boundary event of time scale τ, against the first piezometer: in the test aquifer of the note xs=200 m < x1=250 m, and heads alone recognised only 37% of the surges, so a tide gauge is needed. (2) The slowest rate ω1 of the aquifer, which fixes how long the earliest record must be. (3) The certificate BT against ε⋆, which says whether the operator can be used at all.

T, P, L: theorems, propositions and lemmas of the note Updating the boundary and initial conditions of the Fushë-Kuqe system by data assimilation (October 2026). The uncertainty term L†uT and steps 6 and 7 are added to proposal R1 here.

DIAGRAM 3  ·  TRANSFER BETWEEN AQUIFERS  ·  WHAT CARRIES OVER

Carried over, rescaled, re-learned: each boundary and initial condition, with the check to run at the new aquifer

| Component | Carries over unchanged | Rescaled into the units of T | Re-learned from the data of T | Check at T |
| --- | --- | --- | --- | --- |
| 1 Sea face Dirichlet or Robin | type of condition; hydrostatic sea head; surge state with its own decay (T8) | η⋆=KGηqbd, g⋆=g dKG, τs⋆=qbτsϕd | offset η0, trend, ση, τη from the tide gauge of T; sea-bed conductance g | skin distance xs=2Tτ/S against the first piezometer x1: if x1>xs, a tide gauge is needed |
| 2 Top: recharge flux | flux condition; mean plus Ornstein–Uhlenbeck anomaly; drought or flood regime | R⋆=Rqb, τR⋆=qbτRϕd | R, δR, σR, τR; the recharge law from soil moisture | a chloride mass balance separates δR from a uniform closure (T5) |
| 3–4 Rivers, drains, seepage switching | monotone switches maxh,zb, h−zd+; stable in any aquifer (L2) | z⋆=KGzqbd, C⋆=C dKG | bed and drain levels, conductances (ES-MDA across a switch) | δhn≤δh0+k​ΔtMS−1δbk |
| 5 Inland margin inflow | type of condition; the inland inflow defines the unit | qin⋆=1 | the inflow qb itself, from the water balance of T | the error of qb enters uT through every group |
| 6 Base, aquitards leaky, with memory | Markovian lift: the memory is a set of extra states | ak⋆=akϕdqb, κ⋆=κdKG | clay legacy hk,0, leakage κ | the slowest rate ω1 of MS−1A sets how long the record must be |
| 7 Wells internal sink | unlicensed pumping E as a state; its confounding with a closure (T5) | Q⋆=Qqbd2 | metered rates and E | principal angle between E and the closure, with the network of T |
| IC Initial conditions fields at t0 | prior P0=φM; smoother over the earliest data (T3); linear constraints kept (P6) | modes ψj and rates ωj of the operator of T | h0, c0, hk,0, β0 | mode j is learnt within t≈12ωj; homogeneous strip: ω1=π2T4SL2 |
| Regimes extreme events | regimes that carry the event's own dynamics, merged by moments (T8); spike model (P10) | τs⋆, νs⋆=νsϕdqb | transition matrix Π, storm rate νs, jump sizes, drought statistics from the records of T | coverage of 90% intervals by regime on twins of T |
| Sensors, alarm the network of T | false-alarm rate exactly α (T9) | — | power of the alarm with the sensors of T: Λn=dn⊤Sn−1dn | PWa=1−n∈W​Fχm2a2Λnχm,1−α2, before installing |
| Chloride observation | probit of the sea fraction; saturated samples censored (T7) | w⋆=wd, ζ⋆=ζd | width of the mixing zone, detection limits of the laboratory | distance of the predicted chloride from its Gaussian projection |
| C Missing dynamics closure | hat-function family; the closure is always included (T5) | centres and widths in x⋆ | coefficients γ | principal angles between closure and boundary responses |
| Operator Gw | trained once on the family (Diagram 1, step 4) | descriptor zT | zT, updated with the data (Diagram 2, step 6) | BT≤ε⋆ (Diagram 2, step 5) |

READ WITH DIAGRAM 2  Red column: what the assimilation of Diagram 2, step 6 must re-learn at T. Right column: the check that says the item has carried over. Asterisks mark quantities in the units of Diagram 1, step 2, which turn every boundary datum of T into the units of the family. In xs and ω1, T and S are the transmissivity and storativity of T and τ the time scale of the boundary event (two days for a surge); the damping of a periodic sea level with distance is the classical result of Ferris (1951). T, P, L: theorems, propositions and lemmas of the note Updating the boundary and initial conditions of the Fushë-Kuqe system by data assimilation (October 2026).