SCIENTIFIC LITERATURE SEARCH

Goal-oriented optimal experimental design in coastal aquifers

Theoretical introduction and 20 papers on saltwater intrusion monitoring, data worth, Bayesian OED and geophysics

Networks   ·   Data worth   ·   Bayesian OED   ·   Geophysics and management

CONTENTS  Part I: a theoretical introduction to goal-oriented design for saltwater intrusion: the variable-density equations and the sharp interface, management goals, classical network criteria, linear data worth, Bayesian model averaging, the goal EIG, nonlinear methods, geophysics and decisions under risk, with equations. Part II: 20 verified papers in four groups of 5: monitoring networks (N1–N5), data worth (V1–V5), Bayesian OED (D1–D5) and geophysics with management (G1–G5), each with a key equation in OMML and an innovation. Part III: a synthesis and an innovative proposal for the SWI-Goal-OED project: PRAG-GO (prag: threshold), a design oriented to the risk of exceeding the salinity threshold.

Structure of each card

- Bibliographic data: authors, journal, DOI or link, affiliation, study site and a rating of mathematical depth (★ 1–5).

- Summary: the problem, the method and the main result.

- Key equation: in OMML format, i.e., a Word math equation that can be edited with Insert → Equation.

- Link to the theoretical introduction and Innovation: where the paper fits in Part I and what it adds that is new.

Search date: October 11, 2026  ·  Period covered: 2010–2026, with priority to 2018–2026 (the base literature of Part I is older)

# 1. Theoretical introduction: goal-oriented design for saltwater intrusion

A theoretical introduction written for this document. It draws on the base literature (references B1–B14 at the end) and on the 20 papers of Part II. Sections 1.1–1.8 and 1.10 use standard results; the remarks that link linear data worth to the goal EIG (Sections 1.5 and 1.7) are derived here.

### 1.1 The physics of intrusion and variable-density models

In a coastal aquifer, seawater, about 2.5% denser than freshwater, enters beneath it as a wedge. Flow and salt transport are coupled through density [B2]. With the relative concentration c∈0,1 (0 freshwater, 1 seawater), Darcy's law, mass conservation and the transport equation are:

q=−kμ∇p+ρg∇z,  ∂ϕρ∂t+∇⋅ρq=ρsQs,  ρ=ρf1+εc, ε≈0.025

∂ϕc∂t+∇⋅qc−∇⋅ϕD∇c=csQs

Here k is the permeability, μ the viscosity, p the pressure, ϕ the porosity, D the dispersion tensor and Qs the sources. The Henry problem [B1] is the standard test; in the project, the toe of the wedge in this problem is at 1.3918 m. The coupling through ρc makes the map parameters → goal nonlinear, so Lemma 1 of the project holds after adjoint linearization.

The hypothetical salt and heat model [V5] and the KELM surrogate of the variable-density model [N2] show the two ways of handling the cost of these equations; CHERT monitoring [G2] shows the real time scales, from storms to drought.

### 1.2 The sharp interface: Ghyben–Herzberg and the Strack potential

When the mixing zone is thin, it is approximated by a sharp interface. Hydrostatic equilibrium gives the Ghyben–Herzberg relation: the interface lies α times deeper below sea level than the freshwater head rises above it. For an unconfined aquifer with its base at depth d, the Strack potential [B4] turns the problem into a Poisson equation:

ξ=αh, α=ρfρs−ρf≈40,  φ=12h+d2−1+ααd2 without interface,  φ=1+α2h2 above interface

∇2φ=−NK,  φtoe=1+αd22α2

The position of the toe of the wedge is obtained where φ=φtoe, i.e., as an analytical and differentiable goal. The SWI2 package of MODFLOW [B3] generalizes this to many layers; its speed allows large ensembles and many parameters.

SWI2 models allow calibration with pilot points and with geophysical observations of the interface [V2], optimization with chance constraints [V1] and climate ensembles [G1] in the same island aquifer.

### 1.3 Management goals: the toe of the wedge, the salt mass and thresholds

Goal-oriented design requires a quantity q=Qθ that the decision maker actually uses. Three goals are typical:

MsT=Ωg​ϕ cx,T dx,  xtoeT=maxx: cx,zb,T≥0.5,  qτ=Imaxw∈WcwT>τ

The salt mass in the zone Ωg (the project goal) and the position of the toe are continuous and differentiable quantities. The third goal, threshold exceedance in any of the supply wells W, is the one that regulations use: 250 mg/L chloride, about 1.3% of seawater (19 000 mg/L). It is binary and discontinuous, and this raises the question of Section 9: does the optimal design change when the goal becomes a risk?

The goals in the Part II papers are the interface elevation and the freshwater volume [V2], interface movement [V5], the wellhead protection area [D2] and the probability of exceeding a chloride threshold [G5].

### 1.4 Network design as a combinatorial problem and the classical criteria

The monitoring network is selected from N candidate sites with binary variables zi, under a budget [B12]. The intrusion literature uses four families of criteria J: (a) statistical criteria, such as entropy, the coefficient of variation and correlation; (b) criteria based on vulnerability indices, such as GALDIT; (c) similarity criteria for the reconstructed field; and (d) criteria of the value of information for a prediction.

maxz∈{0,1}N Jz  subject to  i=1Nκizi≤B,  Iθ;yS=HyS−HηS,  Iq;yS≤Iθ;yS

The middle equality shows that maximizing the entropy of the measurements is the EIG for the parameters when the noise η has the same entropy everywhere. The last inequality, from data processing, shows that the information about the goal is always smaller, and that the design for the parameters is not, in general, the design for the goal.

Statistical criteria are represented by entropy [N3], the coefficient of variation with correlation [N5] and the risk objective [N4]; index-based criteria by GALDIT [N1]; and similarity criteria by SSIM [N2].

### 1.5 Linear data worth (FOSM)

In applied hydrogeology, data worth is computed with the linear first-order second-moment analysis (FOSM) [B5]. With the observation Jacobian J, the prediction sensitivity y=∂s/∂θ, the prior covariance Σθ and the noise covariance Σϵ, the posterior variance of the prediction and the relative data worth are:

σs2=y⊤Σθy−y⊤ΣθJ⊤JΣθJ⊤+Σϵ−1JΣθy,  ω=1−σs2y⊤Σθy=R2,  EIGq=−12ln1−ω

The key remark is that, in the linear-Gaussian model, the relative FOSM worth ω is exactly the coefficient of determination R2 of Lemma 1, so the goal EIG is an increasing function of it. The “data worth” literature and the “goal-oriented Bayesian OED” literature give the same ranking of designs, as long as the linearization holds.

FOSM data worth for intrusion is given for interface movement [V5], for groups of geophysical observations [V2] and for 3-D design with a genetic algorithm [V3].

### 1.6 Conceptual model uncertainty and Bayesian model averaging

When there are several conceptual models Mk (e.g., facies scenarios or boundary conditions), the prediction becomes a mixture. Bayesian model averaging (BMA) [B6] splits the variance into a within-model part and a between-model part:

Vars∣D=k​pMk∣DVars∣D,Mk+k​pMk∣DEs∣D,Mk−Es∣D2

The second part is not reduced by data within one model; it is reduced only by data that distinguish the models. With a mixture, the posterior of the goal is no longer Gaussian, and the equivalence ω↔EIGq of Section 1.5 no longer holds.

Bayesian data worth under conceptual uncertainty [V4] is the basis of BMA design in a coastal aquifer [V3].

### 1.7 Bayesian OED and the goal-oriented EIG

Lindley defined the value of an experiment as the expected information gain [B7], and Chaloner and Verdinelli made it the standard framework [B8]. When the decision depends only on q, the criterion is the mutual information between q and the data [B13]; for y=Gdθ+η and q=c⊤θ this is Lemma 1 of the project:

EIGqd=Iq;yd=EydKLpq∣yd ‖ pq=−12ln1−c⊤ΓGd⊤GdΓGd⊤+Γn−1GdΓcc⊤Γc

For a vector goal q=Pθ, the EIG is 12lndetPΓP⊤−12lndetPΓpostP⊤, i.e., the goal-oriented D-criterion. The sensitivities GdΓc are computed with one adjoint solve for the goal, regardless of the number of candidate wells.

GOODE gives goal-oriented A- and D-criteria for PDE problems [D4], and the offline–online decomposition makes the goal EIG computable without PDE solves during optimization [D1].

### 1.8 Nonlinear methods: preposterior analysis, ensembles, surrogates and BEL

When the model is nonlinear, the EIG has no closed form. The nested Monte Carlo estimator requires N×M simulations for each design [B11], so the groundwater literature uses three routes: (a) preposterior analysis with reweighting of a prior ensemble; (b) surrogates (polynomial chaos, Gaussian processes, machine learning) of the model or of the utility function itself; and (c) direct learning of the data–goal relation (Bayesian evidential learning, BEL).

EIGN,Md=1Nn=1Nlnpyn∣θn,d−ln1Mm=1Mpyn∣θn,m,d

For a given goal, reweighting the ensemble directly gives the posterior distribution of q without new simulations, which makes route (a) the cheapest for the binary goals of Section 1.3.

Route (a) is preposterior analysis with a bootstrap filter [D5], route (b) the PCE expansion of the utility function [D3], and route (c) design with BEL [D2].

### 1.9 Geophysics as measurement: Archie's law and ERT design

Seawater is about 100 times more conductive than freshwater, so electrical and electromagnetic methods (ERT, TDEM, AEM) are the main tools for the interface. Archie's law [B9] links the formation conductivity to the pore-water conductivity, and the chain rule turns geophysical sensitivity into sensitivity to salinity:

σb=1aϕmσw,  σw≈σw,f+cσw,s−σw,f,  ∂d∂c=Jρ ∂lnρb∂c

Classical ERT design maximizes the model resolution R=J⊤WdJ+λC−1J⊤WdJ in a target region. Geophysical measurements are integrals over large volumes, like heads; they therefore create strong synergy between measurements (document 14) and belong as candidates in the same criterion as wells.

Optimized ERT design selects electrodes and measurements together for a target region [G3]; arrays for the interface are compared on a model built from two decades of monitoring [G4]; CHERT preserves resolution at depth [G2]; AEM gives regional probabilistic maps [G5]; and geophysical observations of the interface proved the most valuable in calibration [V2].

### 1.10 Decisions under uncertainty: chance constraints and the value of information

Intrusion management (pumping, barriers, managed aquifer recharge MAR) is a decision under uncertainty. Howard's value of information [B10] measures how much the expected utility of the best decision increases thanks to the data, while chance-constrained programming [B14] brings risk in as a constraint:

VOId=Eydmaxa∈AEUa,θ∣yd−maxa∈AEUa,θ≥0

Pζjq≤zjcrit≥r ⇔ ζjq+Φ−1r σζj≤zjcrit  for the Gaussian model

The Gaussian form shows the link to design: data that reduce σζj directly free up a margin of Φ−1r Δσζj for pumping. Risk thus enters management, but not yet the design criterion.

Chance constraints at 23 reliability levels [V1], ensemble optimization under the 2050 climate [G1] and adaptive management cycles with monitoring feedback [N5] [N3] [N4] are the ways information enters decisions today.

### 1.11 Link to SWI-Goal-OED

The project selects monitoring wells and MAR sites to maximize the EIG of the salt mass near the toe of the wedge (Sections 1.3 and 1.7), with adjoint sensitivities of the variable-density model (1.1) and with a facies prior (1.6). The coastal-aquifer literature offers four lines: networks with statistical criteria (1.4), linear data worth (1.5–1.6), goal-oriented Bayesian OED, mostly outside intrusion (1.7–1.8), and geophysics and management under risk (1.9–1.10). No paper found applies Bayesian OED with EIG to a variable-density intrusion goal: this is the gap that the project fills.

INNOVATION  Regulations ask about exceeding the salinity threshold, not about the mean. For a linear-Gaussian scalar goal this does not change the design (Theorem 1 of Section 9), but for risk in several zones the design for the mean can lose up to 35% of the information: PRAG-GO measures and corrects this.

# 2. Search methodology

The search covers the period 2010 – October 11, 2026 and four search lines, with 5 papers each: intrusion monitoring networks, data worth and prediction-oriented approaches, Bayesian and goal-oriented OED for groundwater, and geophysics with management under uncertainty. Priority was given to coastal aquifers and to papers from 2018–2026: 13 of the 20 papers concern intrusion or coastal and island aquifers, and 16 are from 2018–2026.

- Sources: publisher websites (Elsevier, Wiley, ASCE, Springer, Copernicus, MDPI, IOP, SIAM), institutional repositories (USGS Pubs, INRS Espace, AURA, AUB ScholarWorks, JCU ResearchOnline, NORA, the SCK CEN portal, NSF PAR, OSTI), RePEc, DOAJ and arXiv.

- Inclusion criteria: (i) a peer-reviewed journal article; (ii) a direct link to measurement design, data worth or decisions under uncertainty for groundwater; (iii) a clear mathematical formulation; (iv) no overlap with the 280 articles of the 14 previous documents.

- Gap search: four independent searches (120 queries in total) found no peer-reviewed paper that applies Bayesian OED with EIG to a variable-density intrusion goal. Group D therefore contains the methodologically closest papers, mostly outside intrusion, and four papers from before 2018 were kept as foundational.

- ★ rating: mathematical depth (★★★★★ = full theory in infinite dimensions; ★★ = an empirical or index-based criterion). The key equations are representative formulations.

# 3. Summary table of the 20 articles

The table is ordered by group (N = networks, V = data worth, D = Bayesian OED, G = geophysics and management) and, within each group, by year, newest first. The last column gives the study site.

| Code | Authors (year) | Title | Journal | Study site |
| --- | --- | --- | --- | --- |
| N1 | Mahmoudpour et al. (2023) | Optimal Design of Groundwater Quality Monitoring Network Using Aquifer Vulnerability Map | Water Resour. Manage. 37(2) | Talesh, Iran |
| N2 | Fan et al. (2020) | Optimum Design of a Seawater Intrusion Monitoring Scheme Based on the Image Quality Assessment Method | Water Resour. Manage. 34(8) | Longkou, China |
| N3 | Roy & Datta (2019) | Adaptive management of coastal aquifers using entropy-set pair analysis-based three-dimensional sequential monitoring network design | J. Hydrol. Eng. 24(3) | synthetic |
| N4 | Lal & Datta (2019) | Application of Monitoring Network Design and Feedback Information for Adaptive Management of Coastal Groundwater Resources | IJERPH 16(22) | Bonriki, Kiribati |
| N5 | Sreekanth & Datta (2014) | Design of an optimal compliance monitoring network and feedback information for adaptive management of saltwater intrusion in coastal aquifers | J. Water Resour. Plann. Manage. 140(10) | illustrative |
| V1 | Coulon et al. (2022) | Pumping Optimization Under Uncertainty in an Island Freshwater Lens Using a Sharp-Interface Seawater Intrusion Model | Water Resour. Res. 58 | Magdalen, Canada |
| V2 | Coulon et al. (2021) | A framework for parameter estimation using sharp-interface seawater intrusion models | J. Hydrol. 600 | Magdalen, Canada |
| V3 | Safi et al. (2019) | Data-Worth Assessment for a Three-Dimensional Optimal Design in Nonlinear Groundwater Systems | Groundwater (2019) | Beirut, Lebanon |
| V4 | Neuman et al. (2012) | Bayesian analysis of data-worth considering model and parameter uncertainties | Adv. Water Resour. 36 | synthetic |
| V5 | Dausman et al. (2010) | Quantifying data worth toward reducing predictive uncertainty | Ground Water 48(5) | synthetic |
| D1 | Wu et al. (2023) | An Offline-Online Decomposition Method for Efficient Linear Bayesian Goal-Oriented Optimal Experimental Design: Application to Optimal Sensor Placement | SIAM J. Sci. Comput. 45(1) | synthetic (PDE) |
| D2 | Thibaut et al. (2021) | A new framework for experimental design using Bayesian Evidential Learning: the case of wellhead protection area | J. Hydrol. 603 | synthetic (WHPA) |
| D3 | Tarakanov & Elsheikh (2020) | Optimal Bayesian experimental design for subsurface flow problems | Comput. Methods Appl. Mech. Eng. 370 | synthetic |
| D4 | Attia et al. (2018) | Goal-oriented optimal design of experiments for large-scale Bayesian linear inverse problems | Inverse Problems 34(9) | synthetic (PDE) |
| D5 | Leube et al. (2012) | Bayesian assessment of the expected data impact on prediction confidence in optimal sampling design | Water Resour. Res. 48(2) | synthetic |
| G1 | Coulon et al. (2024) | An ensemble-based approach for pumping optimization in an island aquifer considering parameter, observation and climate uncertainty | Hydrol. Earth Syst. Sci. 28(1) | Magdalen, Canada |
| G2 | Palacios et al. (2020) | Time-lapse cross-hole electrical resistivity tomography (CHERT) for monitoring seawater intrusion dynamics in a Mediterranean aquifer | Hydrol. Earth Syst. Sci. 24(4) | Argentona, Spain |
| G3 | Uhlemann et al. (2018) | Optimized survey design for electrical resistivity tomography: combined optimization of measurement configuration and electrode placement | Geophys. J. Int. 214(1) | synthetic + laboratory |
| G4 | Costall et al. (2018) | Electrical resistivity imaging and the saline water interface in high-quality coastal aquifers | Surv. Geophys. 39(4) | Western Australia |
| G5 | Delsman et al. (2018) | Large-scale, probabilistic salinity mapping using airborne electromagnetics for groundwater management in Zeeland, the Netherlands | Environ. Res. Lett. 13(8) | Zeeland, Netherlands |

# 4. Monitoring networks for saltwater intrusion

Intrusion monitoring networks are usually designed with statistical or index-based criteria. The James Cook University line links the network to adaptive management: the compliance network [N5], sequential 3-D design with entropy [N3] and the application to the Bonriki lens [N4]. The Jilin group uses image similarity [N2], and the Iranian papers use the GALDIT vulnerability map [N1]. None of them measures information about a goal.

## N1  Optimal Design of Groundwater Quality Monitoring Network Using Aquifer Vulnerability Map

| Authors | Hedi Mahmoudpour, Somaye Janatrostami, Afshin Ashrafzadeh |
| --- | --- |
| Published in | Water Resources Management, vol. 37(2) (2023) |
| DOI / link | https://doi.org/10.1007/s11269-022-03404-w |
| Affiliation | University of Guilan, Iran |
| Topic | salinity monitoring network · GALDIT index · genetic algorithm · network reduction · Talesh coastal aquifer |
| Mathematical depth | ★★☆☆☆ (2/5) |

Summary. The article seeks the network with the smallest number of wells that still gives good coverage of water quality (salinity, measured as electrical conductivity EC) in the Talesh coastal aquifer, on the Caspian coast. First, a vulnerability map is built with a modified GALDIT index, which summarizes six factors of seawater intrusion. Then a genetic algorithm selects subsets of the existing wells with a weighted three-part objective: the correlation between the vulnerability index and EC in the retained wells, the number of wells, and the Nash–Sutcliffe agreement between the full network and the reduced one. The result was sensitive to the weights, and the authors chose the weight that gives the most balanced solution. No physical intrusion model is used: the design is based only on the data and on the index.

Key equation.  (weighted objective of network reduction (representative formulation))

maxz∈{0,1}N Fz=w1 rVz,ECz−w2 nzN+w3 NSEz,  w1+w2+w3=1

NSEz=1−k=1KECkobs−ECkz2k=1KECkobs−ECobs2

where zi=1 if the existing well i is retained, nz=i​zi, r is the Pearson correlation between the GALDIT index V and EC in the retained wells, ECkobs the value of the full network at point k and ECkz the value interpolated from the reduced network. The normalization and the exact interpolation were not confirmed.

Link to the theoretical introduction. Complements Section 1.4 as a representative of index-based criteria. For the project, the GALDIT map can serve as a cheap prior for the risk zones, but the objective does not measure information about any physical goal: it only preserves what the existing network already sees.

INNOVATION  Combination of the GALDIT vulnerability map with genetic network reduction and with a Nash–Sutcliffe check of the reduced network against the full one.

## N2  Optimum Design of a Seawater Intrusion Monitoring Scheme Based on the Image Quality Assessment Method

| Authors | Yue Fan, Wenxi Lu, Tiansheng Miao, Jiuhui Li, Jin Lin |
| --- | --- |
| Published in | Water Resources Management, vol. 34(8), pp. 2485–2502 (2020) |
| DOI / link | https://doi.org/10.1007/s11269-020-02565-w |
| Affiliation | Wenxi Lu group, Jilin University, China |
| Topic | monitoring of the transition zone · SSIM index · KELM surrogate · Monte Carlo · genetic algorithm · Longkou (China) |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The article designs the monitoring scheme in the transition zone between freshwater and seawater in Longkou, with the aim of maximizing the “effective monitored information”. To express this mathematically, the authors borrow the structural similarity index SSIM from image quality assessment: the salinity field reconstructed from the monitored points is compared, as an image, with the full simulated field. The model is variable-density, and parameter uncertainty is handled with Monte Carlo. So that the expensive model is not called thousands of times, it is replaced by a kernel extreme learning machine (KELM), which approximates it well according to the authors. A genetic algorithm selects the sites, and the optimal scheme “reflects well the real state” of the intrusion.

Key equation.  (SSIM index and design criterion (representative formulation))

SSIMu,v=2μuμv+K12σuv+K2μu2+μv2+K1σu2+σv2+K2

S⋆=argmaxS⊂Ω, S=M 1Rr=1RSSIMcr,cSr

where μ, σ2 and σuv are the means, the variances and the covariance of the two fields, K1,K2 small constants, cr the salinity field predicted by KELM for Monte Carlo realization r and cSr the field reconstructed from the values in the set S. The aggregation and the interpolation method were not confirmed.

Link to the theoretical introduction. Complements Section 1.4. SSIM rewards reproducing the spatial shape of the wedge, not reducing the uncertainty about a goal; for the project, it is a useful comparison objective to show how much the design changes when the goal becomes a physical quantity (the salt mass, the toe of the wedge).

INNOVATION  First use of an image-similarity metric (SSIM) as an information criterion for intrusion monitoring, combined with KELM surrogates and uncertainty propagation.

## N3  Adaptive management of coastal aquifers using entropy-set pair analysis-based three-dimensional sequential monitoring network design

| Authors | Dilip Kumar Roy, Bithin Datta |
| --- | --- |
| Published in | Journal of Hydrologic Engineering, vol. 24(3), art. 04018072 (2019) |
| DOI / link | https://doi.org/10.1061/(ASCE)HE.1943-5584.0001765 |
| Affiliation | James Cook University, Australia |
| Topic | 3-D sequential monitoring · Shannon entropy · metamodel ensemble · set pair analysis · adaptive management |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The article designs a three-dimensional monitoring network that allows sequential modification of the pumping policy in a multilayer coastal aquifer, according to incoming salinity data. A simulation–optimization model first gives Pareto-optimal groundwater extraction strategies; the variable-density model is replaced by an ensemble of metamodels, with weights from set pair analysis (SPA) to handle their uncertainty. The monitoring sites are then chosen by maximizing the Shannon entropy of the simulated concentrations at the candidate sites, i.e., by choosing the most informative and least redundant sites, across layers. In the illustrative aquifer, the salinity measured at these sites allows sequential improvement of the management strategy.

Key equation.  (concentration entropy and selection criterion (representative formulation))

HCi=−l=1Lpi,llnpi,l,  S⋆=argmaxS⊂Ω, S=M i∈S​HCi−RS

TCi,Cj=HCi+HCj−HCi,Cj,  cx,t=k=1Kwk ckx,t

where Ci is the simulated concentration at candidate site i, pi,l the relative frequency of class l, RS≥0 the redundancy among the selected sites (for two sites, the transinformation T), ck the prediction of metamodel k and wk the SPA weights. Whether the article maximizes the joint entropy or the sum of the marginal entropies was not confirmed.

Link to the theoretical introduction. Complements Sections 1.4 and 1.10. The concentration entropy is linked to the EIG: for noise-free measurements, the mutual information between measurements and parameters is exactly the entropy of the measurements. The key difference from the project is that there is no goal here: the uncertainty of the measurement is maximized, not the information about the quantity that drives the decision.

INNOVATION  Combination of the SPA-weighted metamodel ensemble with sequential 3-D entropy design, which closes the monitoring–management cycle.

## N4  Application of Monitoring Network Design and Feedback Information for Adaptive Management of Coastal Groundwater Resources

| Authors | Alvin Lal, Bithin Datta |
| --- | --- |
| Published in | International Journal of Environmental Research and Public Health, vol. 16(22), art. 4365 (2019) |
| DOI / link | https://doi.org/10.3390/ijerph16224365 |
| Affiliation | James Cook University, Australia |
| Topic | three-phase adaptive management · pumping non-compliance · high-risk zones · Bonriki freshwater lens (Kiribati) |
| Mathematical depth | ★★☆☆☆ (2/5) |

Summary. The article applies a three-phase adaptive management framework to the Bonriki aquifer, a freshwater lens in South Tarawa (Kiribati). In the first phase, a simulation–optimization model with an ensemble of surrogate models gives the optimal and robust extraction strategy for the production wells and the barrier wells. In the second phase, the monitoring network is designed at regional scale, taking into account that users may not comply with the recommended pumping and that the aquifer parameters are uncertain. A new objective places wells at sites with high risk and high contamination while representing the whole domain. In the third phase, the measured concentrations feed the sequential modification of the pumping strategy.

Key equation.  (well selection over an ensemble of scenarios (representative formulation))

maxz∈{0,1}N αi=1Nzi cicmax+1−αi=1Nzi sici  subject to  i=1Nzi=M

where zi=1 if candidate i is monitored, ci and si are the mean and the standard deviation of the simulated salinity at i over the non-compliance and parameter scenarios, cmax a normalizing concentration (e.g., that of seawater) and α the weight between risk and variability. The exact form of the objective was not confirmed.

Link to the theoretical introduction. Complements Sections 1.4 and 1.10. The risk-oriented objective is the empirical precursor of the threshold goals of Section 1.3: it targets sites where salinity is high and uncertain, but without linking this to information for a decision. The proposal of Section 9 makes this link exact.

INNOVATION  Transfer of compliance-network design from illustrative aquifers to a real island lens with sparse data, with an objective that targets risk.

## N5  Design of an optimal compliance monitoring network and feedback information for adaptive management of saltwater intrusion in coastal aquifers

| Authors | J. Sreekanth, Bithin Datta |
| --- | --- |
| Published in | Journal of Water Resources Planning and Management, vol. 140(10), art. 04014026 (2014) |
| DOI / link | https://doi.org/10.1061/(ASCE)WR.1943-5452.0000406 |
| Affiliation | James Cook University, Australia |
| Topic | compliance monitoring network · coefficient of variation · correlation between sites · adaptive management of intrusion |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. This is the founding paper of the James Cook University line. A simulation–optimization model first gives the optimal pumping strategy for the control of saltwater intrusion. A monitoring network is then designed that compares the measured salinity with the model predictions, to verify whether the strategy is followed (compliance) and whether the model is adequate. The network is optimized with two objectives: maximize the coefficient of variation of salinity at the monitored sites and minimize the correlation between them, i.e., choose sites that are sensitive to deviations and not redundant. The measurements then feed the revision of the strategy, creating the adaptive management cycle that papers [N3] and [N4] extend.

Key equation.  (two-objective design (representative formulation))

maxz∈{0,1}N f1z=i=1Nzi sici,  minz∈{0,1}N f2z=i=1Nj=i+1Nzizjρij,  i=1Nzi=M

where ci and si are the mean and the standard deviation of the simulated salinity at candidate i over an ensemble of scenarios, and ρij the correlation between the simulated concentrations at i and j. Whether the objectives are combined with weights or solved as a Pareto problem was not confirmed.

Link to the theoretical introduction. Complements Section 1.4. The combination “high variability, low correlation” is the heuristic version of D-optimal design for the measurements: for a Gaussian ensemble, the log-determinant of the measurement covariance increases exactly with the variances and decreases with the correlations.

INNOVATION  First formal link between pumping optimization against intrusion and compliance-network design with information feedback.

# 5. Data worth and prediction-oriented approaches

Data worth measures how much a measurement reduces the uncertainty of a prediction, so it is the practical form of goal-oriented design. The group includes the early demonstration for interface movement [V5], the Bayesian formulation with model uncertainty [V4], the 3-D design with BMA in Beirut [V3] and two papers on the Magdalen Islands: calibration with geophysical observations [V2] and pumping with chance constraints [V1].

## V1  Pumping Optimization Under Uncertainty in an Island Freshwater Lens Using a Sharp-Interface Seawater Intrusion Model

| Authors | Cécile Coulon, Jean-Michel Lemieux, Alexandre Pryet, Peter Bayer, Nathaniel L. Young, John Molson |
| --- | --- |
| Published in | Water Resources Research, vol. 58, art. e2021WR031793 (2022) |
| DOI / link | https://doi.org/10.1029/2021WR031793 |
| Affiliation | Université Laval (Québec, Canada) and collaborators; the affiliation of each author was not confirmed |
| Topic | pumping optimization under uncertainty · chance constraints · PESTPP-OPT · SWI2 sharp-interface model · island lens |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The article addresses a management decision: how much water can be pumped from the freshwater lens of the Magdalen Islands without salinizing the wells. With the MODFLOW-SWI2 sharp-interface model of [V2], the authors maximize the total pumping of the well field while limiting the risk of salinization. The optimization is done under parameter and observation uncertainty with PESTPP-OPT, which turns the uncertainty of the decision-critical outputs (the position of the interface below the wells) into chance constraints. The optimization is repeated for 23 reliability levels and gives the trade-off curve between pumping and salinization risk. In this way the prediction uncertainty, and therefore also the worth of the data that reduce it, enters the decision directly.

Key equation.  (pumping with chance constraints and their FOSM form (representative formulation))

maxq i=1nwqi  subject to  Pζjq≤zjcrit≥r, qimin≤qi≤qimax

ζjq+Φ−1r σζj≤zjcrit,  σζj2=yj⊤Σθ yj

where qi is the discharge of well i, ζj the simulated interface elevation at constraint location j, zjcrit the critical elevation (e.g., the well bottom minus a margin), r the required reliability, Φ−1 the standard normal quantile and σζj2 the FOSM posterior variance with yj=∂ζj/∂θ. The FOSM form is inferred from the use of PESTPP-OPT and was not seen in the text.

Link to the theoretical introduction. Complements Sections 1.2 and 1.10. The constraint Pζj≤zjcrit≥r is the threshold goal of Section 1.3, but in management, not in design. Every reduction Δσζj from new data lowers the margin by Φ−1r Δσζj meters and allows more pumping; PRAG-GO (Section 9) turns this probability into a design goal.

INNOVATION  Pumping optimization with chance constraints at 23 reliability levels in a real island lens, which gives the full trade-off curve between discharge and salinization risk.

## V2  A framework for parameter estimation using sharp-interface seawater intrusion models

| Authors | Cécile Coulon, Alexandre Pryet, Jean-Michel Lemieux, Ble Jean Fidele Yrro, Abderrezak Bouchedda, Erwan Gloaguen, Jean-Christophe Comte, J. Christian Dupuis, Olivier Banton |
| --- | --- |
| Published in | Journal of Hydrology, vol. 600, art. 126509 (2021) |
| DOI / link | https://doi.org/10.1016/j.jhydrol.2021.126509 |
| Affiliation | INRS (Québec, Canada) and University of Aberdeen (Scotland), according to institutional repositories; the affiliation of each author was not confirmed |
| Topic | calibration of sharp-interface models · FOSM · worth of observation groups · interface from TDEM and ERT |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. There is little guidance for the calibration of the intrusion models that support decisions: which observations to use, how to process them and how to weight them. The authors build a regional sharp-interface model in MODFLOW-SWI2 for the Magdalen Islands, fast enough to allow many parameters, with pilot points for the conductivity. The data are heads from shallow, deep and pumping wells, and the interface elevation (the 50% seawater salinity contour) from deep wells and from TDEM and ERT geophysical measurements. The predictions for the decision are the freshwater volume and the interface elevation near the municipal wells. FOSM analysis and the worth of observation groups showed that the interface observations, especially the geophysical ones, were the most useful, even though they are much more uncertain than the well data.

Key equation.  (FOSM posterior variance and the worth of an observation group (representative formulation))

Σθ=Σθ−ΣθJ⊤JΣθJ⊤+Σϵ−1JΣθ,  σs2=y⊤Σθ y

Wgremove=σs2O−g−σs2O,  Wgadd=σs2O0−σs2O0∪g

where Σθ is the prior parameter covariance, J the Jacobian of the simulated observations, Σϵ the noise covariance, y=∂s/∂θ the sensitivity of prediction s, O the full set of observations, O−g the set without group g and O0 a base set. The exact computational tool (pyEMU or PEST) was not confirmed.

Link to the theoretical introduction. Complements Sections 1.2, 1.5 and 1.9. Data worth is judged by the management predictions and not by the fit to the data, so it is goal-oriented. For the project, the finding that geophysical interface measurements are worth more than deep wells supports including ERT as a “candidate well” with its own cost and noise.

INNOVATION  A reproducible workflow that brings geophysical interface data into fast sharp-interface models, with data worth judged by the management predictions.

## V3  Data-Worth Assessment for a Three-Dimensional Optimal Design in Nonlinear Groundwater Systems

| Authors | Amir Safi, Troels Norvin Vilhelmsen, Ibrahim Alameddine, Majdi Abou Najm, Mutasem El-Fadel |
| --- | --- |
| Published in | Groundwater (NGWA), 2019; published online in 2018 |
| DOI / link | scholarworks.aub.edu.lb · AUB repository record |
| Affiliation | American University of Beirut, Department of Civil and Environmental Engineering (Safi, Alameddine, El-Fadel); the other affiliations were not confirmed |
| Topic | linear data worth · Bayesian model averaging · genetic algorithm · 3-D well design · head and salinity |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. Data collection is expensive, so the article asks how many new observation wells to drill in a heterogeneous coastal aquifer with saltwater intrusion and where to place them, in plan and in depth. The authors bring Bayesian model averaging (BMA) and a genetic algorithm into a linear data-worth analysis, and they run a full three-dimensional search over the candidate sites. The aim is to choose the sites for head and salinity measurements that most reduce the uncertainty of the prediction of several intrusion variables. BMA brings conceptual model uncertainty into the data worth. The optimal sites changed with the constraints, and the optimal design depended mainly on the ratio between the initial cost of the monitoring program and the cost of installing the first well.

Key equation.  (BMA prediction variance and the 3-D design problem (representative formulation))

Vars∣D=k=1KpMk∣DVars∣D,Mk+k=1KpMk∣DEs∣D,Mk−Es∣D2

x⋆=argminx∈Xn j=1nsk​pMk∣Dyjk⊤Σθ,kx yjkk​pMk∣Dyjk⊤Σθ,k yjk  subject to  c0+i=1nci≤B

where s is an intrusion prediction, Mk the conceptual models with weights pMk∣D, x the 3-D locations of the n new wells, Σθ,kx the FOSM posterior covariance of model k after the new measurements, yjk the sensitivity of prediction j, c0 the initial cost, ci the cost of well i and B the budget. The exact objective, the journal, the volume and the DOI were not confirmed.

Link to the theoretical introduction. Complements Sections 1.5 and 1.6 and is the only paper found that selects wells for intrusion predictions in a real coastal aquifer. For the project, the dependence on depth and on the initial cost shows that the design must also choose the screen depth, not only the location in plan.

INNOVATION  3-D design, with depth, by data worth for intrusion, which combines averaging over several conceptual models with an explicit cost trade-off.

## V4  Bayesian analysis of data-worth considering model and parameter uncertainties

| Authors | Shlomo P. Neuman, Liang Xue, Ming Ye, Dan Lu |
| --- | --- |
| Published in | Advances in Water Resources, vol. 36, pp. 75–85 (2012) |
| DOI / link | https://doi.org/10.1016/j.advwatres.2011.02.007 |
| Affiliation | University of Arizona (Neuman) and Florida State University (Ye), according to the records; the affiliations at the time of publication were not confirmed |
| Topic | Bayesian data worth · Bayesian model averaging (BMA, MLBMA) · KIC criterion · conceptual uncertainty |
| Mathematical depth | ★★★★☆ (4/5) |

Summary. Traditional value-of-information analyses assume a single model with given parameters, which can bias the predictions and underestimate the uncertainty. The article places data-worth analysis within Bayesian model averaging: it gives a theoretical description of the full BMA and of the maximum likelihood version (MLBMA), in which the models are weighted with an information criterion computed at the maximum likelihood estimates. The worth of future data is measured as the expected reduction of the uncertainty of the BMA prediction, averaged over the possible values of the data before they are collected. MLBMA is applied to a synthetic example, with and without linearization.

Key equation.  (MLBMA weights and the expected data worth (representative formulation))

pMk∣D=exp−12ΔKICkpMkl=1Kexp−12ΔKIClpMl,  ΔKICk=KICk−minlKICl

DWξ=VarΔ∣D−ED⋆∣D,ξVarΔ∣D,D⋆

where Mk are the alternative models, KICk the Kashyap information criterion of model k at the maximum likelihood estimate, Δ the predicted quantity, D⋆ the future data of design ξ, averaged over their BMA predictive distribution, and Var the BMA variance. Whether the worth is defined with the variance or with the trace or the determinant was not confirmed.

Link to the theoretical introduction. Complements Section 1.6 and is the theoretical basis of the BMA design of [V3]. For the project, the facies prior as a mixture of scenarios is exactly a model average: the posterior of the goal is then no longer Gaussian, and the equivalence of Section 1.5 between data worth and the EIG no longer holds.

INNOVATION  The first general Bayesian formulation of data worth for predictions that propagates conceptual model uncertainty and not only parameter uncertainty.

## V5  Quantifying data worth toward reducing predictive uncertainty

| Authors | Alyssa M. Dausman, John Doherty, Christian D. Langevin, Michael C. Sukop |
| --- | --- |
| Published in | Ground Water, vol. 48(5), pp. 729–740 (2010) |
| DOI / link | https://doi.org/10.1111/j.1745-6584.2010.00679.x |
| Affiliation | U.S. Geological Survey and Florida International University, according to the records; the affiliation of each author was not confirmed |
| Topic | data worth for prediction · variable-density model for salt and heat · interface movement · temperature and concentration measurements |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The article gives a method for optimizing the collection of environmental data, based on the principle that the worth of data increases in proportion to their ability to reduce the uncertainty of key model predictions. It is demonstrated on a hypothetical, nonlinear, variable-density model with coupled transport of salt and heat. The prediction of interest is the movement of the saltwater interface when recharge decreases. Concentration data, especially near the interface where movement is expected, are worth more than temperature data, but carefully placed pairs of temperature measurements also improve the prediction. The ranking of worth remained stable across several realizations of the properties, which suggests that the linear method remains useful for a strongly nonlinear problem.

Key equation.  (relative worth of a data set and stability of the ranking (representative formulation))

ωk=1−y⊤ΣθDkyy⊤Σθ y

ωkr=1−yr⊤ΣθDk;θryryr⊤Σθ yr,  r=1,…,Nr

where Dk is the candidate data set (e.g., a concentration or temperature measurement at one well and depth), ΣθDk the FOSM posterior covariance with only the rows of Dk, y=∂s/∂θ the sensitivity of the interface movement and θr realization r of the properties, at which the Jacobians are recomputed. The exact metric and the PEST tool were not confirmed; the DOI comes from the USGS repository.

Link to the theoretical introduction. Complements Sections 1.3 and 1.5. In the linear-Gaussian model, ωk is exactly the R2 of Lemma 1, so EIGq=−12ln1−ωk: the data worth of Dausman and the goal EIG of the project give the same ranking. The pairs of temperature measurements are an early example of the synergy of document 14.

INNOVATION  An early demonstration of prediction-oriented data worth for saltwater intrusion, with a check of the stability of the linear ranking across several realizations.

# 6. Bayesian and goal-oriented OED for groundwater

Goal-oriented Bayesian OED has been developed mainly for contaminant transport and groundwater flow, not yet for intrusion. The group gives the GOODE criteria [D4] and the offline–online decomposition of the goal EIG [D1], nonlinear preposterior analysis [D5], the PCE expansion of the utility function [D3] and design with Bayesian evidential learning [D2].

## D1  An Offline-Online Decomposition Method for Efficient Linear Bayesian Goal-Oriented Optimal Experimental Design: Application to Optimal Sensor Placement

| Authors | Keyi Wu, Peng Chen, Omar Ghattas |
| --- | --- |
| Published in | SIAM Journal on Scientific Computing, vol. 45(1), pp. B57–B77 (2023) |
| DOI / link | arXiv:2102.06627 · preprint of the article |
| Affiliation | Oden Institute for Computational Engineering and Sciences and Department of Mathematics, The University of Texas at Austin (according to the preprint) |
| Topic | goal-oriented Bayesian OED · EIG of a prediction quantity · offline–online decomposition · swapping greedy · sensor placement |
| Mathematical depth | ★★★★★ (5/5) |

Summary. The article addresses optimal sensor placement for large-scale linear Bayesian inverse problems with PDEs, when the goal is the EIG for a predicted quantity (QoI) and not for the parameter field itself. The authors derive a new formula for this EIG and exploit the low-rank structure of two operators. The computation is split into two phases: in the offline phase, the expensive quantities that do not depend on the design and that require PDE solves are computed once; in the online phase, the design optimization requires no new PDE solve. The sensors are selected with a new swapping greedy algorithm. The method is shown to be efficient and scalable on an inverse problem of contaminant transport with an infinite-dimensional parameter.

Key equation.  (the goal EIG and its posterior covariance (representative formulation))

Ψξ=Ey∣ξDKLπρ∣y,ξ ‖ πρ=12lndetΣpr−12lndetΣpostξ

Σpostξ=Σpr−PCprF⊤Wξ⊤WξFCprF⊤+ΓWξ⊤−1WξFCprP⊤,  Σpr=PCprP⊤

where F is the linear parameter-to-observation operator for all candidate sensors, Wξ the sensor-selection matrix of design ξ, Cpr and Γ the prior and noise covariances, P the linear goal operator and ρ=Pm the goal. The blocks FCprP⊤ and the low-rank approximation of FCprF⊤ are computed offline. The new formula of the article and the DOI were not seen.

Link to the theoretical introduction. Complements Section 1.7 and is the PDE equivalent of Lemma 1 of the project. For the project, the offline phase is exactly the construction of the information operator with adjoints, and the swapping greedy matches the “best + swap” step of SINERGO (sinergji: synergy; document 14). For intrusion, which is nonlinear, this formulation requires linearization around the MAP.

INNOVATION  Goal-oriented EIG for large-scale PDE problems, with an online phase free of PDE solves and with swapping greedy.

## D2  A new framework for experimental design using Bayesian Evidential Learning: the case of wellhead protection area

| Authors | Robin Thibaut, Eric Laloy, Thomas Hermans |
| --- | --- |
| Published in | Journal of Hydrology, vol. 603, art. 126903 (2021) |
| DOI / link | https://doi.org/10.1016/j.jhydrol.2021.126903 |
| Affiliation | SCK CEN, Belgian Nuclear Research Centre (according to its portal); the affiliations of T. Hermans and R. Thibaut were not confirmed |
| Topic | Bayesian evidential learning (BEL) · prediction-oriented design · wellhead protection area (WHPA) · tracer tests |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The goal is to predict the protection area of a pumping well (WHPA) from a few tracer tests, and to decide where to place the tracer injection wells. Bayesian evidential learning (BEL) learns a direct statistical relation between the data (the tracer curves) and the target (the WHPA), without inversion for the conductivity. The relation is learned from only 400 training models drawn from the prior of hydraulic conductivity. Once trained, BEL gives the posterior distribution of the WHPA almost instantly, so many candidate designs can be evaluated. The posterior uncertainty of the target is measured with the modified Hausdorff distance and with SSIM. More injection wells reduce the uncertainty, and the method ranks the wells by information, with a ranking validated by cross-validation.

Key equation.  (BEL posterior in the reduced space and the design utility (representative formulation))

hc∣dobsc∼Nμh+ChdCdd−1dobsc−μd, Chh−ChdCdd−1Cdh

Uξ=1Nti=1Nt1Nsj=1NsMHDhijξ,hi,  ξ⋆=argminξ∈ΞUξ

where dc and hc are the low-dimensional projections (e.g., PCA and canonical correlation) of the data and of the WHPA, the moments are estimated from the 400 prior pairs, ξ is the subset of injection wells, hi the true WHPA of test model i, hijξ the posterior samples and MHD the modified Hausdorff distance. The exact use of the MHD and of SSIM was not confirmed.

Link to the theoretical introduction. Complements Sections 1.3 and 1.8. The linear-Gaussian step of BEL is exactly Lemma 1 in the reduced space, so the goal EIG can be computed directly from Chh and ChdCdd−1Cdh. For the project, BEL gives an adjoint-free way to evaluate designs over a SEAWAT ensemble, with the toe of the wedge or threshold exceedance in a well as the goal.

INNOVATION  Prediction-oriented experimental design with learned data–target relations, which evaluates every candidate design without inversion.

## D3  Optimal Bayesian experimental design for subsurface flow problems

| Authors | Alexander Tarakanov, Ahmed H. Elsheikh |
| --- | --- |
| Published in | Computer Methods in Applied Mechanics and Engineering, vol. 370, art. 113208 (2020) |
| DOI / link | https://doi.org/10.1016/j.cma.2020.113208 |
| Affiliation | A. Tarakanov: University of Manchester; A. H. Elsheikh: Heriot-Watt University (Edinburgh) |
| Topic | Bayesian OED · polynomial chaos expansion (PCE) of the utility function · subsurface flow · EIG without the inner loop |
| Mathematical depth | ★★★★☆ (4/5) |

Summary. In subsurface flow, the utility function of Bayesian design contains several high-dimensional integrals, and each integration point requires the solution of a system of coupled PDEs, which makes the standard estimators too expensive. The authors build a polynomial chaos expansion (PCE) of the utility function itself. The key idea is the orthogonality of the PCE basis: the expensive integration over the space of possible observations is replaced by the direct construction of a PCE approximation of the EIG. According to the authors, this gives a response surface of reasonable quality over the whole design space, at a cost comparable to a few single nested Monte Carlo estimates. The method is tested on several numerical cases of different complexity.

Key equation.  (the NMC estimator of the EIG and the PCE shortcut (representative formulation))

Uξ=1Ni=1Nlnpyi∣θi,ξ−ln1Mj=1Mpyi∣θj,ξ

uξ,yζ≈α∈A​cαξΨαζ ⇒ Uξ=Eζu≈c0ξ

where ξ is the design, θ the uncertain parameters, θi,yi the outer samples and θj the inner samples of the nested Monte Carlo, Ψα the orthonormal polynomials in the standard variables ζ that generate y, and cα the PCE coefficients; since EΨα=0 for α≠0, the expectation is the first coefficient. The exact construction of the article and whether the EIG targets the parameters or a goal were not confirmed.

Link to the theoretical introduction. Complements Section 1.8. For the project, the PCE of the EIG over the design space is a cheap way to evaluate designs in the nonlinear SEAWAT model, where Lemma 1 holds only locally; replacing the parameters with the goal (the toe of the wedge) in the utility function makes it goal-oriented.

INNOVATION  A PCE approximation of the utility function itself that avoids the inner integration over the data, for expensive subsurface simulators.

## D4  Goal-oriented optimal design of experiments for large-scale Bayesian linear inverse problems

| Authors | Ahmed Attia, Alen Alexanderian, Arvind K. Saibaba |
| --- | --- |
| Published in | Inverse Problems, vol. 34(9), art. 095009 (2018) |
| DOI / link | https://doi.org/10.1088/1361-6420/aad210 |
| Affiliation | A. Alexanderian and A. K. Saibaba: Department of Mathematics, North Carolina State University; A. Attia: Argonne National Laboratory (to the best of our knowledge) |
| Topic | GOODE · goal-oriented A- and D-optimality · linear Bayesian inverse problems · sparsity penalty · contaminant source identification |
| Mathematical depth | ★★★★★ (5/5) |

Summary. The article introduces the GOODE framework (goal-oriented optimal design of experiments) for large-scale linear Bayesian inverse problems with PDEs. Instead of minimizing the uncertainty of the inferred parameter, GOODE minimizes the uncertainty of a prediction quantity that depends on the parameter. Two criteria are defined, A-GOODE and D-GOODE, the goal analogs of the classical Bayesian A-optimal (trace) and D-optimal (log-determinant) criteria, applied to the posterior covariance of the prediction. The designs are relaxed to continuous sensor weights and optimized with gradients, with an ℓ1 penalty that promotes sparse placements. The application is sensor placement for contaminant source identification in a diffusion–transport model.

Key equation.  (weighted posterior covariance and the GOODE criteria (representative formulation))

Γpostw=F⋆Γnoise−1/2diagwΓnoise−1/2F+Γpr−1−1

ΨAw=trPΓpostwP⋆,  ΨDw=lndetPΓpostwP⋆,  w⋆=argminw∈0,1nsΨw+αw1

where F is the linear parameter-to-observation operator and F⋆ its adjoint, Γpr and Γnoise the prior and noise covariances, wi the weight of candidate sensor i, P the linear goal operator with adjoint P⋆ and α>0 the sparsity penalty. The DOI is given to the best of our knowledge.

Link to the theoretical introduction. Complements Section 1.7. D-GOODE for a scalar goal is exactly the goal EIG of Lemma 1, because 12lnσpr2/σpost2=−12ln1−R2. For the project, the weighted relaxation and the ℓ1 penalty are the continuous alternative to the discrete greedy algorithm over the 32 wells of Fushë-Kuqe.

INNOVATION  A systematic formulation of goal-oriented A- and D-optimality for infinite-dimensional Bayesian inverse problems, with gradient optimization and sparsity.

## D5  Bayesian assessment of the expected data impact on prediction confidence in optimal sampling design

| Authors | P. C. Leube, A. Geiges, W. Nowak |
| --- | --- |
| Published in | Water Resources Research, vol. 48(2), art. W02501 (2012) |
| DOI / link | https://doi.org/10.1029/2010WR010137 |
| Affiliation | University of Stuttgart, Institute for Modelling Hydraulic and Environmental Systems and SimTech (W. Nowak); the other affiliations were not confirmed |
| Topic | sampling design for prediction · preposterior analysis · bootstrap filter · no linearization · geostatistical groundwater |
| Mathematical depth | ★★★★☆ (4/5) |

Summary. The article addresses optimal sampling design for groundwater predictions, with the objective of maximizing the expected prediction confidence, i.e., reducing the uncertainty of a model prediction and not of the parameters. A Bayesian preposterior analysis is used: before the data are collected, it estimates how much a candidate design would reduce the prediction uncertainty, by averaging over the possible values of the data. The method is fully nonlinear, without linearization and without a Gaussian assumption: the posterior statistics for each hypothetical data set are obtained with a bootstrap filter, i.e., by likelihood reweighting of a prior Monte Carlo ensemble. It is the methodological precursor of most later “task-driven” designs in hydrogeology.

Key equation.  (preposterior utility and the bootstrap-filter estimator (representative formulation))

Φd=EydVarz∣yd=∫Varz∣ydpyddyd

Φd=1Nyj=1Nyi=1Nωijzi−μj2,  ωij=Lyj∣θik=1NLyj∣θk,  μj=i=1Nωijzi

where d is the sampling design, yd the future data, z the prediction, θi the prior realizations (e.g., conductivity fields) with predictions zi, yj the hypothetical data sets, L the measurement likelihood and ωij the normalized weights. The variance form is representative: the utility can also be an exceedance probability or an entropy. The name “PreDIA” was not seen in the sources.

Link to the theoretical introduction. Complements Sections 1.8 and 1.3. The same estimator with the weights ωij directly gives the EIG of a binary goal, e.g., q=Iz>τ, by replacing the variance with the binary entropy of i​ωijIzi>τ: this is the nonlinear route of the PRAG-GO proposal.

INNOVATION  A nonlinear, Monte Carlo measure of data worth for prediction that avoids the linear FOSM assumptions by bootstrap reweighting of a prior ensemble.

# 7. Geophysics and intrusion management with monitoring

Electrical geophysics is the main tool for the interface, and management under uncertainty is where information becomes a decision. The group gives optimized ERT design [G3], the comparison of arrays for the interface [G4], CHERT monitoring in the Mediterranean [G2], probabilistic AEM maps in the Netherlands [G5] and ensemble optimization under the 2050 climate [G1].

## G1  An ensemble-based approach for pumping optimization in an island aquifer considering parameter, observation and climate uncertainty

| Authors | Cécile Coulon, Jeremy T. White, Alexandre Pryet, Laura Gatel, Jean-Michel Lemieux |
| --- | --- |
| Published in | Hydrology and Earth System Sciences, vol. 28(1), pp. 303–319 (2024) |
| DOI / link | https://doi.org/10.5194/hess-28-303-2024 |
| Affiliation | Université Laval group (Québec, Canada) and collaborators; the affiliation of each author was not confirmed |
| Topic | ensemble-based optimization under uncertainty · PESTPP-IES · PESTPP-MOU · sea-level rise by 2050 · risk of well salinization |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The management problem is to find sustainable discharges in the island aquifer of Grande Entrée (Magdalen Islands) that avoid the salinization of the wells. A MODFLOW-SWI2 sharp-interface model is conditioned on field observations with the iterative ensemble smoother PESTPP-IES, which gives a posterior parameter ensemble. Sea-level and recharge ensembles for the year 2050 are then combined with this ensemble. The multi-objective optimization PESTPP-MOU uses the ensemble to make the trade-off between higher pumping and lower salinization risk, and the results are compared with the current demand and with the 2050 demand.

Key equation.  (ensemble conditioning and salinization risk (representative formulation))

mja=mjf+CmdCdd+Cϵ−1dobs+ϵj−gmjf,  j=1,…,Ne

maxq k=1Kqk, −Psq,  Psq≈1Nej=1NeImaxkζkq,mja,cj−zkcrit>0

where mjf and mja are the prior and updated parameters of realization j, g the SWI2 model, Cmd and Cdd the ensemble covariances, ϵj the added noise, qk the discharge of well k, ζk the interface elevation below well k, cj the climate realization and I the indicator function. The exact way in which PESTPP-MOU encodes the risk was not confirmed.

Link to the theoretical introduction. Complements Section 1.10. The salinization probability Ps is a threshold goal over several zones (the wells), exactly the case of Theorem 2 of Section 9. For the project, the posterior ensemble conditioned on monitoring naturally raises the question that the article does not ask: which observations most reduce the uncertainty of Ps?

INNOVATION  An end-to-end ensemble workflow (conditioning, 2050 climate, multi-objective optimization under risk) applied to a real island aquifer.

## G2  Time-lapse cross-hole electrical resistivity tomography (CHERT) for monitoring seawater intrusion dynamics in a Mediterranean aquifer

| Authors | Andrea Palacios, Juan José Ledo, Niklas Linde, Linda Luquot, Fabian Bellmunt and 8 co-authors, including Jesús Carrera |
| --- | --- |
| Published in | Hydrology and Earth System Sciences, vol. 24(4), pp. 2121–2139 (2020) |
| DOI / link | https://doi.org/10.5194/hess-24-2121-2020 |
| Affiliation | According to repositories: Universitat de Barcelona and Université de Montpellier; the affiliation of each author was not confirmed |
| Topic | cross-hole electrical resistivity tomography (CHERT) · time-lapse monitoring · seasonal dynamics and storms · Mediterranean alluvial aquifer |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The resolution of surface ERT, the most widely used tool for intrusion, decreases with depth. The authors argue that placing electrodes at depth, in cross-hole ERT, maintains the resolution at the depths that matter. They installed permanent electrodes around wells and carried out time-lapse ERT along a cross-section perpendicular to the coast at the Argentona experimental site, about 40 km northeast of Barcelona. Monitoring lasted about two years and captured seasonal changes, progressive salinization from the long drought and short fluctuations from sea storms and from the floods of a stream. The images were checked against the conductivity of water samples and against induction logs in the wells.

Key equation.  (time-lapse inversion with a reference model and the petrophysical relation (representative formulation))

mt=argminm Wddt−fm22+αWmm−m022

σb=σwF,  F=a ϕ−mc

where m are the cell log-resistivities, dt the measured resistances at time t, f the forward operator for the cross-hole geometry, Wd and Wm the data weights and the smoothing operator, m0 the baseline model, σb and σw the conductivity of the formation and of the pore water, F the formation factor, ϕ the porosity and mc the cementation exponent. The inversion code and the time-lapse scheme were not confirmed.

Link to the theoretical introduction. Complements Sections 1.1 and 1.9. The choice of electrodes at depth versus surface electrodes is a design decision, although not formally optimized; the fluctuations from storms show the temporal frequency that monitoring requires. For the project, CHERT can enter as a candidate with a high initial cost and with high information near the toe of the wedge.

INNOVATION  Two-year monitoring of intrusion dynamics with cross-hole ERT, which captures time scales from storms to drought and preserves resolution at depth.

## G3  Optimized survey design for electrical resistivity tomography: combined optimization of measurement configuration and electrode placement

| Authors | Sebastian Uhlemann, Paul B. Wilkinson, Hansruedi Maurer, Florian M. Wagner, Timothy C. Johnson, Jonathan E. Chambers |
| --- | --- |
| Published in | Geophysical Journal International, vol. 214(1), pp. 108–121 (2018) |
| DOI / link | https://doi.org/10.1093/gji/ggy128 |
| Affiliation | British Geological Survey, ETH Zürich, University of Bonn and Pacific Northwest National Laboratory (according to the abstract; the affiliation of each author was not confirmed) |
| Topic | optimized ERT design · resolution matrix · electrode placement · target region · fewer electrodes |
| Mathematical depth | ★★★★☆ (4/5) |

Summary. Classical methods of optimized ERT design choose which four-electrode measurements to make on a fixed electrode layout. This article also optimizes the electrode locations, with the aim of maximizing the image resolution within a target region and limiting the number of electrodes. The criterion is based on the model resolution matrix. In a synthetic 3-D test with an embankment, the optimized design gave better resolution in the target than a comparable standard design, with 22% fewer electrodes. Resistivity contrasts had little influence on the performance of the optimization, so a design computed for a homogeneous model remains useful. A laboratory experiment confirmed the synthetic results.

Key equation.  (the resolution matrix and the criterion with a target and an electrode budget (representative formulation))

RS=JS⊤WdJS+λC−1JS⊤WdJS

S⋆=argmaxS⊆CE, E⊆E, E≤Ne 1Tj∈T​RjjSRjjCE

where S is the set of four-electrode measurements, JS the Jacobian of the apparent log-resistivities with respect to the cell log-resistivities, Wd the data weights, λC the regularization, Rjj the resolution of cell j (1 full, 0 unresolved), E the candidate electrode locations, E the installed electrodes, CE the possible configurations and T the cells of the target region. The exact form of the objective was not confirmed.

Link to the theoretical introduction. Complements Section 1.9. The target region T is a geophysical “goal”: the criterion focuses resolution where it is needed, exactly like goal-oriented design. For intrusion, T would be the expected interface band or the mixing zone; the project can replace resolution with the EIG of the hydrological goal through the petrophysical chain.

INNOVATION  Joint optimization of electrode locations and measurement configurations, focused on a target region, with 22% fewer electrodes.

## G4  Electrical resistivity imaging and the saline water interface in high-quality coastal aquifers

| Authors | A. Costall, B. Harris, J. P. Pigois |
| --- | --- |
| Published in | Surveys in Geophysics, vol. 39(4), pp. 753–816 (2018) |
| DOI / link | https://doi.org/10.1007/s10712-018-9468-0 |
| Affiliation | A. Costall: Curtin University (Perth, Western Australia); the other affiliations were not confirmed |
| Topic | resistivity imaging · saltwater interface · highly permeable aquifers · electrode arrays · geoelectric model from monitoring |
| Mathematical depth | ★★★☆☆ (3/5) |

Summary. The article examines when and how electrical resistivity imaging (ERI) can reliably image the saltwater interface in highly permeable coastal freshwater aquifers. The authors use two decades of intrusion monitoring data to build a realistic geoelectric model of the aquifer. With this model they run field and synthetic experiments that test how the acquisition parameters (electrode arrays and configurations) and the inversion parameters affect the recovered conductivity distribution. The main conclusion is that combinations of arrays that include dipole–dipole and other quadrupoles can be deployed to resolve the main features of coastal hydrogeology, but only when the formation conductivity distribution is interpreted with confidence.

Key equation.  (the apparent resistivity of a quadrupole and the cumulative sensitivity (representative formulation))

ρa=K ΔVI,  K=2π1rAM−1rBM−1rAN+1rBN−1

Sj=i=1Ndwi∂lnρa,i∂lnρj

where A,B are the current electrodes and M,N the potential electrodes, rXY the distance between electrodes, I the current, ΔV the potential difference, K the geometric factor for the half-space, Nd the number of quadrupoles, wi the weight of the datum and ρj the resistivity of cell j. The design metric used by the article was not confirmed.

Link to the theoretical introduction. Complements Section 1.9. Choosing the array is choosing the set of quadrupoles, i.e., a design problem; the cumulative sensitivity Sj in the interface band shows whether the design “sees” the goal. For the project: without knowledge of the formation conductivity, ERT can give false information about the toe.

INNOVATION  A systematic comparison of ERT acquisition designs for the saltwater interface, on a geoelectric model built from two decades of monitoring.

## G5  Large-scale, probabilistic salinity mapping using airborne electromagnetics for groundwater management in Zeeland, the Netherlands

| Authors | Joost R. Delsman and 15 co-authors |
| --- | --- |
| Published in | Environmental Research Letters, vol. 13(8), art. 084011 (2018) |
| DOI / link | https://doi.org/10.1088/1748-9326/aad19e |
| Affiliation | Recorded on the Utrecht University portal; the affiliations of the authors were not confirmed |
| Topic | airborne electromagnetics (AEM) · probabilistic chloride map · pedotransfer functions · Monte Carlo · coastal delta |
| Mathematical depth | ★★☆☆☆ (2/5) |

Summary. Freshwater management in a coastal delta requires maps of where groundwater is fresh, brackish or saline, and well data are too sparse to provide them. The authors used a regional airborne electromagnetic (AEM) survey over about 1800 km² to build a 3-D model of the probability of chloride concentration. The formation resistivities from the AEM inversion were converted to salinity with pedotransfer functions and lithological information, and Monte Carlo propagated the uncertainty of each step to the final map. About 75% of the area has no freshwater within 15 m of the surface; the freshest water is concentrated under dunes and sandy ridges. The uncertainty was largest in the fresh–saline transition zones.

Key equation.  (petrophysical conversion and exceedance probability (representative formulation))

σwx=Fℓxσbx−σsℓx,  Clx=hσwx

PClx>c⋆≈1Nn=1NIClnx>c⋆

where σb is the formation conductivity from AEM, ℓx the lithological class, Fℓ the formation factor by lithology, σsℓ the correction for surface conductivity, h the empirical conductivity–chloride relation, Cln the chloride field in realization n and c⋆ the threshold. The exact petrophysical relations were not confirmed.

Link to the theoretical introduction. Complements Sections 1.3 and 1.9. The product PCl>c⋆ is exactly the distribution of the binary goal of PRAG-GO; its binary entropy at each location shows where additional data are worth most. For the project, the concentration of uncertainty in the transition zones supports placing wells near the toe of the wedge.

INNOVATION  A regional probabilistic product, the probability of exceeding a chloride threshold, instead of a deterministic resistivity map, built specifically for groundwater management.

# 8. Synthesis: what the 20 articles show together

The 20 papers cover all sections of Part I. The table gives the role, the criterion and the main message of each.

| Code | Role | Criterion or tool | Main message |
| --- | --- | --- | --- |
| N1 | Index-based network | GALDIT + GA + NSE | The smallest network that preserves coverage. |
| N2 | Image-based network | SSIM + KELM | Reproduction of the wedge shape. |
| N3 | Sequential network | Shannon entropy | The monitoring–management cycle in 3-D. |
| N4 | Island lens | Risk objective | Wells in high-risk zones. |
| N5 | Compliance | CV and correlation | Sensitive, non-redundant sites. |
| V1 | Pumping under risk | Chance constraints | Discharge–risk curve at 23 levels. |
| V2 | SWI2 calibration | FOSM + group worth | Geophysics is worth more than deep wells. |
| V3 | 3-D design | FOSM + BMA + GA | Depth and initial cost decide. |
| V4 | Bayesian worth | BMA, MLBMA | Conceptual uncertainty enters the worth. |
| V5 | Worth for the interface | FOSM for the prediction | Concentration near the interface is worth more. |
| D1 | Goal EIG | Offline–online | Optimization without new PDE solves. |
| D2 | BEL | Learned relations | Design without inversion. |
| D3 | Utility PCE | PCE orthogonality | EIG at the cost of a few NMC runs. |
| D4 | GOODE | Goal A- and D-optimality | Prediction uncertainty, not parameter uncertainty. |
| D5 | Preposterior | Bootstrap filter | Nonlinear worth without FOSM. |
| G1 | Ensemble optimization | IES + MOU | The 2050 climate inside the optimization. |
| G2 | CHERT | Cross-hole ERT | Resolution at depth, real dynamics. |
| G3 | ERT design | Resolution matrix | 22% fewer electrodes. |
| G4 | ERI for the interface | Array comparison | Dipole–dipole works in combination. |
| G5 | Probabilistic AEM | Monte Carlo | Map of chloride probability. |

### 8.1 The main gap

Four independent searches, with 120 queries in total, reached the same conclusion: no peer-reviewed paper was found that applies Bayesian OED, with EIG or with nonlinear expected variance, to a variable-density intrusion goal (the Henry problem, the Elder problem or a real aquifer). The closest are linear data worth with BMA in Beirut [V3] and the demonstration for interface movement [V5]. The methods of group D [D1, D4, D5] are ready but have not been applied to intrusion. This makes the core of the SWI-Goal-OED project new: the goal EIG with an adjoint information operator, for the salt mass near the toe of the wedge.

### 8.2 From statistical criteria to information about the goal

Intrusion networks are chosen with entropy [N3], with variability and correlation [N5], with risk [N4], with image similarity [N2] or with indices [N1]. All these criteria concern the measurements or the parameters. The data-processing inequality (Section 1.4) shows why this is not enough: the information about the goal can be much smaller, and the optimal design changes.

### 8.3 Data worth is the goal EIG in another language

In the linear-Gaussian model, the relative FOSM worth of [V5] and [V2] is exactly the R² of Lemma 1, so the goal EIG of the project and D-GOODE [D4] give the same ranking (Sections 1.5 and 1.7). The difference arises with model mixtures [V4, V3] and with binary goals (Section 9), where the equivalence breaks down.

### 8.4 The nonlinear route

For the SEAWAT model with facies, three routes are ready: preposterior reweighting of a prior ensemble [D5], surrogates of the utility function [D3] or of the model [N2], and learned data–goal relations [D2]. Ensembles conditioned with PESTPP-IES [G1] are the practical starting point for all three.

### 8.5 Geophysics as a candidate in the same criterion

Geophysical observations of the interface were the most valuable in calibration [V2], ERT design can focus on a target region [G3], and CHERT preserves resolution at depth [G2]. The practical limitation is the interpretation of formation conductivity [G4]. In the project, an ERT profile or a CHERT well can enter as a candidate with its own cost and Jacobian, through the Archie chain (Section 1.9).

### 8.6 Lessons for SWI-Goal-OED

- The screen depth must be a design variable, not only the location in plan [V3].

- The facies prior is a model average: it breaks the FOSM–EIG equivalence and requires nonlinear estimators [V4, D5].

- The regulatory goal is threshold exceedance, and risk already enters management [V1, G1] and probabilistic maps [G5]; Section 9 brings it into design.

- The offline–online decomposition [D1] and the weighted relaxation [D4] are the scalable alternatives to the greedy algorithm over the 32 wells of Fushë-Kuqe.

- The monitoring–management cycle of the James Cook University line [N3, N4, N5] is the natural framework for the drilling and MAR phases.

### 8.7 Further reading

Papers closely related to the topic that were left out because of the limit of 5 papers per group, the year, or incomplete data.

| Group | Reference | Why it is worth reading |
| --- | --- | --- |
| D | Nowak, W., de Barros, F. P. J. & Rubin, Y. (2010). Bayesian geostatistical design: Task-driven optimal site investigation when the geostatistical model is uncertain. Water Resources Research, 46. https://doi.org/10.1029/2009WR008312 | Task-driven design with an uncertain geostatistical model. |
| G | Beaujean, J., Nguyen, F., Kemna, A., Antonsson, A. & Engesgaard, P. (2014). Calibration of seawater intrusion models: Inverse parameter estimation using surface electrical resistivity tomography and borehole data. Water Resources Research, 50(8), 6828–6849. https://doi.org/10.1002/2013WR014020 | Calibration of intrusion models with ERT and wells. |
| G | Mostafaei-Avandari, M. & Ketabchi, H. (2020). Coastal groundwater management by an uncertainty-based parallel decision model. Journal of Water Resources Planning and Management, 146(6). https://doi.org/10.1061/(ASCE)WR.1943-5452.0001227 | Coastal management with SUTRA and Monte Carlo. |
| N | Amiri, H., Azadi, S., Javadpour, S., Naghavi, A. A. & Boczkaj, G. (2022). Selecting wells for an optimal design of groundwater monitoring network based on monitoring priority map: A Kish Island case study. Water Resources and Industry, 27, 100172. | The priority map and the gamma test for network size. |

# 9. The proposed innovation

## PRAG-GO: risk-oriented design for exceedance of the salinity threshold

This is a research proposal built on Part I and on the 20 papers above, not a published result. The theorems are proved here directly, with the Blackwell argument of document 13; preposterior analysis [D5] gives the nonlinear route. The numbers come from the numerical tests we ran for this proposal, with the 48-cell cross-section, head measurements and the 32 candidate wells of the previous documents.

The gap. The project and the literature on goal-oriented OED for coastal aquifers maximize the information about a mean quantity: the salt mass near the toe of the wedge, the position of the interface or the salinity in a well [V2, V5] [D1, D4]. Risk enters today only in management, as a chance constraint or as a salinization probability [V1, G1], and in probabilistic maps of threshold exceedance [G5], but not in the design criterion. Regulations and managers ask something else: “will salinity exceed the threshold (e.g., 250 mg/L chloride) in any supply zone?”. This is a binary and nonlinear goal. The question is whether it changes the optimal design.

### Theorem 1: invariance for a scalar goal

In the linear-Gaussian model, let z=c⊤m be the physical quantity (e.g., the mean salinity of a zone) and q=gz any goal that depends only on z: the threshold-exceedance indicator 1z>τ, a quantile or an arbitrary loss. Then the EIG of q is a nondecreasing function of R2d of Lemma 1, so all these goals have the same optimal design:

R12≤R22 ⇒ Igz;yd1≤Igz;yd2  for every measurable function g

Proof. The posterior of z depends on yd only through zd=Ez∣yd, so Igz;yd=Igz;zd. For standardized z, z,zd is jointly normal with Varzd=Covz,zd=R2d. If R12≤R22, the variable R12R22zd2+ε, with independent ε∼N0,R12−R14/R22, has the same joint law with z as zd1. Hence zd1 is a “garbling” of zd2 in the sense of Blackwell, and the data-processing inequality gives the result.

For the threshold, the EIG has a closed form that depends only on R2 and on the standardized threshold t=τ−μz/σz:

EIGτR2,t=hΦ−t−Eu∼N0,1hΦRu−t1−R2,  hp=−plnp−1−pln1−p

### Theorem 2: when the invariance breaks

When the risk concerns several zones, q=1maxizi>τ with z=Hm∈Rk, the ranking depends on the whole covariance-reduction matrix Kd, not on a scalar. By Theorem 1 of document 13, if Kd1≽Kd2, then d1 wins for every goal, including risk; otherwise the optimal design for risk can differ from the one for the mean. The risk EIG is computed exactly with Gauss–Hermite quadrature over zd∼N0,Kd and with the bivariate normal distribution function:

EIGmaxd=hp0−Ezdh1−Φ2τ−z1s1,τ−z2s2;r,  Σpost=Σz−Kd

where si are the posterior standard deviations, r the posterior correlation, p0 the prior exceedance probability and Φ2 the bivariate normal distribution function.

### The numerical test

We took two zones: A (cells 20–26) and the coastal zone B (36–43), with head measurements in 32 candidate wells and 3 wells per design (4 960 combinations, all computed). Theorem 1 was confirmed: for risk in one zone, the ranking by the threshold EIG matches the ranking by R2 (Spearman 0.99996, no violations). For the risk “in either zone”:

| Exceedance probability | Optimal design for risk | Optimal design for the mean | It achieves |
| --- | --- | --- | --- |
| 50% | wells 16, 31, 45 · 0.350 nats | wells 16, 37, 43 · 0.305 nats | 87.1% |
| 10% | wells 16, 33, 45 · 0.184 nats | wells 16, 37, 43 · 0.156 nats | 84.7% |
| 2% | wells 16, 33, 45 · 0.062 nats | wells 16, 37, 43 · 0.053 nats | 85.5% |

An independent Monte Carlo run with 40 000 realizations confirmed the quadrature within the statistical error. In 30 random configurations (zones, correlation length and threshold), the design for the mean was also optimal for risk in 57% of the cases, but it lost more than 5% in 30% of them and achieved only 64.5% in the worst case. As the event becomes rarer, the optimal design moves toward the one for the zone with the larger variance.

### The algorithm

- Goals: the supply zones H and the regulatory threshold τ; if the goal is a single zone, the design of Lemma 1 is enough (Theorem 1).

- Moments: Kd and Σpost for each candidate design, from adjoint sensitivities or from the ensemble.

- Criterion: EIGmaxd with 24×24 quadrature and with Φ2 computed by Gauss–Legendre quadrature.

- Comparison: if the design for risk and the design for the mean differ, the manager is given both, with the corresponding loss of information.

### Expected contributions

- An invariance theorem: for a linear-Gaussian scalar goal, every risk or quantile goal has the same optimal design as the mean.

- A closed-form EIG for threshold exceedance that depends only on R² and on the standardized threshold.

- A computable criterion for risk in several zones, with losses of up to 35% when the design for the mean is used.

- A direct link between OED and regulatory salinity thresholds.

### Validation plan

(a) The Henry problem with the goal “salinity in the supply well exceeds 1% of seawater”: comparison of the design for risk with the design for the salt mass. (b) The facies prior as a mixture of scenarios, where even a single-zone goal loses the invariance. (c) Fushë-Kuqe: the risk of exceeding 250 mg/L chloride in the supply-well zones, compared with the current design (5 monitoring wells and 3 MAR sites) and with the value 0.595 nats.

### How it builds on the 20 papers

| Component of the proposal | Papers | What is taken from them |
| --- | --- | --- |
| Threshold goal and risk | [V1] [G1] [G5] | Chance constraints; ensemble risk; map of the threshold probability. |
| Theorem 1: reduction to R² | [D4] [D1] [V5] | GOODE; offline–online goal EIG; worth for the interface. |
| The moments K_d | [V2] [V3] [V4] | FOSM; BMA and 3-D; worth under conceptual uncertainty. |
| The nonlinear case | [D5] [D2] [D3] | Preposterior analysis; BEL; utility PCE. |
| Zones and candidates | [N1] [N4] [N5] | GALDIT; the risk objective; compliance. |
| Comparison criteria | [N2] [N3] | SSIM; entropy. |
| Geophysical measurements | [G2] [G3] [G4] | CHERT; optimized ERT; arrays for the interface. |

# 10. References

### References for Part I (base literature)

[B1]	Henry, H. R. (1964). Effects of dispersion on salt encroachment in coastal aquifers. In Sea Water in Coastal Aquifers, U.S. Geological Survey Water-Supply Paper 1613-C, C70–C84. https://pubs.usgs.gov/publication/wsp1613C

[B2]	Werner, A. D., Bakker, M., Post, V. E. A., Vandenbohede, A., Lu, C., Ataie-Ashtiani, B., Simmons, C. T. & Barry, D. A. (2013). Seawater intrusion processes, investigation and management: Recent advances and future challenges. Advances in Water Resources, 51, 3–26. https://doi.org/10.1016/j.advwatres.2012.03.004

[B3]	Bakker, M., Schaars, F., Hughes, J. D., Langevin, C. D. & Dausman, A. M. (2013). Documentation of the Seawater Intrusion (SWI2) Package for MODFLOW. U.S. Geological Survey Techniques and Methods, book 6, chap. A46. https://doi.org/10.3133/tm6A46

[B4]	Strack, O. D. L. (1976). A single-potential solution for regional interface problems in coastal aquifers. Water Resources Research, 12(6), 1165–1174. https://doi.org/10.1029/WR012i006p01165

[B5]	Moore, C. & Doherty, J. (2005). Role of the calibration process in reducing model predictive error. Water Resources Research, 41(5), W05020. https://doi.org/10.1029/2004WR003501

[B6]	Hoeting, J. A., Madigan, D., Raftery, A. E. & Volinsky, C. T. (1999). Bayesian model averaging: A tutorial. Statistical Science, 14(4), 382–417. https://doi.org/10.1214/ss/1009212519

[B7]	Lindley, D. V. (1956). On a measure of the information provided by an experiment. The Annals of Mathematical Statistics, 27(4), 986–1005. https://doi.org/10.1214/aoms/1177728069

[B8]	Chaloner, K. & Verdinelli, I. (1995). Bayesian experimental design: A review. Statistical Science, 10(3), 273–304. https://doi.org/10.1214/ss/1177009939

[B9]	Archie, G. E. (1942). The electrical resistivity log as an aid in determining some reservoir characteristics. Transactions of the AIME, 146(1), 54–62. https://doi.org/10.2118/942054-G

[B10]	Howard, R. A. (1966). Information value theory. IEEE Transactions on Systems Science and Cybernetics, 2(1), 22–26. https://doi.org/10.1109/TSSC.1966.300074

[B11]	Huan, X. & Marzouk, Y. M. (2013). Simulation-based optimal Bayesian experimental design for nonlinear systems. Journal of Computational Physics, 232(1), 288–317. https://doi.org/10.1016/j.jcp.2012.08.013

[B12]	Loaiciga, H. A., Charbeneau, R. J., Everett, L. G., Fogg, G. E., Hobbs, B. F. & Rouhani, S. (1992). Review of ground-water quality monitoring network design. Journal of Hydraulic Engineering, 118(1), 11–37. https://doi.org/10.1061/(ASCE)0733-9429(1992)118:1(11)

[B13]	Spantini, A., Cui, T., Willcox, K., Tenorio, L. & Marzouk, Y. (2017). Goal-oriented optimal approximations of Bayesian linear inverse problems. SIAM Journal on Scientific Computing, 39(5), S167–S196. https://doi.org/10.1137/16M1082123

[B14]	Charnes, A. & Cooper, W. W. (1959). Chance-constrained programming. Management Science, 6(1), 73–79. https://doi.org/10.1287/mnsc.6.1.73

### References for Part II (the 20 papers)

[N1]	Mahmoudpour, H., Janatrostami, S. & Ashrafzadeh, A. (2023). Optimal design of groundwater quality monitoring network using aquifer vulnerability map. Water Resources Management, 37(2). https://doi.org/10.1007/s11269-022-03404-w

[N2]	Fan, Y., Lu, W., Miao, T., Li, J. & Lin, J. (2020). Optimum design of a seawater intrusion monitoring scheme based on the image quality assessment method. Water Resources Management, 34(8), 2485–2502. https://doi.org/10.1007/s11269-020-02565-w

[N3]	Roy, D. K. & Datta, B. (2019). Adaptive management of coastal aquifers using entropy-set pair analysis-based three-dimensional sequential monitoring network design. Journal of Hydrologic Engineering, 24(3), 04018072. https://doi.org/10.1061/(ASCE)HE.1943-5584.0001765

[N4]	Lal, A. & Datta, B. (2019). Application of monitoring network design and feedback information for adaptive management of coastal groundwater resources. International Journal of Environmental Research and Public Health, 16(22), 4365. https://doi.org/10.3390/ijerph16224365

[N5]	Sreekanth, J. & Datta, B. (2014). Design of an optimal compliance monitoring network and feedback information for adaptive management of saltwater intrusion in coastal aquifers. Journal of Water Resources Planning and Management, 140(10), 04014026. https://doi.org/10.1061/(ASCE)WR.1943-5452.0000406

[V1]	Coulon, C., Lemieux, J.-M., Pryet, A., Bayer, P., Young, N. L. & Molson, J. (2022). Pumping optimization under uncertainty in an island freshwater lens using a sharp-interface seawater intrusion model. Water Resources Research, 58, e2021WR031793. https://doi.org/10.1029/2021WR031793

[V2]	Coulon, C., Pryet, A., Lemieux, J.-M., Yrro, B. J. F., Bouchedda, A., Gloaguen, E., Comte, J.-C., Dupuis, J. C. & Banton, O. (2021). A framework for parameter estimation using sharp-interface seawater intrusion models. Journal of Hydrology, 600, 126509. https://doi.org/10.1016/j.jhydrol.2021.126509

[V3]	Safi, A., Vilhelmsen, T. N., Alameddine, I., Abou Najm, M. & El-Fadel, M. (2019). Data-worth assessment for a three-dimensional optimal design in nonlinear groundwater systems. Groundwater (online 2018). https://scholarworks.aub.edu.lb/items/b17f5569-5aac-4660-ab43-ab4991ed1aa7

[V4]	Neuman, S. P., Xue, L., Ye, M. & Lu, D. (2012). Bayesian analysis of data-worth considering model and parameter uncertainties. Advances in Water Resources, 36, 75–85. https://doi.org/10.1016/j.advwatres.2011.02.007

[V5]	Dausman, A. M., Doherty, J., Langevin, C. D. & Sukop, M. C. (2010). Quantifying data worth toward reducing predictive uncertainty. Ground Water, 48(5), 729–740. https://doi.org/10.1111/j.1745-6584.2010.00679.x

[D1]	Wu, K., Chen, P. & Ghattas, O. (2023). An offline-online decomposition method for efficient linear Bayesian goal-oriented optimal experimental design: Application to optimal sensor placement. SIAM Journal on Scientific Computing, 45(1), B57–B77. https://arxiv.org/abs/2102.06627

[D2]	Thibaut, R., Laloy, E. & Hermans, T. (2021). A new framework for experimental design using Bayesian Evidential Learning: The case of wellhead protection area. Journal of Hydrology, 603, 126903. https://doi.org/10.1016/j.jhydrol.2021.126903

[D3]	Tarakanov, A. & Elsheikh, A. H. (2020). Optimal Bayesian experimental design for subsurface flow problems. Computer Methods in Applied Mechanics and Engineering, 370, 113208. https://doi.org/10.1016/j.cma.2020.113208

[D4]	Attia, A., Alexanderian, A. & Saibaba, A. K. (2018). Goal-oriented optimal design of experiments for large-scale Bayesian linear inverse problems. Inverse Problems, 34(9), 095009. https://doi.org/10.1088/1361-6420/aad210

[D5]	Leube, P. C., Geiges, A. & Nowak, W. (2012). Bayesian assessment of the expected data impact on prediction confidence in optimal sampling design. Water Resources Research, 48(2), W02501. https://doi.org/10.1029/2010WR010137

[G1]	Coulon, C., White, J. T., Pryet, A., Gatel, L. & Lemieux, J.-M. (2024). An ensemble-based approach for pumping optimization in an island aquifer considering parameter, observation and climate uncertainty. Hydrology and Earth System Sciences, 28(1), 303–319. https://doi.org/10.5194/hess-28-303-2024

[G2]	Palacios, A., Ledo, J. J., Linde, N., Luquot, L., Bellmunt, F., et al. (2020). Time-lapse cross-hole electrical resistivity tomography (CHERT) for monitoring seawater intrusion dynamics in a Mediterranean aquifer. Hydrology and Earth System Sciences, 24(4), 2121–2139. https://doi.org/10.5194/hess-24-2121-2020

[G3]	Uhlemann, S., Wilkinson, P. B., Maurer, H., Wagner, F. M., Johnson, T. C. & Chambers, J. E. (2018). Optimized survey design for electrical resistivity tomography: Combined optimization of measurement configuration and electrode placement. Geophysical Journal International, 214(1), 108–121. https://doi.org/10.1093/gji/ggy128

[G4]	Costall, A., Harris, B. & Pigois, J. P. (2018). Electrical resistivity imaging and the saline water interface in high-quality coastal aquifers. Surveys in Geophysics, 39(4), 753–816. https://doi.org/10.1007/s10712-018-9468-0

[G5]	Delsman, J. R., et al. (2018). Large-scale, probabilistic salinity mapping using airborne electromagnetics for groundwater management in Zeeland, the Netherlands. Environmental Research Letters, 13(8), 084011. https://doi.org/10.1088/1748-9326/aad19e

Note on verification: the bibliographic data were checked against the sources listed in the methodology; no full text was opened, so all equations of the papers are representative formulations. Unconfirmed fields: [V3] the journal is inferred from the NGWA copyright line, while the volume, pages and DOI were not found, so the link to the AUB repository record is given; [D1] the DOI was not seen, so the preprint is given; [D4] the DOI is given to the best of our knowledge; [V5] the DOI comes from the USGS repository, but it was not confirmed by a direct search; [G2] the author order after the fifth author comes from the data registry; [G5] the names of the 15 co-authors were not seen; [V1] the issue number; [N1] the pages. The affiliations are partly confirmed. The DOIs of the base literature B1–B14 are given to the best of our knowledge. The numbers in the innovation section come from the numerical tests run for this document.