# The objective function of the stakeholder decision under extremes

Well site, pumping rule, certified alarm and chloride sampling, robust over intense rainfall, drought and causal change (MONTIMI-GO + SPIRANCA-GO + ROJA-GO)

Dulian Zeqiraj

## 1. What is decided

| Symbol | Decision | Decided by | Part of the method |
| --- | --- | --- | --- |
| xs | position of the supply-well screen | utility and water agency | MONTIMI-GO |
| π | pumping rule: ut+1=udec while the alarm is silent, ucut after it sounds | utility, agreed with the stakeholders | Section 4 |
| m | margin of the alarm (its certificate) | modeller | SPIRANCA-GO |
| σ=Δ,K,B | sampling rule: interval, re-anchoring window, budget of extra samples | monitoring office | ROJA-GO |
| Hmax | cap on private abstraction in a declared drought (optional lever) | water agency | Section 6 |

## 2. The model the decision acts on (the equality constraints)

The assembled system of the section, as in the assembled-system note: flow with hydraulic memory, Darcy with buoyancy, salt with two immobile zones, k=1,2:

Ss∂h∂t+∇⋅q=Wxs,Qt,Ht−k​skak h−hk,  ∂hk∂t=ak h−hk  1

q=−K ∇h+δ c ez  2

n∂c∂t+∇⋅qc−c ∇⋅q−∇⋅nD∇c+k​n bkak c−ck=0,  ∂ck∂t=ak c−ck  3

Boundaries and sources, where the storyline ξ enters:

hsea=1+δ ηξt−δz, qn,inland=qinRξ,t, W=−Q1+ut eQxs−Hhξ,t eP  4

Rξ,t=1+0.2 rteff+0.06 etR,  hξ,t=1−0.25+ϕξ,t rteff+0.15 etH  5

The goal is the sea fraction over the screen Γs and its chloride:

Jt=1ΓsΓs​c dz,  Clt=Cf+Jt Cs−Cf,  Clt>250 mg/L⇔Jt>εlim=0.0109  6

| Storyline ξ | Recorded gauge rtg vs. effective rteff | Behaviour ϕξ,t | Sea ηξ |
| --- | --- | --- | --- |
| ordinary | rg=reff, AR(1), lag-one correlation 0.7 | 0 | trend |
| intense rainfall | rg=r+2.5, reff=r+0.5 for two years (runoff) | 0 | trend (+ surge) |
| record drought | rg=reff=0.5r−2 for six years | 0 | trend |
| causal change | as the drought | 0.5 during the drought (pumping response ×3) | trend |
| compound | drought, then intense rainfall | 0 | trend (+ surge) |

## 3. The objective function

Deliver as much water as possible in the worst storyline, while counting the days the supply is unfit and the samples spent. The three terms inside the sum are the water delivered, the customer disruption days and the cost of samples:

maxxs, π, m, σ minξ∈Ξ Eωt=1TβtQ 1+ut Δt−λD Nc Δt 1{Jt>εlim}−cs nt  7

subject to (1)–(6) and

1Tt=1TPξJt>εlim≤ε  for every ξ∈Ξ  8

PξYt−bt−Yt>m≤α,  Yt=−log10Jt  9

ut+1=ucut,At=1udec,At=0,  At=1{Yt−bt≤yℓ+m}, yℓ=−log10εlim  10

nt=1{t ends a year}+1{Ft},  t∈year​1{Ft}≤B,  bt=1Ktj=1Ktej  11

m=q1−αb+δ max{0,ℓb}  12

| Term | Meaning | Source |
| --- | --- | --- |
| minξ | the worst of the five storylines; the decision must hold in all | storylines and UNSEEN (Kelder et al. 2020) |
| Q1+utΔt | water pumped in season t | utility |
| λDNcΔt 1{⋅} | customer disruption days, Nc customers, value λD per day | Becher et al. (2023) |
| csnt | cost of chloride samples; B caps the extra ones | Considine et al. (2025) |
| (8) | exceedance share at most ε in every storyline | chance constraint |
| (9), (12) | the alarm under-warns at most a share α; δ is the certified shift radius | SPIRANCA-GO, Theorem S2 |
| (10) | cut pumping in the season after an alarm | risk threshold (Schneider et al. 2022) |
| (11) | yearly sample, extra sample when the watch fires (Ft), re-anchoring on the last Kt sample errors ej | ROJA-GO |

## 4. Three equivalent forms for the stakeholders

(a) Constrained form (recommended). Drop the penalty in (7) and keep (8): the most water in the worst storyline, with the exceedance share at most ε everywhere.

maxxs,π,m,σ minξ Et​Q1+utΔt s.t. (1)–(6), (8)–(12)  13

(b) Weighted form. Keep the penalty and drop (8); λD is the shadow price of (8): the water the stakeholders accept to lose to avoid one disruption day.

(c) Minimax regret. minπmaxξVξbest−Vξπ, where Vξ is the value (7) in storyline ξ and Vξbest the best value attainable if ξ were known (Xu et al. 2021). It is less conservative than (13) when one storyline is much harder than the others.

## 5. Innovation: the inner constraints are met by construction

Problem (7)–(12) looks large, but two of its parts have closed forms, so the outer search runs over a handful of numbers.

- The margin (12) satisfies (9) for every shift of size up to δ in the span of the anchors (SPIRANCA-GO, Theorem S2), with δ taken from an UNSEEN-type ensemble instead of the short record.

- The window Kt in (11) minimises the error bound of re-anchoring (Proposition R1 of review 61):

Kt=argminK≥1σ21+1K+ρt2Δ2K+124,  Kt2Kt+1=2σ2ρtΔ2,  Kt=1 if ρtΔ≥σ  14

- What is left is the outer search over xs,udec,ucut,B,Hmax, evaluated with the simulator over the storylines; only (8) needs simulation, because (9) and (11) hold by construction.

## 6. Check on the variable-density section

Each tested rule is scored on the two numbers of the constrained form (13): the exceedance share in the worst storyline (constraint (8), ε=10%) and the water delivered in the worst storyline (the objective), mean over 20 aquifers and 6 weather sequences each, 30 years after the decision.

| Rule | Worst-storyline exceedance (8) | Worst-storyline water (13) | Samples in 30 years | Feasible for ε=10% |
| --- | --- | --- | --- | --- |
| No action | 59.3% (causal change) | 100.0% (ordinary) | 0 | no |
| Static alarm | 21.2% (causal change) | 76.2% (causal change) | 0 | no |
| Yearly sample (S5) | 11.5% (causal change) | 72.1% (causal change) | 29 | no |
| Screen sensor at 89% of the standard | 24.5% (causal change) | 84.4% (causal change) | 0 | no |
| ROJA-GO | 9.1% (causal change) | 71.2% (causal change) | 35 | yes |

Only ROJA-GO satisfies (8) with ε=10%, so among the tested rules it is the solution of (13): it delivers 71% of the planned water in the worst storyline, the causal change, where the exceedance share is 9.1%. To tighten to ε=5% the binding storyline is the causal change, so the next lever is the cap Hmax on private abstraction in a declared drought, or a lower udec; the objective (13) tells how much water each costs.

Caveat: synthetic aquifers on a 2-D section; λD, cs and ε are for the stakeholders to set. Code: experiments/spiranca_go/sx_run.py and sh_run.py; results res/sx.json and res/sh.json.