TECHNICAL NOTE  ·  THE ASSEMBLED SYSTEM  ·  OCTOBER 2026

Assembling the equations of a coastal aquifer, with the placement of recharge and pumping wells as the final goal

D. Zeqiraj

Faculty of Geology and Mining, Polytechnic University of Tirana, Albania. Assembles the equations of the author's series into one system for the Fushë-Kuqe aquifer, states the placement of recharge and pumping wells as its final goal, and adds the innovation MONTIMI-GO (montimi: the assembly).

Abstract. This note assembles into one system every equation the author's series has written for the Fushë-Kuqe coastal aquifer. The equations are: the flow equation with the two terms added to it (the memory of the silt and clay lenses, through a kernel κ_h and its tail K_h, and a learned closure Ψ); the salt equations with immobile zones; the long equation of seven ions with cation exchange, calcite, sulfate reduction and electro-diffusion; the toe and its mobility; the wells; and every boundary and initial condition. The assembled differential-algebraic system carries 31 unknowns per cell. Its block Jacobian couples flow and salt both ways and everything else one way, and its exact discrete adjoint gives the gradient of any goal in one backward sweep. The final goal is the placement of recharge and pumping wells. This is a chance-constrained, mixed-integer problem whose constraint is the chloride in the supply screen and whose safe yield is an order statistic of critical pumping rates. The innovation, MONTIMI-GO, lets this goal decide the assembly in four steps: one forward and one adjoint run weigh every term; the ion block is cut exactly, and terms below 2% of the limit are cut; the mesh is graded by the dual-weighted residual; the error that remains is calibrated into the constraint. On a vertical section with a 50-year pumping history the goal cut the memory of the lenses and the sea-level rise (weights at most 2.1×10⁻⁵, a five-hundredth of the limit) and kept the immobile zones, dispersion, buoyancy and the same-step coupling (weights 3.0×10⁻³ to 2.6×10⁻²); the one-pass split of flow and salt was 16% below the coupled screen chloride at half-year steps. The goal-guided assembly runs a 30-year forecast 2.5 times faster than the complete assembly, ranks the 25 layouts as it does (rank correlation 0.997) and, with its calibrated margin, chooses the supply well at 3.5 km and recharge well at 3.0 km at 5.95 m²/d per metre of coast, whose risk judged on 25 draws of the complete assembly is 8% against the 10% allowed. Run once afterwards, the ion block leaves the chloride goal unchanged and keeps sodium at the screen at 125 mg/L.

Keywords: seawater intrusion; variable-density flow; memory kernel; multi-species reactive transport; saltwater toe; equation assembly; discrete adjoint; goal-oriented error estimation; managed aquifer recharge; well placement; chance constraint; Fushë-Kuqe

# 1. What this note does

The series has written the equations of the Fushë-Kuqe coastal aquifer piece by piece: the variable-density flow of the base paper [1], the two terms added to the flow equation in reviews 42 and 43 and their synthesis, a memory of the silt and clay lenses and a learned closure [2–4], the long equation of multi-species transport with cation exchange, immobile zones and reactions that follows it [4], the salt equations and the chloride standard [5, 6], the mobility of the saltwater toe [7], the boundary and initial conditions [7–9], and the objectives that place monitoring, recharge and supply wells [1, 6, 10, 11]. Review 60 collected twenty papers from Oxford, Caltech, Harvard and MIT for the next step [12]. This note takes that step: it assembles all the equations into one system, and it states the placement of recharge and pumping wells as the final goal of the assembly.

Section 2 writes every equation in one notation, with the two added flow terms, the salt equations, the long multi-species equation, the toe and its mobility, the wells and every boundary and initial condition. Section 3 assembles them: the state, the discrete operators, the assembled differential-algebraic system, its block Jacobian and the order in which the blocks depend on one another. Section 4 states the final goal as a chance-constrained, mixed-integer problem constrained by the assembled system, and derives its adjoint. Section 5 is the innovation, MONTIMI-GO, an assembly set by the placement goal. Section 6 tests it on a vertical section of a coastal aquifer, solving the assembled system in full and in the assembly the goal chooses, and placing a recharge well and a supply well with both. Section 7 gives the recipe for Fushë-Kuqe and the limits, and Section 8 lists the system at a glance.

Table 1. Map of the note. The equation numbers refer to this note; the last column gives where each group was first written in the series.

| Group of equations | Equations | Block of the assembled system | Source in the series |
| --- | --- | --- | --- |
| Flow with the two added terms (memory, closure), Darcy, density | (1)–(6) | flow block h, memory states h_k | base paper; reviews 42–43; synthesis |
| Salt (sea fraction) with immobile zones, dispersion, screen chloride | (7)–(12) | salt block c, immobile states c_k | base paper; chloride study; synthesis |
| The long equation: seven ions, exchange, reactions, electro-diffusion, invariants | (13)–(21) | ion block C, exchanger β | reviews 30, 42, 43; synthesis; leak note |
| Toe and its mobility | (22)–(25) | diagnostic X(t) | review 45 (KUFIRI-GO); this note |
| Wells | (26)–(28) | controls u | complete-system note; review 60 |
| Boundary conditions | (29)–(32) | boundary data b(t) | reviews 44–45; assimilation note |
| Initial conditions | (33)–(35) | initial state y_0 | reviews 44–45; assimilation note |
| Assembly | (36)–(43) | all | this note |
| Final goal: placing the wells | (47)–(53) | controls u, adjoint λ | complete-system note; review 60; this note |
| MONTIMI-GO | (54)–(57) | the assembly itself | this note |

INNOVATION MONTIMI-GO (montimi: the assembly). The usual order is to assemble the most complete model one can and then optimise the wells on it. MONTIMI-GO reverses the order: the placement of recharge and pumping wells is stated first, and the adjoint of its constraint, the chloride at the supply screen, decides how the equations are assembled. One forward and one adjoint run of the complete system give the weight of every term for the decision. Blocks that the goal cannot see are cut exactly: the seven ions, the exchanger and the reactions (Theorem M1). Terms whose weight is below a small fraction of the chloride limit are cut, and their removal is predicted to first order. The mesh is graded by the dual-weighted residual of the goal. The error left by the cheap assembly is moved into the chance constraint as a calibrated margin, so that a layout accepted on the cheap assembly is accepted on the complete one. On the test of Section 6 the goal cut the memory of the lenses and the sea-level rise (weights at most 2.1×10⁻⁵, a five-hundredth of the limit) and kept the immobile zones, dispersion, buoyancy and the same-step coupling (weights 3.0×10⁻³ to 2.6×10⁻²);; the goal-graded assembly runs a 30-year forecast 2.5 times faster than the complete assembly, ranks the 25 layouts as it does (rank correlation 0.997) and, with its calibrated margin, chooses the supply well at 3.5 km and recharge well at 3.0 km at 5.95 m²/d per metre of coast, whose risk judged on 25 draws of the complete assembly is 8% against the 10% allowed.

# 2. The equations, in one notation

The domain Ω is the aquifer between its top and its base, x=x,y,z with z the elevation (m a.s.l.) and ez the upward unit vector; time t is in days unless stated. The unknowns are listed in Table 2. Sea water is described by its fraction c, computed from chloride, the reaction invariant (Section 2.3), so that the density follows a conservative tracer.

Table 2. The unknowns of the assembled system.

| Symbol | Unknown | Units | Equations |
| --- | --- | --- | --- |
| hf | equivalent freshwater head | m | (1), (3) |
| hk | head of the storage compartment k of the silt and clay lenses (Markovian lift of the memory) | m | (3) |
| q | Darcy flux | m/d | (4) |
| c | sea-water fraction from chloride | – | (7) |
| ck | sea-water fraction in the immobile zone k | – | (8) |
| c | concentrations of Cl, Na, K, Ca, Mg, SO₄, HCO₃ in the mobile water | meq/L | (13) |
| ck | ion concentrations in the immobile zone k | meq/L | (14) |
| β | equivalent fractions of Na, K, Ca, Mg on the exchanger | – | (15) |
| X | position of the saltwater toe | m | (22) |
| u | controls: sites, recharge rates, pumping rates and screens of the wells | m³/d | (47) |
| θ | uncertain parameters (conductivity, inflow, dispersivity, memory, sea level, ...) | various | (50) |

## 2.1 Flow, with the two added terms

The classical equation of variable-density flow balances storage, the divergence of the Darcy flux and the wells [13, 14]. The series added two terms to it [2–4]: a memory of the silt and clay lenses, through a kernel κh and its tail Kh, and a learned closure Ψ for dynamics that are missing from the model:

| Ss ∂thf+∇⋅q=w​Qwt δx−xw − t0tKht−s ∂shfx,s ds + Ψhf,x;γ | (1) |
| --- | --- |

where Ss is the specific storage (1/m), Qw the rate of well w at xw (positive for injection), Kh the memory tail and Ψ the closure with coefficients γ. The first added term is storage that is released late, by lenses that drain slowly; the second is everything the model leaves out, learned from data.

The memory was written in the synthesis article as a convolution of the head with a kernel κh. The two forms are the same, by the moment split (Lemma 1 of [4]):

| 0∞κhr hft−r dr=κ hft−0∞Khr ∂thft−r dr, κ=0∞κhr dr, Khr=r∞κhu du | (2) |
| --- | --- |

where κ is the total weight of the kernel, an instantaneous leakage that belongs to the base condition (32), and Kh is the tail, a storage memory that acts only on changes of the head and vanishes at steady state. Equation (1) keeps the tail; the steady state of the assembled system is therefore the classical one.

For lenses that drain at rates ak the tail is a sum of exponentials, completely monotone, and the memory has an exact Markovian lift with one head hk per compartment, which is what is assembled:

| Khr=k=1Kskake−akr ⇒ t0tKht−s ∂shf ds=k=1Kskak hf−hk, ∂thk=ak hf−hk | (3) |
| --- | --- |

where sk (1/m) is the storage capacity of compartment k and ak (1/d) its rate; the compartment heads start from their legacy hkt0, which absorbs the term k​skake−akt−t0hft0−hkt0 of the integration by parts. In the kernel form κhr=k​skak2e−akr and κ=k​skak (Section 3.2 of [9]).

| q=−K∇hf+ρ−ρfρf ez, K=diagK,K,Kz, hf=pρfg+z | (4) |
| --- | --- |

where K is the conductivity tensor for fresh water, p the pressure and g gravity. The buoyancy term makes salty water sink and drives the wedge.

| ρ=ρf1+δ c, δ=ρs−ρfρf, c=Cl−ClfCls−Clf | (5) |
| --- | --- |

where δ=0.025 is the density contrast and Clf, Cls the chloride of fresh water (12.92 mg/L, well 193) and of Adriatic sea water (21,781 mg/L) [6]. Viscosity is constant (Boussinesq).

| Ψhf,x;γ=j=1Jγj ψjhf,x, Jγ=1T0Ty−f0y−my;γ2dt+Rγ | (6) |
| --- | --- |

where ψj are basis functions (zone indicators, hf, ∇hf), f0 the known model, m its learned correction and R a regulariser [9]. The closure is zero in every computed model of the series, including the test of Section 6; Section 6.2 gives where along the aquifer it would matter for the placement goal.

## 2.2 The salt equations

The sea-water fraction is transported with the flux of (4), dispersed by the Bear–Scheidegger tensor and exchanged with immobile zones in the silt and clay [4, 15]. In non-conservative form, which keeps c unchanged where water is released from storage:

| n ∂tc+∇⋅qc−c ∇⋅q−∇⋅nD∇c+k=1Kn bkak c−ck=w​Qw+ δx−xw cinj,w−c | (7) |
| --- | --- |

where n is the porosity, bk the capacity ratio and ak the rate of immobile zone k, Qw+ the injection rate of recharge well w and cinj,w the sea fraction of the water it injects (zero for fresh water). Pumping wells remove water at the resident concentration and need no term.

| ∂tck=ak c−ck, k=1,…,K | (8) |
| --- | --- |

where ck is the sea fraction in zone k; at Fushë-Kuqe zone 1 is the silt (a1≈1 yr⁻¹, b1=0.3) and zone 2 the clay (a2≈0.05 yr⁻¹, b2=0.6) [3].

| nD=nDm+αTqI+αL−αTqq⊤q | (9) |
| --- | --- |

where Dm is the molecular diffusion and αL, αT the longitudinal and transverse dispersivities [16].

Eliminating the zones turns them into a memory of the salt, with its own kernel and a term for the initial disequilibrium (Proposition 1 of [4]):

| n∂tc+t0tκcs ∂tct−s ds+∇⋅qc−c∇⋅q−∇⋅nD∇c=−nk​bkake−akt−t0ct0−ckt0 | (10) |
| --- | --- |

where κcs=k​bkake−aks; the source terms of (7) are omitted for brevity. The clay zone, with a time constant of twenty years, holds salt behind an advancing front and releases it behind a retreating one.

What a supply well delivers is the chloride averaged over its screen, which is the quantity the drinking-water standard is applied to [6, 17]:

| Clwt=Clf+Cls−Clfcwt, cwt=1Lwzw−Lwzwcxw,z,tdz | (11) |
| --- | --- |

where xw is the position of the well, zw the top of its screen and Lw the screen length.

| Clw>250 mg/L=cw>εlim, εlim=250−ClfCls−Clf=0.01089 | (12) |
| --- | --- |

where εlim is the sea fraction at which the screen reaches the chloride standard: about one per cent of sea water.

## 2.3 The long equation: seven ions, exchange and reactions

The equation that follows the flow in the synthesis article transports the seven major ions with an exchanger in local equilibrium, the immobile zones and the reactions, plus a learned closure confined to the span of the stoichiometric matrix [4]:

| ∂tc+E sc+k=1Kbkakc−ck+∇⋅vc−∇⋅D∇c=Nrc+Φc,x,t;ϑ | (13) |
| --- | --- |

where v=q/n, E the exchange capacity (meq per litre of pore water, median 60), sc the equivalent fractions on the exchanger for the cations and zero for the anions, N the stoichiometric matrix (Na–Ca, Na–Mg and K–Ca exchange, sulfate reduction, calcite), r the known rates and Φ the learned closure.

| ∂tck=akc−ck+Γk ck | (14) |
| --- | --- |

where Γk holds the reactions inside zone k (sulfate reduction in the clay).

| KNa∖I=βNa aI1/zIβI1/zI aNa, I=K,Ca,Mg, βNa+βK+βCa+βMg=1, βNa=2bβ+bβ2+4aβ | (15) |
| --- | --- |

where aI are activities, zI charges, the Gaines–Thomas selectivities are 0.2, 0.4 and 0.5, and aβ=cCa/KCa2+cMg/KMg2/cNa2, bβ=1+cK/KKcNa give the closed form [6, 18, 19].

| Ω=aCa2+ aCO32−Ksp, rcal=κ gΩ, gΩ=1−Ωn1,Ω≥Ωc1−Ωcn11−Ω1−Ωcn2,Ω<Ωc | (16) |
| --- | --- |

where Ω is the calcite saturation ratio (logKsp=−8.43 at 15 °C) and g a rate law with a near- and a far-from-equilibrium branch [6, 20].

| dcSO4,kdt=−ksr cSO4,k, dcHCO3,kdt=+2ksr cSO4,k, log10γi=−0.509 zi2I1+I−0.3 I | (17) |
| --- | --- |

where ksr is the rate of sulfate reduction in the clay (median 0.05 yr⁻¹), and γi the Davies activity coefficient with ionic strength I [21].

| Ji=vci−D+Di∇ci−ziDici∇ψ, ∇ψ=−j​zjDj∇cjj​zj2Djcj, i​zici=0 | (18) |
| --- | --- |

where the electro-diffusive flux with zero electric current [22]; at Fushë-Kuqe the Péclet number is about 2.3×105 and the leak of a reactive error into the chloride forecast is suppressed by about 4×10−6 (Theorem 2 of [23]).

| u=W⊤c, W⊤N=0; f=Cl−ClfCls−Clf, ΔX=Xobs−Xf+fXs−Xf=NξX | (19) |
| --- | --- |

| BEX=Na+K+Mg−1.0716 Cl | (20) |
| --- | --- |

where the reaction invariants obey (13) without reactions; for these ions and reactions they are chloride and the charge balance, so chloride, hence c and the density, are free of every reaction and of the closure Φ (Proposition H1 of [6]). f, ΔX and BEX are the field indicators of mixing, reaction extents and base exchange [24].

| dqjdt=1τjQj ff+hj−qj, Φj=dqjdt min1,cιjcιjmixf | (21) |
| --- | --- |

where the learned exchanger of GJURMA-GO, one example of the closure Φ: the loading qj of exchanger j relaxes toward a learned equilibrium with the sea fraction f [2, 4].

## 2.4 The toe and its mobility

The toe is where the wedge of sea water meets the base of the aquifer. In the assembled system it is not a separate unknown but a functional of the salt field, the landward end of the half-sea-water line on the base:

| Xt=sup x: cx,zb,t≥12 | (22) |
| --- | --- |

where zb is the elevation of the base. Where the base is not flat, c is taken on the base surface.

Its mobility follows from the salt equation by the implicit-function theorem: on the line c=12 the total change of c along the moving toe is zero, so

| dXdt=−∂tc∂xcXt, zb, t | (23) |
| --- | --- |

where ∂tc is given by (7): the toe moves with the salt field, landward when the wedge gains salt at its tip and seaward when it loses it. This is exact and is what the assembled system computes.

Two reduced forms of the toe mobility are used for screening and for the boundary budget of review 45. The sharp-interface equilibrium of a confined aquifer with a seaward fresh flux qs per metre of coast [16, 25], and a first-order relaxation toward it:

| Xeq=K δ B22 qs, qs=qin+r​qr−w​Qw, dXdt=Xequt−XτX | (24) |
| --- | --- |

where B is the thickness, qin the regional fresh inflow and qr, Qw the recharge and pumping landward of the toe; τX is the time the wedge takes to adjust, lengthened by the immobile zones by up to the factor 1+k​bk. KUFIRI-GO used the linearised form dX/dt=−γ h0.5 km−X/τX with γ=1,000 m per m of head and τX=8 yr [7]; Section 6.5 calibrates the relaxation law on the assembled system.

The toe is not what a supply well cares about. In the sharp-interface wedge the fresh water above the interface thickens landward as Bx/X, so a screen in the top Lw metres at xw meets sea water only when

| X ≥ xwBLw2 | (25) |
| --- | --- |

where which for the screens of Section 6 (B=60 m, Lw=20 m) asks the toe to lie nine times as far inland as the well. In the dispersive, pumped aquifer the mixing zone and the upconing below the well bring the standard much earlier; the toe is a late and indirect indicator of the event that matters, which is why the goal of Section 4 is the screen chloride (11) and not the toe.

## 2.5 Wells

| w​Qwt δx−xw → w​Qwt ℓwzLw δx−xwδy−yw | (26) |
| --- | --- |

where each well is a line source over its screen, distributed by the length ℓwz open in each layer (by transmissivity where layers differ); supply wells have Qw<0 over the top of the aquifer, recharge wells Qw=qr>0 over the full thickness.

| hwell=hcell+Qw2πTlnrerw, re≈0.2 Δx; c=cinj, c=cinj at recharge wells | (27) |
| --- | --- |

where the head inside the well from the cell head (Peaceman [26]), needed when a drawdown limit is written for the well; injected water carries the composition of the recharge source (well 193) [6].

| u=zR,zS,q,Q,L, k∈CR​zkR qk ρϵx−xk, zkR∈{0,1} → 0,1 | (28) |
| --- | --- |

where zR, zS are the site indicators of recharge and supply wells over the candidate sets CR, CS, q and Q the rates, L the screens, and ρϵ a smoothed source used when the indicators are relaxed to intensities for gradient-based placement [12].

## 2.6 Boundary conditions

The boundary is split into the sea face, the top (recharge, rivers, drains), the land margins and the base, with the wells inside [6–8]:

| hf=ρsρf ηt−δz on Γsea, ηt=η0+η t+j​Ajcosωjt−φj+ηsurget+ηiat | (29) |
| --- | --- |

where hydrostatic sea water in equivalent freshwater head; the sea level has a mean, a rise, tides, surges and interannual anomalies. In the areal models the sea is linked through a conductance (the Robin condition (32) of [6]).

| c=1, c=csea where q⋅n<0, nD∇c⋅n=0 where q⋅n≥0 on Γsea | (30) |
| --- | --- |

where the sea face switches with the direction of flow, for the salt and for every ion; n is the outward normal.

| q⋅n=−qin or 0, c=0, c=c193 on Γin; nZrdsdt=P−Es−Ls, −q⋅n=R=Ls on Γtop | (31) |
| --- | --- |

where the land margins receive the regional fresh inflow or are closed; recharge is the drainage of a root-zone water balance with soil memory (moisture s, root depth Zr, rainfall P, evaporation E) [7].

| qr=Crhrt−maxhf,zb, qd=Cdhf−zd+, −q⋅n=κhf−hbase+t0tKht−s ∂shf ds on Γbase | (32) |
| --- | --- |

where rivers and drains as Robin conditions, and a leaky base whose leakage follows the moment split (2): the instantaneous part κ and the memory tail.

## 2.7 Initial conditions

| hf⋅,t0=h0, hk⋅,t0=hk,0, c⋅,t0=c0, ck⋅,t0=ck,0, c⋅,t0=c0, β⋅,t0=βeqc0 | (33) |
| --- | --- |

where every state of the assembled system needs an initial value; the memory states hk,0, ck,0 are the legacy of the lenses and the immobile zones.

| h0=ρsρfηt0−δz on Γsea, i​zic0,i=0, I​β0,I=1, ∂tg⋅,t0=f−Au0⋅,t0 on ∂Ω | (34) |
| --- | --- |

where compatibility of the boundary and initial data of order 0 and 1, charge balance and the closure of the exchanger [8].

| yt0=Φt0←tdevyss; pumping record or pyt0∣d0:T∝pyt0k​pdk∣ytk | (35) |
| --- | --- |

where the present state is either the pre-development equilibrium yss carried forward through the pumping record by the assembled system itself (Φ is its flow map; this note), or the analysis of the data d by a smoother [9]. In either case the memory states are not in equilibrium at t0.

# 3. The assembly

## 3.1 The state and the discrete operators

After discretisation in space by finite volumes on N cells, every field becomes a vector of cell values and every memory a set of local states. The assembled state is

| y=h, h1,…,hK, c, c1,…,cK, C, C1,…,CK, β, dimy=N2+2K+71+K+4 | (36) |
| --- | --- |

where h, c are the heads and sea fractions of the cells, hk, ck the memory states, C the N×7 array of ion concentrations and β the N×4 exchanger fractions. With K=2 there are 31 unknowns per cell; the Fushë-Kuqe grid of 5,589 active cells with 20 layers would carry about 3.5 million unknowns. The toe X is a functional of c, not a state.

On every face f between the cells f− and f+ (in the direction of increasing x or z), with area Af, distance df and conductance Tf=KfAf/df, the flux and the salt flux are

| Qf=−Tfhf+−hf−−f verticalTf df δ cf++cf−2, Ff=Qf c↑f−nDfAfdfcf+−cf− | (37) |
| --- | --- |

where c↑f is the upwind value (the sea value 1 on a sea face with inflow) and nDf=nDm+αLqn2+αTqt2/q the normal dispersion, with qn the normal flux of the face and qt the mean of the four perpendicular fluxes of its two cells. In matrix form Q=Ghh+Gcc+g0t, where g0 carries the sea level and the regional inflow; Div sums the outward fluxes of each cell, U selects the upwind cell and Grad takes differences across faces.

## 3.2 The assembled system

Written block by block, with MS=diagSsV, Mn=diagnV, Sk, Bk the diagonal capacities and Ak=akI the rates of the memory states, V the cell volumes and W, W+ the well sources (net and injection), the semi-discrete system is

| MS h=−Div Qh,c,t−k​SkAkh−hk+Wu+VΨh;γ, hk=Akh−hk | (38) |
| --- | --- |

| Mnc=−Div FQ,c+diagc Div Q−k​MnBkAkc−ck−diagW+c−cinj | (39) |
| --- | --- |

where with ck=Akc−ck for the immobile zones.

| MnddtC+E Sβ=−Div FQ,C+diagDiv Q C−k​MnBkAkC−Ck+MnNr+Φ | (40) |
| --- | --- |

where the third block is the long equation (13) with the same flux operator, Sβ places the exchanger fractions on the cations, and the algebraic constraint 0=gexC,β is the Gaines–Thomas equilibrium (15) in every cell; the immobile ion states follow (14).

Collected, the assembly is a differential-algebraic system of index one (the exchanger fractions are fixed by the solution through the closed form of (15)):

| M y=Fy;θ,bt,ut, 0=gy, yt0=y0 | (41) |
| --- | --- |

where θ are the uncertain parameters, bt the boundary data of Section 2.6, ut the controls of Section 2.5 and y0 the present state (35).

## 3.3 Time stepping and the block Jacobian

Implicit Euler (or a Radau IIA method whose stage equations are generated from the same weak form [27]) turns (41) into one nonlinear system per step:

| Rnyn,yn−1,zn−1;un=M yn−yn−1Δt−Fyn=0 | (42) |
| --- | --- |

where z collects the local memory states, which are eliminated exactly in every cell:

| hkn=αkhkn−1+1−αkhn, αk=11+akΔt ⇒ SkAkhn−hkn=SkAk αkhn−hkn−1 | (43) |
| --- | --- |

where and likewise for ck: the memory becomes a diagonal storage term and a source from the previous step, so it adds nothing to the size of the system that is solved.

The Jacobian of (42) has the block structure that decides how the system can be solved:

| Jn=∂Rn∂yn=JhhJhc00JchJcc00JChJCcJCCJCβ00JβCJββ | (44) |
| --- | --- |

where with the flow–salt block

| Jhh=MSΔt+k​SkAkαk+Div Gh, Jhc=Div Gc, Jch=Div diagc↑Gh−diagc Div Gh | (45) |
| --- | --- |

| Jcc=MnΔt+k​MnBkAkαk+DivdiagQ U+diagc↑Gc−diagdGrad |  |
| --- | --- |

| −diagDiv Q−diagc Div Gc+diagW+ | (46) |
| --- | --- |

where c↑ is Uc plus the sea inflow values, df=nDfAf/df. The dispersion coefficients are taken from the fluxes of the previous step, so the step also depends on yn−1 through them; that dependence enters the adjoint of Section 4.4. The test assembles these blocks exactly; Newton then converges quadratically (largest residuals 7×10⁻¹, 2×10⁻², 2×10⁻³, 6×10⁻⁶, 2×10⁻¹¹, 1×10⁻¹³ in one half-year step), which is the check that the blocks are right.

Proposition A1 (the order of the blocks). In the assembled system (38)–(40) the only two-way coupling between blocks is between flow and salt: the flux depends on the salt through the density, and the salt on the flow through the flux. The ion block, the exchanger, the reactions, the closure Φ and the toe are driven by h,c and feed nothing back, apart from electro-diffusion, which couples the ions back to chloride at a relative order of 4×10−6 at Fushë-Kuqe. The Jacobian (44) is therefore block lower-triangular below the 2×2 flow–salt block.

Proof. The density (5) depends on c, which is computed from chloride, and chloride is a reaction invariant (Proposition H1 of [6]), so neither the reactions nor Φ, which lies in the span of N, nor the exchanger enter the flow or the salt equations. The flux (4) depends on hf,c only; the ion equations depend on Q and on the ions; the exchanger on the ions; the toe (22) on c. Hence ∂Rh/∂C,β=0 and ∂Rc/∂C,β=0. The electro-diffusive coupling (18) is the exception, bounded by Theorem 2 of [23]. □

Figure 1. (a) The assembled system: boundary and initial data and the controls drive the flow block; flow and salt are coupled both ways (flux and density); the ion block is driven by them and feeds nothing back; the goal, the chloride at the supply screen, sends its adjoint back into the flow and salt blocks only. (b) The pattern of the assembled Jacobian (44) on a grid of 8 × 4 cells: flow–salt block (dark), ion and exchanger blocks (grey) below it, and the empty blocks above them that make the system block lower-triangular (Proposition A1).

## 3.4 Two clocks, and why the flow–salt block is solved whole

Heads diffuse with Dh=T/S≈2×107 m²/d and feel a pumping change kilometres away within a day; salt moves kilometres in decades. The ratio of the two clocks is about 10−5 [4], which suggests solving the flow and the salt one after the other, with the density lagged by a step. The test shows that this split, though consistent, is not harmless for the goal. Picard iteration between the two blocks did not converge at half-year steps (the change of the sea fraction between successive iterations grew from 6×10⁻² after the first to 3×10⁻¹ after thirty), while Newton on the whole block converged quadratically. The one-pass split approached the coupled value of the screen sea fraction only as the step was cut: it was 24%, 16%, 10%, 2% below the coupled value at steps of 1, 0.5, 0.25, 0.125 years, while the coupled value itself changed by at most 0.7% over the same steps. The flow–salt block is therefore solved whole, as in the monolithic solvers of [28, 29], and Section 5 lets the goal confirm that choice.

## 3.5 The reductions of the series as cuts of the assembly

Table 3. The models computed in the series are cuts of the assembled system. MONTIMI-GO chooses the cut by the goal instead of by habit.

| Model of the series | Blocks kept | Blocks cut or frozen | Exact for |
| --- | --- | --- | --- |
| R1: areal steady flow with the Ghyben–Herzberg test [1] | flow (steady, depth-integrated) | salt, memory, ions; interface from the head | head-based questions at steady pumping |
| R2: Strack potential with a Gaussian mixing zone [5] | flow (steady), sharp interface + mixing width | salt transport, memory, ions | steady screen chloride, threshold form of P1–P2 [6] |
| R3: vertical variable-density section (this test, Section 6) | flow, salt, memory, immobile zones, ions | the third dimension | transients of a cross-section |
| R4: seven-ion flow line [2, 3] | salt, ions, exchange, memory | flow (prescribed velocity history) | chemistry along a flow path |
| Toe relaxation law (24) [7] | one ODE for X | everything else | the toe, slowly varying forcing |

# 4. The final goal: placing recharge and pumping wells

## 4.1 Decisions, goal and constraint

The decision is where to put recharge wells and supply wells, how much to inject and pump and how to screen, within the constraints of the series (spacing, exclusion around supply wells, budget; (50) of [6]):

| u=zR,zS,q,Q,L∈U, U=xk−xl≥dmin, xr−xw≥dex, j​cjzj≤B | (47) |
| --- | --- |

where zR, zS are binary site indicators over the candidate sets, q, Q the recharge and pumping rates and L the screens; cj are costs and B the budget.

The goal of every supply well is the screen chloride (11) at the end of the planning horizon tH; following PUSI-GO [6], chloride is the standard that binds, and sodium is checked afterwards (Proposition P3 there):

| Jwu;θ=cwtH;u,θ=ww⊤ctH, event maxwJw>εlim | (48) |
| --- | --- |

where ww averages the cell values over the screen of well w (interpolated linearly to xw, so that every grid computes the same functional).

| maxu∈U w​vxwQw−λRr​qr s.t. PrθmaxwJwu;θ>εlim≤α, My=Fy;θ,b,u, 0=gy | (49) |
| --- | --- |

where vxw is the value of a cubic metre pumped at xw, net of conveyance to the demand at the coast, and λR the price of recharged water. The problem is mixed-integer (the sites), chance-constrained (the posterior of θ) and constrained by the assembled system [30–32]. At scale it can be solved with learned constraints inside a mixed-integer program [33], learned neighbourhood search [34] or Bayesian optimisation over mixed spaces [35], with the sites fixed first and the rates as recourse [36]; with a few candidate sites, as in the test, it is enumerated.

| 1Nθi=1Nθ1maxwJwu;θi>εlim≤α | (50) |
| --- | --- |

where the sample-average form over Nθ posterior draws, which is what is solved.

Proposition G1 (the safe yield of a layout is a quantile). Fix the sites and the recharge of a layout ℓ and let Jℓ,Q;θ be nondecreasing in the pumping Q for every θ. Define the critical rate Qcℓ;θ=sup Q≥0:Jℓ,Q;θ≤εlim. Then the largest Q that satisfies (50) is the order statistic

| Q⋆ℓ=Qc,mℓ, m=αNθ+1 | (51) |
| --- | --- |

where the m-th smallest of the critical rates of the draws, and the placement problem becomes a choice among layouts of the value v Q⋆ℓ−λRqℓ.

Proof. By monotonicity Jℓ,Q;θi>εlim if and only if Q>Qcℓ;θi, so the left side of (50) is the fraction of draws with Qc<Q; it is at most α exactly for Q≤Qc,m. □

Monotonicity holds in every run of the test (more pumping never freshened a screen within the horizon). It reduces the chance constraint to one root-finding per draw and layout, each a few forward runs of the assembled system.

## 4.2 The adjoint of the assembled system

Gradients of the goal with respect to every rate and every candidate site, and the weights of Section 5, come from one backward sweep. With λn the adjoint of step n and μn that of the memory states,

| Jn⊤λn=δn,Nt∂J∂yNt​⊤−∂Rn+1∂yn​⊤λn+1+B⊤μn, μn−1=α μn−∂Rn∂zn−1​⊤λn | (52) |
| --- | --- |

where for n=Nt,…,1 with λNt+1=0, μNt=0; B and α are the diagonal maps of (43); a last solve with the Jacobian of the pre-development equilibrium carries the adjoint into the initial state. ∂Rn+1/∂yn contains the storage and the dependence of the dispersion coefficients on the previous fluxes.

| dJdp=−n​λn⊤∂Rn∂p, dJdQw=−n​λh,n⊤ew, dJdqrx=n​λh,n−diagcnλc,n​⊤erx | (53) |
| --- | --- |

where ew, erx distribute a unit pumping over the supply screen and a unit injection over the full thickness at the candidate x; one sweep gives the sensitivity of the goal to a recharge well at every column of the grid. In the test the adjoint gradients agree with central differences of the complete assembly to 8 significant digits for the pumping rate and for the recharge rate (supply well at 2.0 km, recharge well at 1.0 km, step 10−5). At a layout where the net seaward flux past the supply well nearly vanishes they agree to 0.3%: there some face fluxes change direction under the perturbation, and the upwind goal is only piecewise differentiable.

# 5. MONTIMI-GO: an assembly set by the placement goal

The assembled system of Section 3 is large, and the placement problem of Section 4 asks for it thousands of times: one root-finding per layout and posterior draw, each a few forecasts. Review 60 proposed to let the goal decide the assembly [12]. Four results make this concrete. The first is exact, the second and third are first-order, and the fourth moves what is left into the constraint.

Theorem M1 (exact cuts). Let the goal J depend only on the flow and salt states h,c, as the screen chloride (48) does, and let the assembled Jacobians be block lower-triangular below the flow–salt block (Proposition A1). Then at every step the adjoint vanishes on every downstream block, λC,n=0 and λβ,n=0. Consequently (i) J and its gradient with respect to every control are the same whether or not the ions, the exchanger, the reactions and the closure Φ are assembled; (ii) the weight (54) of every parameter inside those blocks (selectivities, exchange capacity, reaction rates, closure coefficients) is zero; (iii) with electro-diffusion, (i) and (ii) hold up to the relative order of the leak bound, 4×10−6 at Fushë-Kuqe [23].

Proof. In the adjoint recursion (52) the right-hand side has no downstream component at n=Nt, because J does not depend on C,β. The transposed Jacobian Jn⊤ is block upper-triangular, so its downstream rows read JCC⊤λC+JβC⊤λβ=rC and JCβ⊤λC+Jββ⊤λβ=rβ, with an invertible diagonal block (the ion transport operator and the exchange equilibrium of index one). With rC=rβ=0 the only solution is zero. The cross-step Jacobian ∂Rn+1/∂yn has the same triangular structure, so the downstream right-hand side stays zero at every earlier step, by induction backwards. Statement (i) follows because the flow–salt rows of the forward system do not contain the downstream unknowns. Statement (ii) follows because a parameter p of a downstream block has ∂Rn/∂p supported on downstream rows only, so λn⊤∂Rn/∂p=0. Statement (iii) follows from the perturbation bound of [23]. □

The theorem is the reason the series could put a learned chemistry closure into the model without touching the chloride forecast (the "cut" of [4, 23]), here turned into a rule of assembly: for a chloride goal the ion block is computed once, after the decision, to check sodium and hardness for the chosen layout (Proposition P3 of [6]).

Proposition M2 (the goal weight of a term). Give every term T of the assembly a scale sT, with sT=1 for the term as assembled and sT=0 for the term removed; for the coupling, let sT interpolate between the same-step density and the density of the previous step. The weight of T for the goal, and the first-order change of the goal when T is removed, are

| wT=dJdsTsT=1=−n​λn⊤∂Rn∂sT, JsT=0−JsT=1=−wT+OΔy2 | (54) |
| --- | --- |

where with one forward and one adjoint run of the complete assembly for all terms at once. The rule of assembly is

| cut T⇔maxreference caseswT<τ εlim | (55) |
| --- | --- |

where with τ a small fraction (here 2%, i.e. wT<2.2×10⁻⁴, about 5 mg/L of chloride at the screen). The weight is the derivative with respect to the scale of the term, not the residual difference at a fixed state: removing the sea-level rise at a fixed state shifts the heads of the sea face by tens of centimetres and reverses inflow and outflow there, while the actual change of the goal is nil because the whole head field rises with the sea.

Proof. J is a function of sT through the solution of Rny;sT=0; differentiating the constraints and using (52) gives dJ/dsT=−n​λn⊤∂Rn/∂sT [37, 38]. Taylor's theorem in sT gives the remainder. □

Proposition M3 (a mesh graded by the goal). Let yH be the solution of the assembly on a coarse mesh and IyH its interpolation to the complete mesh h. The dual-weighted residual estimates the error of the goal and gives a cell-wise indicator [39]:

| JhIyH−Jhyh≈n​λn⊤RnIyH, ηj=n​i∈column j​λn,i Rn,iIyH | (56) |
| --- | --- |

where the column indicator ηj is summed over the reference cases. The goal-graded mesh equidistributes ηj/Δxj1/3 over the same number of columns as the uniform coarse mesh, with a cap on the widest column.

The estimate itself is only first order and, for a goal that sits in the tail of a sharp front, it was not reliable in the test (Section 6.3); learned conservative fluxes are one way to keep such fronts sharp on coarse cells [40]. MONTIMI-GO therefore uses the indicator only to rank where the mesh matters, and does not rely on the estimate; the remaining error is calibrated by Proposition M4.

Proposition M4 (the error of the cheap assembly moves into the constraint). Let A be the cheap assembly and F the complete one. Draw C calibration cases ℓi,θi like the decision cases, compute ri=QcAℓi;θi−QcFℓi;θi, and let mβ be the ⌈1−βC+1⌉-th smallest ri. Solve the placement on A with the critical rates lowered by mβ:

| QA,β⋆ℓ=Qc,mAℓ−mβ+, PrJFℓ,QA,β⋆ℓ;θ>εlim≤α+β | (57) |
| --- | --- |

where for a new draw θ exchangeable with the calibration cases, up to the sampling error of the draws.

Proof. If the case is exchangeable with the calibration cases, Prr>mβ≤β by the rank argument of split conformal prediction [41, 42]. On {r≤mβ} we have QcF≥QcA−mβ, so exceeding the standard on F at QA,β⋆ implies QcA−mβ<QA,β⋆, i.e. QcA<Qc,mA, which has probability at most α (Proposition G1). A union bound gives α+β. □

The procedure. (1) Choose a few reference cases: layouts at their critical rates on the complete assembly. (2) For each, one forward and one adjoint run of the complete assembly give the weights of all terms, the gradients with respect to every rate and every recharge site, and the mesh indicator. (3) Cut the downstream blocks (Theorem M1) and the terms below the tolerance; keep the rest; grade the mesh. (4) Run C calibration cases on both assemblies and compute the margin. (5) Solve the placement on the cheap assembly with the margin. (6) Run the complete assembly, ions included, once for the chosen layout: chloride against the margin, sodium and hardness against Proposition P3 of [6]. The cheap assembly can be replaced or completed by a neural-operator surrogate trained on runs of the assembly [43, 44], or differentiated end to end for gradient-based placement [45]; the margin of Proposition M4 applies to either, and the decision should then be operated and revalidated as in field practice for injection wells [46].

# 6. The test

## 6.1 The aquifer, the history and the decision

The test assembles the system on a vertical section of a confined coastal aquifer, the reduction R3 of Table 3. It is 4 km long and 60 m thick, its top is 10 m below sea level, and it has silt and clay lenses with hydraulic memory, immobile zones, and the ion block with exchange. The parameters are those of Fushë-Kuqe where the series has them (Table 4). The present state is made as at the field site, where the coastal screens salinised under pumping [47]: from the pre-development equilibrium, 50 years of pumping 4 m²/d per metre of coast from a well field 1.5 km from the coast. At the end of the history its screen delivers 364 mg/L of chloride in the median aquifer and the toe has moved from 349 m to 1070 m. The decision is where to put the new supply well (2.0, 2.5, 3.0 or 3.5 km from the coast), where to put one recharge well injecting 1 m²/d of fresh water (none, 0.5, 1.0, 1.5 km, the old well field, 2.0, 2.5 or 3.0 km), and how much to pump, so that the probability that the supply screen exceeds 250 mg/L at the end of a 30-year horizon is at most 10%: 25 layouts.

Table 4. The test bed. ξ denotes independent standard normal variables.

| Quantity | Value or prior | Source |
| --- | --- | --- |
| Grid of the complete assembly | 120 × 20 cells (33 m × 3 m); half-year steps | this note |
| Conductivity K, anisotropy, porosity | 100 e0.25ξ m/d (T ≈ 4,700–7,700 m²/d), Kz/K=0.1, n=0.25 | complete-system note, Table 2 |
| Specific storage; lens capacities and rates | 5×10−6 1/m; sk=2×10−5, 5×10−5 1/m; ak=1, 0.05 yr⁻¹ | reviews 43–45 (rates); this note |
| Immobile zones | b1=0.3, b2∼U0.3,0.9; a1=1, a2=0.05 yr⁻¹ | RITMI-GO |
| Dispersivities | αL=20 e0.4ξ m, αT=αL/10 | chloride study (10 m, 1 m) widened |
| Regional fresh inflow | 4 e0.2ξ m²/d per metre of coast | recharge 0.5 mm/d over about 8 km |
| Sea-level rise | N4,22 mm/yr | KUFIRI-GO |
| Density contrast; end-members; standards | 0.025; well 193 and Adriatic water; Cl 250, Na 200 mg/L | complete-system note |
| Exchange | E = 60 meq/L; selectivities 0.2, 0.4, 0.5 | GJURMA-GO |
| Draws | 40 design draws (the cheap assemblies choose), 40 evaluation draws (the complete assembly judges) | this note |

Figure 2. The test bed with the median parameters. (a) The present state after the 50-year history: the old screen at 1.5 km delivers 364 mg/L of chloride; the toe (triangle) is at 1070 m, against 349 m before development. (b) Thirty years later with a supply well at 2.0 km pumping 5.26 m²/d per metre of coast, its critical rate, and a recharge well at 2.5 km; the red line is the 250 mg/L contour, which touches the bottom of the screen; the toe has moved to 1330 m. (c) Where the goal looks: the adjoint of the salt equation for the screen chloride, integrated over the 30 years. The ticks on the base are the column edges of the goal-graded mesh.

## 6.2 What the goal kept and what it cut

Table 5 and Figure 3 give the weights of the eight terms at four reference cases, each a layout at its critical rate on the complete assembly, so that the screen sits exactly at 250 mg/L: S2.0-R2.5, S2.5, S3.0-R1.5, S3.5-R1.0 (S: supply well, R: recharge well, km from the coast). One forward and one adjoint run of the complete assembly took 11 s and about 4 s per case. The hydraulic memory of the lenses and the sea-level rise weigh at most 2.1×10⁻⁵, a five-hundredth of the limit. The lenses release stored water slowly, but at these capacities they change the head at the screen by millimetres. The rise lifts the whole head field with the sea and moves almost no water. The rule cut all three; removing them changed the goal by at most 2.1×10⁻⁵, as predicted to within 25%, and to within 1% in three of the four cases. The immobile zones, transverse dispersion and buoyancy weigh 6.3×10⁻³ to 2.6×10⁻², between half and twice the limit itself: they set the width and the speed of the mixing zone that reaches the screen. A 10% reduction of the immobile capacities or of the transverse dispersivity was predicted to within 8%. The same-step coupling weighs 3.0×10⁻³ to 6.6×10⁻³ and was kept. For this term the first-order number only says that it matters: the goal is not monotone in the lag (it rose by 5.5×10⁻⁵ for 1% of lag and by 7.5×10⁻⁴ for 20% in a separate check), and the full one-pass split moved it by -4.3×10⁻³ to -1.9×10⁻³, in the opposite direction to the first-order value.

Table 5. Goal weights wT of (54) over the four reference cases (sea fraction at the screen per unit scale of the term; the limit is εlim=0.0109 and the tolerance τεlim=2.2×10⁻⁴). Check: the change of the goal predicted from the weight against the change computed by running the complete assembly without the term (cut terms), with the term reduced by 10% (kept terms), or with the one-pass split.

| Term | Weight $w_T$ (range over 4 cases) | Check: predicted / actual change | Decision |
| --- | --- | --- | --- |
| Hydraulic memory, silt lenses | 6.6×10⁻⁷ – 9.6×10⁻⁶ | removed: 9.6×10⁻⁶ / 9.6×10⁻⁶ (largest) | cut |
| Hydraulic memory, clay lenses | 7.8×10⁻⁶ – 2.1×10⁻⁵ | removed: 2.1×10⁻⁵ / 2.1×10⁻⁵ (largest) | cut |
| Immobile zone, silt | 6.3×10⁻³ – 1.4×10⁻² | −10%: 6.3×10⁻⁴ / 6.2×10⁻⁴ (first case) | kept |
| Immobile zone, clay | 1.0×10⁻² – 1.2×10⁻² | −10%: 1.1×10⁻³ / 1.1×10⁻³ (first case) | kept |
| Transverse dispersion | 7.5×10⁻³ – 1.3×10⁻² | −10%: 1.3×10⁻³ / 1.4×10⁻³ (first case) | kept |
| Sea-level rise | 3.6×10⁻⁶ – 6.8×10⁻⁶ | removed: 6.8×10⁻⁶ / 7.7×10⁻⁶ (largest) | cut |
| Buoyancy (density coupling) | 9.8×10⁻³ – 2.6×10⁻² | δ −10%: -2.6×10⁻³ / -3.8×10⁻³ (first case; δ also enters the sea face) | kept |
| Same-step flow–salt coupling | 3.0×10⁻³ – 6.6×10⁻³ | one-pass split: -3.0×10⁻³… / -4.3×10⁻³ to -1.9×10⁻³ | kept |

Figure 3. The goal weights of every term of the assembly, four reference layouts (dots). Grey terms lie three orders of magnitude below the tolerance (dashed) and are cut; dark terms lie near the limit itself (dotted) and are kept. The ion block is not shown: its weight is zero by Theorem M1.

The learned closure Ψ is zero in the test, but its weight shows where dynamics missing from the model would matter for the decision. A missing source equal to 10 mm/yr of recharge spread over a 500-m band changes the screen sea fraction by at most 2.8×10⁻⁵ in the band at the coast and by up to 7.6×10⁻⁴ (7% of the limit) in the bands landward of the supply well, where every litre of fresh water passes the well. The closure, and the data that identify it, are therefore worth most for the inland water balance, not at the coast.

## 6.3 The mesh

The indicator (56), summed over the four reference cases, is largest where the supply screens draw water through the mixing zone (Figure 2c). The goal-graded mesh puts columns of 33–67 m at the candidate supply sites and at the old well field and columns of 100–133 m elsewhere (39 columns; the uniform coarse mesh has 40 columns of 100 m). Table 6 compares, on the four reference cases and eight further cases with random layouts and draws, the error of each coarse assembly in the goal with its dual-weighted estimate. Both coarse meshes overestimate the screen sea fraction, because a coarse cell smears the mixing zone upward. The goal-graded mesh does so on every case by 6.0×10⁻³ to 1.3×10⁻², and its estimate has the right sign and order on every case (ratio of estimate to error 0.40–0.87). The uniform mesh errs by anywhere between 2.4×10⁻³ and 1.8×10⁻², depending on where the supply well falls relative to its columns, and its estimate has the wrong sign in 6 of 12 cases. The goal-graded mesh is therefore not always the more accurate, but its error is the more predictable, which is what Proposition M4 needs.

Table 6. Error of the coarse assemblies in the goal (screen sea fraction, limit 0.0109) at the critical rate of the complete assembly, against the dual-weighted-residual estimate (56). Uniform: 40 columns of 100 m; goal-graded: 39 columns of 33–133 m. Same physics as the complete assembly.

| Case | True error, uniform | Estimate, uniform | True error, goal-graded | Estimate, goal-graded |
| --- | --- | --- | --- | --- |
| reference: S2.0-R2.5, Q = 5.26 | 2.36×10⁻³ | -2.02×10⁻² | 1.28×10⁻² | 5.32×10⁻³ |
| reference: S2.5, Q = 5.12 | 1.53×10⁻² | 8.45×10⁻³ | 1.05×10⁻² | 6.08×10⁻³ |
| reference: S3.0-R1.5, Q = 6.20 | 1.21×10⁻² | 1.06×10⁻² | 9.04×10⁻³ | 7.84×10⁻³ |
| reference: S3.5-R1.0, Q = 6.74 | 2.69×10⁻³ | -4.65×10⁻³ | 5.98×10⁻³ | 4.63×10⁻³ |
| random: S3.5-R2.0, Q = 10.28 | 3.99×10⁻³ | -2.42×10⁻³ | 8.43×10⁻³ | 6.22×10⁻³ |
| random: S3.5-R1.0, Q = 9.02 | 2.77×10⁻³ | -3.93×10⁻³ | 7.39×10⁻³ | 5.76×10⁻³ |
| random: S2.0-R3.0, Q = 3.84 | 3.47×10⁻³ | -1.67×10⁻² | 1.11×10⁻² | 4.48×10⁻³ |
| random: S3.5, Q = 6.62 | 3.49×10⁻³ | -4.51×10⁻³ | 7.48×10⁻³ | 5.85×10⁻³ |
| random: S3.0, Q = 4.46 | 1.20×10⁻² | 1.11×10⁻² | 8.11×10⁻³ | 6.95×10⁻³ |
| random: S2.5-R3.0, Q = 7.21 | 1.81×10⁻² | 8.09×10⁻³ | 1.28×10⁻² | 7.18×10⁻³ |
| random: S3.0-R0.5, Q = 3.95 | 1.13×10⁻² | 9.92×10⁻³ | 7.03×10⁻³ | 5.23×10⁻³ |
| random: S3.0-R2.5, Q = 6.94 | 1.04×10⁻² | 9.88×10⁻³ | 8.16×10⁻³ | 6.23×10⁻³ |

## 6.4 Placing the wells

Every assembly solves the placement problem (49) the same way. For each of the 25 layouts and each draw it finds the critical rate by a bracketed secant on the logarithm of the goal (Proposition G1; 5.5 forecasts per root on average). It takes the safe yield as the order statistic (51) at α=10%, and chooses the layout of largest value vxsQ⋆−λRqr. The value counts water at the supply well less 0.2 per km of conveyance beyond 2 km, and recharged water at 0.5 of its value. The complete assembly does this on 25 evaluation draws and acts as the referee. The cheap assemblies do it on separate design draws (25 for the goal-guided and uniform assemblies, 10 for the two counter-examples), with and without the calibrated margin of Proposition M4 estimated from 40 calibration cases. The full assembly needed 15 minutes per draw for all layouts.

Table 7. The placement on each assembly. Layout chosen by the value vxsQ⋆−λRqr with v=1−0.2 xs−2 km/km and λR=0.5; safe rate in m² per day per metre of coast; risk: fraction of the 25 evaluation draws of the complete assembly on which the screen exceeds 250 mg/L at that layout and rate (target 10%, guarantee 10% + β with β = 10%). Cost: median wall time of one 30-year forecast.

| Assembly | Layout chosen | Safe rate (m²/d) | Risk on the complete assembly | Cost of one forecast |
| --- | --- | --- | --- | --- |
| Complete assembly (referee, 25 evaluation draws) | S3.5-R3.0 | 6.36 | ≤ 10% by construction | 3.9 s |
| goal-guided (MONTIMI-GO), 25 design draws, no margin | S3.5-R3.0 | 5.80 | 4% | 1.6 s (2.5× faster) |
| … with calibrated margin m = −0.15 m²/d | S3.5-R3.0 | 5.95 | 8% |  |
| uniform coarse mesh, same terms, 25 design draws, no margin | S3.5-R3.0 | 5.90 | 4% | 1.7 s (2.3× faster) |
| … with calibrated margin m = −0.08 m²/d | S3.5-R3.0 | 5.98 | 8% |  |
| goal mesh, density lagged one step, 10 design draws, no margin | S2.5-R3.0 | 5.16 | 20% | 0.2 s (17.8× faster) |
| … with calibrated margin m = −0.001 m²/d | S2.5-R3.0 | 5.16 | 20% |  |
| goal mesh, immobile zones removed, 10 design draws, no margin | S2.5-R3.0 | 4.39 | 4% | 1.8 s (2.3× faster) |
| … with calibrated margin m = −0.38 m²/d | S2.5-R3.0 | 4.77 | 8% |  |

Figure 4. (a) Safe pumping rate Q⋆ of every layout on the complete assembly (black, evaluation draws) and on the cheap assemblies (design draws, no margin); the layout chosen by the complete assembly is shaded. (b) Risk of the layout and rate each cheap assembly chooses, judged on the complete assembly, without and with the calibrated margin; dashed: α=10%, dotted: α+β. (c) Calibration residuals Qccheap−Qccomplete and the margins (dashed).

Both coarse assemblies are conservative: they overestimate the screen sea fraction (Table 6), so their safe rates lie below those of the complete assembly at 25 and 25 of the 25 layouts, by 0.51 m²/d on average for the goal-guided assembly and 0.51 m²/d for the uniform one. Yet they rank the 25 layouts almost exactly as the complete assembly does (rank correlation 0.997 and 0.999). Without a margin the goal-guided assembly chooses safely but gives water away. It chooses the supply well at 3.5 km and recharge well at 3.0 km at 5.80 m²/d, with a risk of 4% on the complete assembly; the complete assembly would allow 6.36 m²/d at that layout. The calibrated margins are negative: −0.15 m²/d for the goal-guided and −0.08 m²/d for the uniform assembly. A negative margin returns the water that the conservative bias withholds, and the guarantee of Proposition M4 still holds. With its margin the goal-guided assembly chooses the supply well at 3.5 km and recharge well at 3.0 km at 5.95 m²/d, with a risk of 8% on the complete assembly. The uniform assembly chooses the supply well at 3.5 km and recharge well at 3.0 km at 5.98 m²/d, with a risk of 8%. The complete assembly itself chooses the supply well at 3.5 km and recharge well at 3.0 km at 6.36 m²/d. The goal-guided assembly chooses the complete assembly's own best layout. Over conveyance prices from 0 to 0.4 per km, the goal-guided assembly with its margin chooses the complete assembly's layout for 4 of 5 prices and loses at most 0.1% of the value elsewhere. The two counter-examples remove a term the goal kept, on the same 10 design draws, so they can be compared with each other. Lagging the density one step (S) removes the conservative bias of the goal mesh: the 90% calibration residual is −0.001 m²/d against −0.15 for the goal-guided assembly, so the lag raises the critical rates by about 0.15 m²/d, in the direction of the one-pass split error of Section 3.4. With nothing left to cover the sampling error of 10 draws, its choice, the supply well at 2.5 km and recharge well at 3.0 km at 5.16 m²/d, exceeds the standard in 20% of the evaluation draws of the complete assembly, 2.0 times the risk allowed, with or without its margin. Without the immobile zones (N) the assembly is conservative by at least 0.38 m²/d in nine calibration cases of ten; that covers the same sampling error (risk 4% without the margin, 8% with it), but it chooses the supply well at 2.5 km and recharge well at 3.0 km and gives up 0.7% of the value. Part of both risks comes from having only 10 design draws; the difference between the two, on the same draws, is the effect of the term removed. One 30-year forecast took 3.9 s on the complete assembly and 1.6 s on the goal-guided one, 2.5 times less; the whole placement (all layouts, all draws) took 6.1 CPU-hours on the complete assembly and 1.9 on the goal-guided one.

## 6.5 The toe

The toe moves as (23) says, and the relaxation law (24) describes it. Fitted to each of the 75 forecasts of the complete assembly (three layouts at their safe rates, 25 evaluation draws), with the head at mid-depth 0.5 km from the coast as the driver, it reproduces the toe over 30 years with a median largest error of 1.2 and 0.8 m for 2 of the 3 layouts, and of 20 m for the layout where the toe hardly moves (median displacement 4 m), so that there is little to fit. The median relaxation time is 7–12 years and the median γ is 556–1191 m per m of head (KUFIRI-GO assumed 8 years and 1,000 m). As a proxy for the goal the toe does better here than the sharp-interface picture suggests: a threshold on the toe position, tuned to flag as many aquifers as actually exceed 250 mg/L at the screen, catches 6 of the 7 exceedances of the three layouts. With 7 events this is thin evidence, and the toe explains only part of the variation of the screen sea fraction (correlation 0.51–0.67). The inequality (25) says why the toe cannot replace the screen: the screen meets sea water through the mixing zone and the upconing below the well, long before a sharp-interface toe would reach nine times the distance of the well, so a toe criterion must be calibrated against the screen, layout by layout.

## 6.6 The ions, computed once

For the first reference layout at its critical rate, the complete assembly was run again with the seven-ion block and Gaines–Thomas exchange (E = 60 meq/L) through the whole history and forecast. The goal was identical to the last digit, as Theorem M1 says. The chloride computed by the ion block agrees with the chloride of the salt block within 3.0×10⁻³ mg/L. The ion block cost 1 s of the 13 s of that run (11%). The check it is kept for is sodium. At the screen it is 125 mg/L at the end of the horizon, when chloride is 250 mg/L: below 200 mg/L, as Proposition P3 of [6] predicts for intrusion with sodium taken up by the exchanger. Hardness at the screen is 6.30 meq/L against 3.65 for the recharge water. Behind the recharge well, 100 m landward at mid-depth, the water carries 18 mg/L of sodium and 13 mg/L of chloride.

## 6.7 One adjoint ranks the recharge sites

The adjoint sweep of each reference case also gives the sensitivity of the screen chloride to a small recharge well at every column (53). Ranking the candidate recharge sites by this marginal value, from one forward and one adjoint run, and comparing with the ranking by the safe yield of the complete assembly with a 1 m²/d recharge well at each site (25 draws and some 685 forecasts per supply site), the rank correlation is 0.90, 1.00, 1.00, 1.00 for the supply wells at 2.0, 2.5, 3.0, 3.5 km, and the best site is the same in 3 of 4 cases. The adjoint puts the largest marginal value of recharge landward of the supply well in 2 of 4 cases.

# 7. For Fushë-Kuqe: the recipe, and the limits

- Assemble flow and salt as one block, solved by Newton with the exact Jacobian; at Fushë-Kuqe the density coupling and the immobile zones of the silt and clay are not optional for a chloride goal (weights near the limit itself), while the hydraulic memory of the lenses can wait until a goal on heads or on the clay piezometers needs it.

- State the goal before the model: the chloride in the screens of the supply wells of the Gorre–Fushë-Kuqe field at the planning horizon, with the risk α the utility accepts. One forward and one adjoint run of the complete three-dimensional system at a few reference layouts then give the weights of every term and the sensitivity to recharge at every candidate site.

- Cut the chemistry for the decision, not for the check: the seven ions, the exchanger and the learned chemistry closure do not change the chloride goal (Theorem M1) and are run once for the chosen layout, to check sodium (Proposition P3 of the complete-system note) and the softened water behind recharge wells.

- Calibrate the cheap model against the complete one with a few dozen paired runs and put the margin into the chance constraint; a coarse model that is consistently conservative gets its water back, one whose error changes sign does not.

- Rank the candidate recharge sites by one adjoint run before running any placement; on the test it agreed with the complete placement in 3 of 4 cases.

- Watch the placed wells with an alarm built for the new pumping: a head-based chloride alarm learned before the decision is valid after it only if the decision is corrected through the recorded pumping and an unrecorded change of climate is certified [48].

- Measure what the weights point to: the inland water balance, which carries the largest weight of a missing source, and chloride profiles through the transition zone near the supply screens, which set the mixing width that decides the standard.

The limits of the test are those of a cross-section. The section is two-dimensional, so wells are lines along the coast and rates are per metre of coast; the Peaceman correction, rivers, the lagoon and a leaky base are in the assembly but not in the test. The parameters follow the Fushë-Kuqe values of the series, but the aquifer is synthetic and its present state comes from an assumed pumping history, not from assimilated data. The chance constraint was judged on 25 evaluation draws, so the realised risks are resolved to about 4%. The safe yield is an order statistic, which controls the risk on the draws used; a certificate at confidence 1−δ in the Learn-then-Test sense would allow no exceedance among 25 draws at δ=10%, lowering the value of the chosen layout by 11.6%, and none could be issued at δ=5% [48]. The conformal margin assumes that the decision cases are exchangeable with the calibration cases, which the selection of the best layout breaks mildly. The first-order weights are reliable for the terms that are cut and for small changes of the terms that are kept, but not for the full one-pass split, whose effect is not monotone in the lag. The dual-weighted estimate was not reliable on the uniform mesh, and the upwind goal is only piecewise differentiable where flow reverses. The prices of conveyance and recharge are illustrative. The next step is the same procedure on the three-dimensional Fushë-Kuqe sector, with the present state from the assimilation of the chloride series of 1984–2001.

# 8. The system at a glance

Table 8. The assembled system at a glance, and what the placement goal needs of each block.

| Block | Equations | Assembled in the test | For the chloride goal |
| --- | --- | --- | --- |
| Flow with storage and wells | (1), (4), (26) | yes (monolithic with salt) | kept |
| Memory of the lenses (kernel, tail, lift) | (2), (3) | yes | cut (weight below 2×10⁻⁵) |
| Learned closure Ψ | (6) | zero | sensitivity mapped (Section 6.2) |
| Density and buoyancy | (5) | yes | kept |
| Salt with immobile zones and dispersion | (7)–(10) | yes | kept |
| Screen chloride and the standard | (11), (12) | yes (the goal) | goal |
| Seven ions, exchange, reactions, Φ | (13)–(21) | yes, once (Section 6.6) | cut exactly (Theorem M1); checked after |
| Electro-diffusion | (18) | no | cut (leak bound 4 × 10⁻⁶) |
| Toe and its mobility | (22)–(25) | diagnostic | not a goal (Section 6.5) |
| Sea boundary, sea-level rise, tides | (29), (30) | hydrostatic sea, rise | rise cut for this horizon (weight below 10⁻⁵) |
| Land, recharge, rivers, base | (31), (32) | regional inflow; closed top and base | kept (inflow); others not in the test |
| Initial state | (33)–(35) | pre-development + 50-year history | kept |
| Placement problem and adjoint | (47)–(53) | yes | the final goal |

Prepared on 27 October 2026 from the code and results of experiments/montimi_go in the author's repository (stages 0–5 and the analysis); every number about the test is read from experiments/montimi_go/res and checked when the document is built. References marked "not verified" were not checked against the publisher.

# References

[1]	Zeqiraj, D. (2026). Goal-oriented Bayesian experimental design for variable-density seawater intrusion: adjoint expected information gain for joint monitoring and managed-aquifer-recharge placement. Journal of Hydrology, 680, 136559. https://doi.org/10.1016/j.jhydrol.2026.136559

[2]	Zeqiraj, D. (2026). Missing dynamics in coastal-aquifer flow and multi-species transport: 30 papers for a COST candidate system of equations. Literature review 42 of the author's series (23 October 2026), with the numerical experiment GJURMA-GO.

[3]	Zeqiraj, D. (2026). The transient state: flow and multi-species transport with memory, and what time-dependent data identify. Literature review 43 of the author's series (23 October 2026), with the numerical experiment RITMI-GO.

[4]	Zeqiraj, D. (2026). The trace and the rhythm: an augmented transient system of variable-density flow and multi-species transport for coastal aquifers, and what chemistry and time-dependent data can identify. Synthesis article of reviews 42 and 43 (24 October 2026).

[5]	Zeqiraj, D. (in preparation). From sparse hydrochemistry to decision-relevant information: probabilistic chloride mapping and goal-oriented monitoring design for seawater intrusion in the Fushë-Kuqe coastal aquifer, Albania. Code: chloride_fk in the author's repository.

[6]	Zeqiraj, D. (2026). The Fushë-Kuqe system in full: variable-density flow, multi-species hydrogeochemistry, boundary and initial conditions, and the objective that places the wells. Technical note of the author's series (26 October 2026), with the numerical experiment PUSI-GO.

[7]	Zeqiraj, D. (2026). All the boundary and initial conditions of a coastal aquifer: sea, atmosphere, soil moisture, rivers, pumping regimes and the initial state, as stochastic and causal objects in the augmented transient system. Literature review 45 of the author's series (24 October 2026), with the numerical experiment KUFIRI-GO.

[8]	Zeqiraj, D. (2026). Boundary and initial conditions: the mathematics and computation of what a model is given. Literature review 44 of the author's series (24 October 2026), with the numerical experiment FILLIMI-GO.

[9]	Zeqiraj, D. (2026). Updating the boundary and initial conditions of the Fushë-Kuqe system by data assimilation: the expanded equations, why the update is right, and how it stays right in extreme events. Technical note of the author's series (October 2026), with the numerical experiment ASIMILIMI-GO.

[10]	Zeqiraj, D. (2026). PRAG-GO and PRAG-MAR at Fushë-Kuqe: monitoring and managed aquifer recharge oriented to the intrusion risk of supply wells, recomputed with the geolocated sea boundary. Research note of the author's series.

[11]	Zeqiraj, D. (2026). Bandits and the value of information for siting managed aquifer recharge: which pilot tests to make, when to stop and where to build. Literature review 53 of the author's series, with the numerical experiment VENDI-GO.

[12]	Zeqiraj, D. (2026). Assembling the equations, placing the wells: twenty papers from Oxford, Caltech, Harvard and MIT. Literature review 60 of the author's series (27 October 2026).

[13]	Voss, C. I. & Souza, W. R. (1987). Variable density flow and solute transport simulation of regional aquifers containing a narrow freshwater–saltwater transition zone. Water Resources Research (volume, pages and DOI not verified).

[14]	Werner, A. D., Bakker, M., Post, V. E. A., Vandenbohede, A., Lu, C., Ataie-Ashtiani, B., Simmons, C. T. & Barry, D. A. (2013). Seawater intrusion processes, investigation and management: Recent advances and future challenges. Advances in Water Resources, 51, 3–26. https://doi.org/10.1016/j.advwatres.2012.03.004

[15]	Haggerty, R. & Gorelick, S. M. (1995). Multiple-rate mass transfer for modeling diffusion and surface reactions in media with pore-scale heterogeneity. Water Resources Research (volume, pages and DOI not verified).

[16]	Bear, J. (1979). Hydraulics of Groundwater. McGraw-Hill (details not verified).

[17]	Council of the European Union (1998). Council Directive 98/83/EC of 3 November 1998 on the quality of water intended for human consumption. Official Journal of the European Communities, L 330; indicator parameters chloride 250 mg/L and sodium 200 mg/L (page numbers not verified).

[18]	Gaines, G. L. & Thomas, H. C. (1953). Adsorption studies on clay minerals. II. A formulation of the thermodynamics of exchange adsorption. The Journal of Chemical Physics (volume, pages and DOI not verified).

[19]	Appelo, C. A. J. & Postma, D. (2005). Geochemistry, Groundwater and Pollution, second edition. Balkema (pages not verified).

[20]	Plummer, L. N., Wigley, T. M. L. & Parkhurst, D. L. (1978). The kinetics of calcite dissolution in CO2–water systems at 5° to 60 °C and 0.0 to 1.0 atm CO2. American Journal of Science (volume, pages and DOI not verified).

[21]	Davies, C. W. (1962). Ion Association. Butterworths (details not verified).

[22]	Zeqiraj, D. (2026). Transport of charged species in water and porous media: from structural electroneutrality to hydrogeochemistry. Literature review 30 of the author's series.

[23]	Zeqiraj, D. (2026). The electrodiffusive leak of the cut: how much error of a reactive closure can reach the chloride forecast through ion coupling. Research note of the author's series (Theorem 2, experiment LIDHJA-GO).

[24]	Stuyfzand, P. J. (2008). Base exchange indices as indicators of salinization or freshening of (coastal) aquifers. Proceedings of the 20th Salt Water Intrusion Meeting, Naples, Florida (details not verified).

[25]	Strack, O. D. L. (1976). A single-potential solution for regional interface problems in coastal aquifers. Water Resources Research, 12(6), 1165–1174. https://doi.org/10.1029/WR012i006p01165

[26]	Peaceman, D. W. (1978). Interpretation of well-block pressures in numerical reservoir simulation. Society of Petroleum Engineers Journal, 18(3), 183–194. https://doi.org/10.2118/6893-PA

[27]	Farrell, P. E., Kirby, R. C. & Marchena-Menéndez, J. (2021). Irksome: automating Runge–Kutta time-stepping for finite element methods. ACM Transactions on Mathematical Software, 47(4), Article 30, 1–26. https://doi.org/10.1145/3466168

[28]	Farrell, P. E., Knepley, M. G., Mitchell, L. & Wechsung, F. (2021). PCPATCH: software for the topological construction of multigrid relaxation methods. ACM Transactions on Mathematical Software, 47(3), Article 25. https://doi.org/10.1145/3445791

[29]	Haghighat, E., Amini, D. & Juanes, R. (2022). Physics-informed neural network simulation of multiphase poroelasticity using stress-split sequential training. Computer Methods in Applied Mechanics and Engineering, 397, 115141. https://doi.org/10.1016/j.cma.2022.115141

[30]	Charnes, A. & Cooper, W. W. (1959). Chance-constrained programming. Management Science, 6(1), 73–79. https://doi.org/10.1287/mnsc.6.1.73

[31]	Nemirovski, A. & Shapiro, A. (2006). Convex approximations of chance constrained programs. SIAM Journal on Optimization, 17(4), 969–996. https://doi.org/10.1137/050622328

[32]	Belotti, P., Kirches, C., Leyffer, S., Linderoth, J., Luedtke, J. & Mahajan, A. (2013). Mixed-integer nonlinear optimization. Acta Numerica, 22, 1–131. https://doi.org/10.1017/S0962492913000032

[33]	Maragno, D., Wiberg, H., Bertsimas, D., Birbil, Ş. İ., den Hertog, D. & Fajemisin, A. O. (2025). Mixed-integer optimization with constraint learning. Operations Research, 73(2), 1011–1028. https://doi.org/10.1287/opre.2021.0707

[34]	Song, J., Lanka, R., Yue, Y. & Dilkina, B. (2020). A general large neighborhood search framework for solving integer linear programs. Advances in Neural Information Processing Systems 33 (NeurIPS 2020). https://proceedings.neurips.cc/paper/2020/hash/e769e03a9d329b2e864b4bf4ff54ff39-Abstract.html

[35]	Wan, X., Nguyen, V., Ha, H., Ru, B., Lu, C. & Osborne, M. A. (2021). Think global and act local: Bayesian optimisation over high-dimensional categorical and mixed search spaces. Proceedings of the 38th International Conference on Machine Learning, PMLR 139, 10663–10674. https://proceedings.mlr.press/v139/wan21b.html

[36]	Bertsimas, D., Shtern, S. & Sturt, B. (2022). Technical note—two-stage sample robust optimization. Operations Research, 70(1), 624–640. https://doi.org/10.1287/opre.2020.2096

[37]	Giles, M. B. & Pierce, N. A. (2000). An introduction to the adjoint approach to design. Flow, Turbulence and Combustion, 65(3–4), 393–415. https://doi.org/10.1023/A:1011430410075

[38]	Hinze, M., Pinnau, R., Ulbrich, M. & Ulbrich, S. (2009). Optimization with PDE Constraints. Springer, Dordrecht. https://doi.org/10.1007/978-1-4020-8839-1

[39]	Becker, R. & Rannacher, R. (2001). An optimal control approach to a posteriori error estimation in finite element methods. Acta Numerica, 10, 1–102. https://doi.org/10.1017/S0962492901000010

[40]	Zhuang, J., Kochkov, D., Bar-Sinai, Y., Brenner, M. P. & Hoyer, S. (2021). Learned discretizations for passive scalar advection in a two-dimensional turbulent flow. Physical Review Fluids, 6(6), 064605. https://doi.org/10.1103/PhysRevFluids.6.064605

[41]	Vovk, V., Gammerman, A. & Shafer, G. (2005). Algorithmic Learning in a Random World. Springer, New York (details not verified).

[42]	Lei, J., G'Sell, M., Rinaldo, A., Tibshirani, R. J. & Wasserman, L. (2018). Distribution-free predictive inference for regression. Journal of the American Statistical Association, 113(523), 1094–1111 (not verified against the publisher).

[43]	Azizzadenesheli, K., Kovachki, N., Li, Z., Liu-Schiaffini, M., Kossaifi, J. & Anandkumar, A. (2024). Neural operators for accelerating scientific simulations and design. Nature Reviews Physics, 6(5), 320–328. https://doi.org/10.1038/s42254-024-00712-5

[44]	Lu, H., Saló-Salgado, L., Marzouk, Y. M. & Juanes, R. (2025). Uncertainty quantification of fluid leakage and fault instability in geologic CO₂ storage. Water Resources Research, 61, e2024WR039275. https://doi.org/10.1029/2024WR039275

[45]	Alhashim, M. G., Hausknecht, K. & Brenner, M. P. (2025). Control of flow behavior in complex fluids using automatic differentiation. Proceedings of the National Academy of Sciences, 122(8), e2403644122. https://doi.org/10.1073/pnas.2403644122

[46]	Hager, B. H., Dieterich, J., Frohlich, C., Juanes, R., Mantica, S., Shaw, J. H., Bottazzi, F., Caresani, F., Castineira, D., Cominelli, A., Meda, M., Osculati, L., Petroselli, S. & Plesch, A. (2021). A process-based approach to understanding and managing triggered seismicity. Nature, 595(7869), 684–689. https://doi.org/10.1038/s41586-021-03668-z

[47]	Cenameri, S. & Beqiraj, A. (2016). Assessment of seawater intrusion in Fushë-Kuqe aquifer, Albania. Bulletin of the Geological Society of Greece, 50(2), 665–670. https://doi.org/10.12681/bgsg.11772

[48]	Zeqiraj, D. (2026). What the twenty chloride papers and anchor regression give the Fushë-Kuqe work: checks, extractions and a chloride alarm that survives the decision. Note of the author's series (27 October 2026), with the experiment SPIRANCA-GO (experiments/spiranca_go).